3GPP TSG-SA WG1 Meeting #108 	S1-244531

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-244312)

Title:	Use Case On Glasses-free 3D Communication

Agenda Item:	8.1.4

Source:	China Mobile

Contact:	Shenxiu Du <dushenxiu@chinamobile.com>

Xiaonan Shi <shixiaonan@chinamobile.com>

Abstract: This document proposes a new use case on glasses-free 3D communication.

* * * First Change* * * *

8.x	Use case on glasses-free 3D communication

8.x.1	Description

As a video display effect enhancement technology, glasses-free 3D technology can be widely applied in video display scenarios, which also means that glasses-free 3D technology has broad application prospects in the field of communications. At present, various video communication services such as video calls, video conference, video CAT and Video CRS already have a large user scale. Introducing glasses-free 3D technology into the 6G system and making it one of the characteristic functions of 6G will effectively enhance the video display effect of video services and give users a brand-new usage experience.

Glasses-free 3D technology generally refers to a display technology in which users can directly see 3D videos on a display device without wearing wearable devices. The essence of glasses-free 3D technology is that the display device utilizes optical technologies such as grating technology to make the user's two eyes see two different pictures respectively. If these two pictures are from different angles of the same object, the brain of person will mistake them for three-dimensional objects, thus achieving the 3D display effect.

Since the display device needs to display two images simultaneously in one frame, and the physical hardware such as resolution and grating arrangement varies among different display devices, when generating the glasses-free 3D video stream, the two images cannot be directly arranged well in the same frame. Instead, the two images must be completely separated, for example, placed side by side (left-right format) or one above the other (top-bottom format). Then, after receiving the glasses-free 3D video stream, the display device analyzes it and superimposes it into a glasses-free 3D video based on the physical hardware requirements of the device itself.

Figure 8.x.1-1  Left-right format:The images of the left eye and the right eye are transmitted in the same frame, and the horizontal resolution is halved.

Figure 8.x.1-2  Top-bottom format:The images of the left eye and the right eye are transmitted in the same frame, and the vertical resolution is halved.

In the glasses-free 3D communication scenario, the terminal will be one of the main display devices. In the glasses-free 3D communication scenario, the terminal should first report its supported glasses-free 3D capabilities to the network side. After the terminal and the network complete the signaling negotiation and media negotiation for glasses-free 3D communication, the network will send the glasses-free 3D video stream in the corresponding format according to the terminal's capabilities. If the video stream that the network needs to send does not match the terminal's glasses-free 3D capabilities, the network can use AI technology to perform additional processing on the video stream, such as converting a 2D video stream into a 3D video stream. After the terminal obtains the glasses-free 3D video stream, the application layer will decode and reconstruct the video stream according to the requirements of the physical properties of its screen.

In addition, while the glasses-free 3D video decoding and reconstruction capabilities provided by the application layer are increasing, and the AI technology used by the network side to convert video streams into glasses-free 3D ones is also able to enhance accordingly. If there are more formats of glasses-free 3D video streams, both the terminal and the network side can be expanded to support the new formats.

8.x.2	Pre-conditions

1. UEA supports decoding and playing the glasses-free 3D video.

2. UEB does not support glasses-free 3D communication.

8.x.3	Service Flows

- User A uses UEA to request to establish a glasses-free 3D video call with UEB.

- UEA reports to the 6G system that only glasses-free 3D video in left-right format is supported .

- The 6G system conducts glasses-free 3D video media negotiation with UEB.

- UEB reports to the 6G system that  glasses-free 3D communication is not supported.

- User B uses UEB to answer this video call.

- 6G system converts the video stream from UEB into a glasses-free 3D video stream in left-right format with the help of AI technology.

- The 6G system sends the converted glasses-free 3D video stream to UEA.

- UEA receives the glasses-free 3D video stream and decodes and plays the stream.

8.x.4	Post-conditions

User A can see glasses-free 3D videos on UEA without the help of any wearable device.

8.x.5	Existing features partly or fully covering the use case functionality

None

8.x.6	Potential New Requirements needed to support the use case

[PR 8.X.6-1] Subject to operator policy and user’s consent, 6G system shall support glasses-free 3D video call.

NOTE: 6G system may need UE to report the supported glasses-free 3D video stream formats to better support glasses-free 3D video call if format transfer is needed between different users. And 6G system need to support converting 2D video streams into glasses-free 3D video streams in the specified format.

* * * End of Change * * * *