3GPP TSG SA WG 1 Meeting #108 	S1-244558

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-244237)

Source:		OPPO

Title:	6G system operation with Minimum-Full functionality-set transition

Agenda item:	8.1.1

Document for:	agreement

Contact:	Yang Xu
xuyang@oppo.com

Abstract: This contribution provides a use case in the area of network operatio. A minimum-full functionality-set based system operation is proposed so that a UE may stay in the minimum functionality-set for regular services and only switch to other kernel(s) when an advanced service is needed.

********** Start of Change (all text below are new) *********

5.x	6G system operation with Minimum-Full functionality-set transition

5.x.1	Description

Today, most of daily usage such as social media, on-line shopping, payment, voice call, instant message/photo/video, et al, can be fulfilled with a data rate of 10 Mbps (DL). Only when UE uses an advanced service such as VR/AR, it needs a very big data rate e.g. 100 Mbps - 1Gbps. Therefore, a regular user behaviour leads to the fact that the UE transmit with low data rate in most of time (e.g. 90% time of daily usage) when UE is in connected mode.

On the other hand, in 5G, the eMBB data rate is defined as 50/25Mbps (DL/UL) for upper limit. The downstream group has to make a design fulfilling this upper limit no matter whether the user really need it or not.

If the upper limit can be reduced e.g. to 10 Mbps, it will be helpful for downstream group to design a simpler system.

Figure-5.x.1-1 6G day-1 system supporting Minimum-full functionality set for eMBB (the red dash line)

As the figure-5.x.1-1 shown, it is proposed in 6G day-1, for eMBB service, 3GPP system has a common design with supporting a minimum functionality-set, in which UE may stay in the minimum set with low data rate (e.g. 10 Mbps) in most of time, when the user uses an advanced service, it can be fast transitioned to a full functionality-set status with much higher data rate (e.g. 1 Gbps). Both minimum and full status belong to the same RAT. When the system (UE and the network) stay in the minimum set, the complexity can be reduced compared to 5G eMBB.

Minimum functionality-set: include basic registration, authentication, mobility and session management as well as the data rate of 10 Mbps for DL, for most of usage including voice, instant message, video, and IoT-like service.

Full functionality-set: include a more complicated functionality as well as data rate of 1 Gbps in order to fulfil the advanced service such as Immersive.

In the figure-5.x.1-2, it illustartes the difference from the system capability perspective.

-  In 5G, the NR is designed in the first release with a high capability supporting a data rate of 50/25 Mbps (DL/UL), then it is struggled to reduce the capability for RedCap/NB-IoT in further release. Different capabilities lead to different type of end-device, which brings complexity for modular design.

-  Comparing to 5G, 6G is expected to have a common design with minimum functionality-set and a full functionality-set. The minimum functionality-set supports the data rate of 10 Mbps which may fulfil most of daily usage such as voice, instant message, video, as well as the IoT-like service. Only when an advanced service is used e.g. AR/VR service, the UE is transtioned to the full functionality-set.

NOTE 1: the 6G eMBB, the minimum functionality-set can at least fulfill the NB-IoT services

5G/4G system capability												6G system capability

Figure 5.x.1-2 System capability for 5G and 6G

5.x.2	Pre-conditions

Not applicable

5.x.3	Service Flows

Not applicable

5.x.4	Post-conditions

Not applicable

5.x.5	Existing features partly or fully covering the use case functionality

In 3GPP TS22.261 clause 7.1, high data rates and traffic densities, it defined the KPI as below:

Table 7.1-1 Performance requirements for high data rate and traffic density scenarios.

|  | Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) | Area traffic capacity (UL) | Overall user density | Activity factor | UE speed | Coverage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Urban macro | 50 Mbit/s | 25 Mbit/s | 100 Gbit/s/km2 (note 4) | 50 Gbit/s/km2 (note 4) | 10 000/km2 | 20 % | Pedestrians and users in vehicles (up to 120 km/h | Full network (note 1) |
| 2 | Rural macro | 50 Mbit/s | 25 Mbit/s | 1 Gbit/s/km2 (note 4) | 500 Mbit/s/km2 (note 4) | 100/km2 | 20 % | Pedestrians and users in vehicles (up to 120 km/h | Full network (note 1) |
| 3 | Indoor hotspot | 1 Gbit/s | 500 Mbit/s | 15 Tbit/s/km2 | 2 Tbit/s/km2 | 250 000/km2 | note 2 | Pedestrians | Office and residential (note 2) (note 3) |
| 4 | Broadband access in a crowd | 25 Mbit/s | 50 Mbit/s | [3,75] Tbit/s/km2 | [7,5] Tbit/s/km2 | [500 000]/km2 | 30 % | Pedestrians | Confined area |
| 5 | Dense urban | 300 Mbit/s | 50 Mbit/s | 750 Gbit/s/km2 (note 4) | 125 Gbit/s/km2 (note 4) | 25 000/km2 | 10 % | Pedestrians and users in vehicles (up to 60 km/h) | Downtown (note 1) |

In TS22.261 clause 7.6.1 AR/VR, it defined KPI as below

5.x.6	Potential new requirements needed to support the use case

Table 7.6.1-1 KPI Table for additional high data rate and low latency service

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area (note 2) |
| Cloud/Edge/Split Rendering (note 1) | 5 ms (i.e. UL+DL between UE and the interface to data network) (note 4) | 0,1 to [1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120 frames per second content. | 99,99 % in uplink and 99,9 % in downlink (note 4) | - | Stationary or Pedestrian (note 7) | Countrywide |
| Gaming or Interactive Data Exchanging (note 3) | 10ms (note 4) | 0,1 to [1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120 frames per second content. | 99,99 % (note 4) | ≤ [10] | Stationary or Pedestrian (note 7) | 20 m x 10 m; in one vehicle (up to 120 km/h) and in one train (up to 500 km/h) |
| Consumption of VR content via tethered VR headset (note 6) | [5 to 10] ms (note 5) | 0,1 to [10] Gbit/s (note 5) | [99,99 %] | - | Stationary or Pedestrian | - |

5.x.6	Potential New Requirements needed to support the use case

Functionality Requirements:

[P.R. 5.X.6-1] The 6G system shall be able to support a minimum functionality-set with basic registration, session management and authentication, which can fulfil regular services such as voice, message and emergency services.

[P.R. 5.C.6-2] The 6G system shall be able to support to transition the minimum functionality-set to a high functionality-set for the advanced services such as XR, in which the minimum functionality-set and advanced functionality-set belong to the same RAT.

Editor’s Note: It is FFS for the KPI value the minimum functionality-set can reach

*******************End of Change*****************