3GPP TSG-SA1 Meeting #108	S1-244640

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.101 | CR | 0600 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On Emergency SMS requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | SMS2EC |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Regulatory requirements (e.g. EECC Article 109 and EECC Recital 285) can apply to emergency SMS, should a Member State decide to implement SMS as a means of access to emergency services, and are not limited to specific solutions (e.g. can apply to SMS over IMS or NAS). Curent SA1 requirements on SMS to emergency centre are limited to SMS over IMS, whilst other options should be considered (e.g. if UE or NW does not support IMS). Other aspects are missing in current requirements and should be clarified, including roaming or limited service state restriction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added SMS over NAS as additional option, clarified other aspects and added new requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current requirements remain unnecessarily constrained and unclear on certain aspects. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.11	Short Message Service over IMS to emergency centre

EPS and 5GS shall route a Short Message over IMS using the emergency numbers to a local emergency response centre (i.e. PSAP), according to the local regulations.

The Short Message shall be routed to the emergency response centre serving the location where the subscriber is located and replies routed back to the subscriber in accordance with national regulations as specified in 10.1.

The 3GPP system shall support a Short Message to be routed to different emergency response centres, depending on the type of emergency as specified in 10.1.

The Short Message Service to emergency response centre may be supported for roaming UEs and in limited service state, including the case of SIM/USIM/ISIM not being present.

Note 1: 	It is left to the national authorities to decide whether the network accepts Short Message Service to emergency response centre in limited service state, including the case without the SIM/USIM/ISIM.

The Short Message Service to emergency response centre shall be able to use the same identification methods as for emergency calls, as specified in 10.1.1.

Subject to local/regional regulations Short Message Service to emergency response centre shall support location service as specified in chapter 10.6.

NOTE: The above requirements may apply to SMS over NAS, subject to local/regional regulations.