3GPP TSG-SA1 Meeting # 108	S1-244834

Orlando, Florida, USA 18-22 November 2024	(revision of S1-244123, S1-244436)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.153 | CR | 0071 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MPS when access to EPC/5GC is satellite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs [CISA ECD, AT&T, T-Mobile US, Verizon] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS_Sat |  |  |  |  |  | Date: |  |  | 2024-11-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current requirements in TS.22.153 do not cover 3GPP system with satellite support of MPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text modifications and addition of a new clause with normative requirements for 3GPP system with satellite support of: - MPS for MMTEL voice/video, - MPS for DTS. - MPS for Messaging services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of explicit requirements for 3GPP system with satellite support of MPS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18 and new clause 11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START of 1st CHANGE***

## 5.18	3GPP and non-3GPP accesses

When a UE is using 3GPP RAT(s) as well as non-3GPP WLAN access technology, the 3GPP system will select the most appropriate 3GPP RAT or non-3GPP WLAN access technology for one or more services active on theUE. This subclause covers MPS handling when 3GPP RAT(s) and non-3GPP WLAN access technology are being used for one or more services active on a UE.

The system shall support MPS based on operator policy when the UE is using 3GPP RAT(s) and non-3GPP WLAN and connected to the same EPC.

The system shall support MPS based on operator policy when the UE is using 3GPP RAT(s) and non-3GPP WLAN and connected to the same 5GC.

*** END of 1st CHANGE***

*** Start of 2nd CHANGE***

# 11	MPS support in 3GPP system with satellite

MPS can be supported in 3GPP systems with satellite. Support of MPS when the UE/IoT device has satellite access is important during certain disaster scenarios such as when terrestrial access networks are unavailable or degraded. In some cases, satellite might be the only available access (e.g., remote areas with only satellite coverage or during terrestrial access network failure) and therefore critical for MPS communications.

The following subclauses provide requirements specific to MPS support in 3GPP systems with satellite (e.g., satellite access to EPC/5GC).

NOTE:	The following conditions apply to all requirements in clause 11:

-	MPS support is subject to regulatory rules,

-	MPS is supported by terrestrial operator policy,

-	MPS is supported by satellite operator policy (if the satellite operator is not the terrestrial operator), and

-	MPS is supported by terrestrial operator and satellite operator agreements (if any).

## 11.1	MPS for MMTEL voice and video

### 11.1.1	General

The system with satellite shall support MPS for MMTEL voice and video as specified in clause 9.1 and 9.2.

### 11.1.2	Service continuity

The system shall support MPS for MMTEL voice/video continuity when features for service continuity as specified in clause 6.46.3 of TS 22.261 [7] are supported within a satellite network (e.g., ISLs), between satellite networks, and between a terrestrial access network and a satellite access network.

## 11.2	MPS for DTS

### 11.2.1	General

The system with satellite shall support MPS for DTS as specified in clause 9.3.

### 11.2.2	Service continuity

The system shall support MPS for DTS continuity when features for service continuity as specified in clause 6.46.3 of TS 22.261 [7] are supported within a satellite network (e.g., ISLs), between satellite networks, and between a terrestrial access network and a satellite access network.

## 11.3	MPS for Messaging services

### 11.3.1	General

The system with satellite shall support MPS for Messaging services as specified in clause 9.4.

### 11.3.2	Service continuity

The system shall support MPS for Messaging continuity when features for service continuity as specified in clause 6.46.3 of TS 22.261 [7] are supported within a satellite network (e.g., ISLs), between satellite networks, and between a terrestrial access network and a satellite access network.

*** END of 2nd CHANGE***

*** END of CHANGE***