Si3GPP TSG-SA WG1 Meeting #108 	S1-244850

Orlando, US, 18-22 November 2024	(revision of S1-244241 and S1-244609)

Title:	Utility infrastructure monitor and control

Agenda Item:	8.1.5 Massive Communication

Source:	Ericsson, Itron, AT&T, EDF, Sony, Anterix

Contact:	Daniel Lönnblad (Daniel.lonnblad at ericsson.com)

Abstract: Proposal to develop a 6G technology that will fit into the Utility Infrastructure use cases. In a world where virtual replicas of the real-world utilities, the communication technology will be important. To set the requirement of such a technology already from day 1 in 6G would be beneficial for the industry. Improved bulk management, cost of the device and simplification of the onboarding and connectivity management are vital for the enabling of massive IoT cases. Security considerations and simplification and possibilities of treating the payload according to use case requirements are also potential elements that has a demand on the industrial and consumer cases.

# 9	Massive Communication

Editor's Note: "Massive communication" include e.g. LPWA

## 9.1	Use case 1

## 9.x	Use case on Utility infrastructure monitor and control

### 9.x.1	Description

Utility infrastructures are central and critical across societies and enterprises. They are increasingly becoming more and more connected at large scale. This is driven by both new advanced applications having new requirements, as well as enabled by new technologies, e.g. renewable energy sources and battery technologies. Utility components in mind here include primarily electricity next-generation grid edge intelligence smart meters, but also partially or fully battery-powered devices, e.g. Direct Transfer Trip (DTT) and Fault Location, Isolation, and Service Restoration (FLISR) units, that have quite similar requirements around metering, monitoring and control of distribution assets.

The electricity utility sector has already massively deployed a first generation of smart metering infrastructures that are operational on a global scale since the last 15+ years. The primary focus has been the monitoring of consumption towards e.g. accurate billing and to automate meter readings. The utility industry is now planning for the next generation of a data driven advanced metering infrastructure (AMI 2.0) that will enable a range of new more advanced use cases for monitoring and controlling the production, distribution, and consumption of electricity, and at the same time making the electrical grid even more resilient to disruptions and to become more stable.

The introduction of distributed energy resources (DER) for more sustainable energy production as well as securing energy availability with solar panels on homes, buildings and on land as a prominent example require connectivity for proper monitoring and further distribution of surplus electricity into the overall grid, The electrification of road transportation (EV) requires connected charging infrastructure that can also serve as intermediate storage, and the latter is also possible by standalone batteries where DER is used. Connected smart streetlights is another example to ensure proper energy savings. Another application is Demand-Response control (DR) that can shift electricity usage to the benefit of both the consumer (cost control), and the utility operator (control connected appliances to shave off peaks) by controlling appliances and other energy intensive assets. A further set of applications target the operations of the distribution grid itself: to do load balancing, fast outage detection, fault isolation, pacing etc. All of these applications are data-driven requiring timely access to data and events, even down to “real-time” to make it possible to monitor and control the entire distribution network with all its various components in order to achieve the desired outcomes and objectives.

All this requires efficient and effective use of cellular network across all deployed connected assets in the entire grid. These applications are demanding and require both new and extended requirements beyond what e.g. 4G NB-IoT and Cat-M1 can support. Further, this infrastructure is expected to be around for around 20 years up to 2050 thus putting a hard requirement on longevity of the used technologies and solutions, including the supporting communication networks.

A communication technology for this would need to have the flexibility to be able to manage different devices; from low complexity sensors with only small data and limited battery to more complex devices that may need real time communication with high reliability. Existing 3GPP solutions are based on 4G (NB-IoT and Cat M) and these will be complex to integrate with a new 6G system. The new 6G system supporting the utility infrastructure monitor and control use case still needs to have similar features as existing technology with features such as extended range and optimized towards small and low complexity devices, but with longevity to be at least 20 years and with an overall improved energy efficiency, including reduced energy consumption for the network. All factors of energy saving needs to be considered including different usage profiles, for instance devices that only report data occasionally if requested by the application may require technologies such as Wake-up signals.

From a security point of view, same or improved ways to manage and update devices would be needed including remote operation of the network ID (e.g. eSIM, RSP) and life cycle management, including methods for securely manage a large set of devices with similar traffic patterns and communication requirement. Other methods for sharing data in a secure way e.g. excluding internet and exposure and utilize direct mapping to data collection infrastructure can be considered.

Positioning of these devices will be important for some user scenarios. These devices will produce a large set of data and hence some processing (e.g., preparation, filtering, aggregation, inferencing, fusion, provenance) of the data would be required to enable a more efficient sharing with 3rd party application

To enable the utility infrastructure monitor and control use case in the 2030 era, it would be important to design the 6G system to cater for such technology in an efficient and energy saving manner form day 1.

Key value impact analysis

The positive impact from the 6G system use case will depend on where the technology is applied. For instance, a 6G system used in energy supply systems will help provide reliable power distribution to businesses and households, while massive use of a 6G system with sensors in a factory will result in other positive impacts. Below results are written from a general perspective.

Energy resources: As explained, the 6G system can play a central role in sustainable and resilient use of energy resources via a wide range of applications in the electricity distribution network. Also, the network energy consumption supporting the utility infrastructure monitor and control use case must be taken into account.

Emissions: More operational efficiency can reduce emissions in parallel to energy and material use, however there is a risk that the sensors will lead to additional emissions from electronic waste as massive amounts of equipment spread out will be expensive to collect.

Trustworthiness: There is an introduced risk of cyber-attacks from introducing connected devices across society-critical infrastructures such as utilities that require security measures towards various threat vectors. Also, the risk for privacy intrusion needs to be considered. There are mature and efficient security technologies that are adequate and fit for use in M-IoT deployments, such as eSIM, TEE and efficient secure protocols with end-to-end security.

Infrastructure: The 6G system with appropriate and new requirements can serve the growing utility infrastructure needs for better monitoring and control of distribution grid infrastructure in an efficient way. Vulnerability includes the increased risk of cyber-attacks as mentioned, something that comes with any connected operations.

### 9.x.2	Pre-conditions

Company NoName is a utility company that utilizes next-generation grid edge intelligence smart meters for electricity in a region NewLand. The company has the requirements that the meters can be used with 20years longevity and should be possible to be deployed in hard-to-reach areas, such as manholes and basements below ground.

The company may want to receive instantaneous load and voltage status from the meters at a fixed time interval of one time per minute and in a flexible way some other battery-powered sensors should only report occasionally when the application requires it. These meters work with limited energy consumption and can be deployed in both new 6G and existing 5G spectrum in a flexible way.

Company NoName has today services currently served by NB-IoT and eMTC but the new meters and their frequent minute level communications are overloading those networks, and therefore be supported by a more efficient radio and an evolved 5GC and independent from EPC. This is also in the interest of the mobile operator ABC, who wants to reduce the cost of operations.

ABC wants to provide the new 6G service for utility infrastructure monitor and control  to the customer such that the utility company NoName experience excellent service performance, while ABC also experience low total cost of operation, with efficient management, low energy consumption, and flexible use of spectrum.

### 9.x.3	Service Flows

Company NoName wants new meters to be future proof. This is also in the interest of the mobile operator ABC, who wants to reduce cost of operation and therefore have the utility infrastructure monitor and control use case supported by a 6G system. Company NoName provides new next-generation grid edge intelligence smart meters with 6G capability to its customers. ABC experiences a reduced cost of operations.

### 9.x.4	Post-conditions

The customers use next-generation grid edge intelligence smart meters and associated battery-powered grid edge sensors with confidence that the distribution grid can be run at a high usage level but not be overloaded despite an increasing number of EV Supply Equipment, Roof-top Solar and other DER equipment connected at the edge.

### 9.x.5	Existing features partly or fully covering the use case functionality

In TS 22.261-chapter 6.4 Resource efficiency states some general requirements for IoT devices.

The 5G system shall minimize control and user plane resource usage for data transfer from send only UEs.

The 5G system shall minimize control and user plane resource usage for stationary UEs (e.g. lower signalling to user data resource usage ratio).

The 5G system shall minimize control and user plane resource usage for transfer of infrequent small data units.

The 5G system shall optimize the resource use of the control plane and/or user plane for transfer of small data units.

The 5G system shall optimize the resource use of the control plane and/or user plane for transfer of continuous uplink data that requires both high data rate (e.g. 10 Mbit/s) and very low end-to-end latency (e.g. 1-10 ms).

The 5G network shall optimize the resource use of the control plane and/or user plane to support high density connections (e.g. 1 million connections per square kilometre) taking into account, for example, the following criteria:

-	type of mobility support;

-	communication pattern (e.g. send-only, frequent or infrequent);

-	characteristics of payload (e.g. small or large size data payload);

-	characteristics of application (e.g. provisioning operation, normal data transfer);

-	UE location;

-	timing pattern of data transfer (e.g. real time or non-delay sensitive).

The 5G system shall efficiently support service discovery mechanisms where UEs can discover, subject to access rights:

-	status of other UEs (e.g. sound on/off);

-	capabilities of other UEs (e.g. the UE is a relay UE) and/or;

-	services provided by other UEs (e.g. the UE is a colour printer).

The 5G system shall be able to minimise the amount of wireless backhaul traffic (e.g. consolidating data transmissions to 1 larger rather than many smaller), when applicable (e.g. providing service in an area subject to power outages).

The 5G system shall support small form factor UEs with single antenna.

NOTE:	Small form factor UEs are typically expected to have the diagonal less than 1/5 of the lowest supported frequency wave length.

In TS 22.261-chapter 8 Security, security requirement for IoT devices is listed.

In TS 22.104- Annex A4.7 Advanced metering following proposed requirement can be found

Table A.4.7-1: Communication KPI for advanced metering

| Use case# | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bit rate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area |
| 1 | > 99.99 |  | Accuracy fee control: < 100 (note 1); General information data collection: < 3000 | UL: < 2 M DL: < 1 M | – | – | – | stationary | < 10 000/km2 (note2) | – |
| NOTE 1: One-way delay from 5G IoT device to backend system. The distance between the two is below 40 km (city range). NOTE 2: It is the typical connection density in today city environment. With the evolution from centralised meters to socket meters in the home, the connection density is expected to increase 5 to 10 times. |  |  |  |  |  |  |  |  |  |  |

Use case#1: Advanced metering

### 9.x.6	Potential New Requirements needed to support the use case

The 6G system supporting the utility infrastructure monitor and control use case shall provide a broad range of communication capabilities over TN and/or NTN, ranging from 500-byte payload meter readings and instantaneous load/voltage status messages to 1 MB differential and 20 MB complete firmware upgrades  (Linux-based computer platforms with associated applications and AI Machine Learning models) with a minimal resource usage..

The 6G system supporting the utility infrastructure monitor and control use case shall provide low-complexity devices for massive deployment into next-generation grid edge intelligence smart meters.

The 6G system supporting the utility infrastructure monitor and control use case, shall be capable of handling a traffic intensity of one connection request per minute with up to 10 messages per burst from each UE.

The 6G system supporting the utility infrastructure monitor and control use case should facilitate low energy consumption for partially or fully battery-powered devices, e.g. Direct Transfer Trip (DTT) and Fault Location, Isolation, and Service Restoration (FLISR) units in remote locations, to achieve > 5-year battery lifetime on a 16 AH battery with acceptable reduction of status message sending up to once per hour while maintaining low latency control message reception (see table below).

The following KPIs should be supported:

| Profile | Characteristic parameter |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Bit rate down link | Bit rate up link | End-to-end latency: maximum | # of UEs connection | Communication service availability: target value |
| Utility infrastructure monitor and control | [5] Mbit/s | [5] Mbit/s | [< 100] ms | [50 000]/km2 | > 99.99 |