3GPP TSG-SA1 Meeting #108	S1-244xxx

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024				Revision of S1-244646

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.011 | CR | 0364 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on support of 3GPP PS data off for 5G |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, PS_Data_Off |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The generic functional description of 3GPP PS data off and its requirements are specified in TS 22.011, section 10. The functional description and requirments are defined for the case where PDN connections are established in 3GPP access networks. Whilst it captures the EPS case well, it remains a lack of clarifty for 5GS where PDU sessions are used instead. From stage-2 viewpoint, it is crystal clear that PS data off is supported for 5GS, e.g., as in TS 23.501, section 5.24. Therefore, PS data off for 5GS should be clarified in the stage-1 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In sub-clause 1.2.1 and 10.1, “PDU session” is added to cover the 5GS case. - In sub-clause 10.2, PS data off requirements corresponding to the 5GS case are amended. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-1 support of PS data off for 5G remains unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1.2.1, 10.1 and 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 1.2.1	Definitions

In addition to those below, abbreviations used in this 3GPP TS are listed in 3GPP TR 21.905 [2].

3GPP PS Data Off: A feature which when configured by the HPLMN and activated by the user prevents transport via PDN connections or PDU sessions in 3GPP access of all IP packets except IP packets required by 3GPP PS Data Off Exempt Services.

<< skip unchanged part >>

* * * Next Change * * * *

# 10 	3GPP PS Data Off

## 10.1	Description

3GPP PS Data Off is a feature which, when configured by the HPLMN and activated by the user, prevents transport via PDN connections or PDU sessions in 3GPP access networks of all data packets except IP packets required by 3GPP PS Data Off Exempt Services.

## 10.2	Requirements

The 3GPP system shall provide a mechanism by which an operator can configure which operator services are defined as 3GPP PS Data Off Exempt Services for their own subscribers.

When 3GPP PS Data Off is activated in the UE, in order to preserve charging consistency:

-	the UE shall inform the network that 3GPP PS Data Off is activated;

-	the UE shall cease the sending of uplink IP Packets of all services that are not 3GPP PS Data Off Exempt Services via PDN connections in 3GPP access networks;

-	the network shall cease the sending of downlink IP Packets to the UE for all services that are not 3GPP PS Data Off Exempt Services via PDN connections in 3GPP access networks;

-	the UE shall cease the sending of uplink traffic over non-IP PDN types via PDN connections in 3GPP access networks; and

-	the network shall cease the sending of downlink traffic over non-IP PDN types via PDN connections in 3GPP access networks;

-	the UE shall cease the sending of uplink IP packets of all services that are not 3GPP PS data Off Exempt Services via PDU sessions in 3GPP access networks;

-	the network shall cease the sending of downlink IP packets to the UE for all services that are not 3GPP PS data Off Exempt Services via PDU sessions in 3GPP access networks;

-	the UE shall cease the sending of uplink unstructured and ethernet data via PDU sessions in 3GPP access networks;

-	the network shall cease the sending of downlink unstructured and ethernet data via PDU sessions in 3GPP access networks.

NOTE 1:	Disabling of traffic on both the uplink and downlink is needed in order to provide consistency of charging between HPLMN and VPLMN, as well as consistency between what the user expects and what the user may be billed for.

Each of the following operator services shall be configurable by the HPLMN operator to be part of the 3GPP PS Data Off Exempt Services:

-	MMTel Voice;

-	SMS over IMS;

-	USSD over IMS (USSI);

-	MMTel Video;

-	Services over IMS Data Channel;

-	Particular IMS services not defined by 3GPP, where each such IMS service is identified by an IMS communication service identifier;

-	Device Management over PS;

-	Management of USIM files over PS (e.g. via Bearer Independent Protocol); and

-	IMS Supplementary Service configuration via the Ut interface using XCAP.

-	User plane location services protocol (LCS-UPP).

NOTE 1a: IMS Data Channel is defined in 3GPP TS 26.114[20].

NOTE 1b: Services over IMS Data Channel are treated as a whole to be exempt or not to be exempt. Usage of individual applications on the data channel can be controlled by operators when the PS Data Off Exempt of services over IMS Data Channel is on.

NOTE 1c:	3GPP PS Data Off Exempt for LCS-UPP in roaming is subject to agreement between operators.

3GPP PS Data Off may be activated based on roaming status, and the HPLMN may configure up to two sets of 3GPP PS Data Off Exempt Services for its subscribers: one is used when in HPLMN and another when roaming.

NOTE 2: The updating to the sets of configured 3GPP Data Off Exempt Services in the VPLMNs and HPLMN is not guaranteed to take effect in real time. The updating to the sets of configured 3GPP Data Off Exempt Services in the UEs is not guaranteed take effect in real time.

The user should be made aware of the operator services that are 3GPP PS Data Off Exempt Services.

NOTE 3: 	The system can support falling back to operate over the CS domain in case an operator service is not configured to be a 3GPP PS Data Off Exempt Service and an equivalent CS domain operator service exists for the operator service.