3GPP TSG-SA1 Meeting #108	S1-244391

18-22 Nov 2024, Orlando, US	(revision of S1-244204)

Source:	China Telecom, ZTE

pCR Title:	Use case on Distributed Networks and PLMN

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.1

Document for:	Approval

Contact:	Yinglin Chen, chenyl37@chinatelecom.cn; Lijuan Chen, chen.lijuan@zte.com.cn

Abstract: This use case introduces a concept of distributed networks. S1-244152 and S1-244155 were merged in this contribution.

1. Introduction

Please read the use case below.

2. Reason for Change

This is a use case.

3. Conclusions

None.

4. Proposal

It is proposed to study the use case in System and Operation Aspects.

* * *Start of Change * * * *

5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, sustainability and energy efficiency, device diversity, support of legacy services

## 5.x	Distributed Networks and PLMN

### 5.x.1	Description

6G network operation should be highly flexible and highly efficient. While using one public network to provide 6G basic services (e.g. connectivity), the public network operator can deploy distributed networks to meet differentiated service needs on demand.

The public network operator operating one public network providing basic service and multiple customized distributed networks providing differentiated service holds a network of distributed networks.

This use case demonstrates the 6G network's ability to automatically customize and deploy a distributed network, utilize services exposed by the PLMN, and operate independently when its connection to the public network fails.

### 5.x.2	Pre-conditions

Several valuable antiques were discovered in a rural area, drawing media organizations to the site to produce live news reports.

The media companies collaborate with Operator A to provide communication and network services for this event. During the period of the event, the following requirements must be met for the wireless communication network at the site:

-	Since the area is small and requires no handover, mobility procedures are simplified for operation efficiency. The distributed network's control plane capabilities (e.g. QoS and traffic steering) are optimized for video traffic.

-	Security monitoring to prevent excessive gathering around the antiques is provided via sensing services, which is not supported in the incumbent PLMN of Operator A covering this area.

-	Intelligent fault diagnosis of network conditions services is provided.

-	If its connection to PLMN fails, the distributed network continues to operate and provide service in best-effort.

-	Operations and maintenance of the distributed network can be managed locally without the need to go through the operator's centralized control system.

### 5.x.3	Service Flows

### Creation of the distributed network:

- The incumbent PLMN intelligently analyzes received requirements and identifies that new sensing functionality is required and handover / mobility functionalities are not required. Leveraging both existing and new service capabilities, the incumbent PLMN automatically orchestrates network functions and dynamically creates a customized temporary network to provide service for this event.

- To support intelligent fault diagnosis of network conditions, the distributed network needs to train a local model. Since the local computing resources are insufficient and the amount of available training data is small, the distributed network sends an AI service request to the PLMN to obtain the foundational model for the AI service.

- The PLMN validates that the required parameters can be met and responds by providing the requested AI service to the distributed network.

- The distributed network uses the fine-tuned model based on the received foundation model from PLMN to provide intelligent fault diagnosis of network conditions service for this event.

Service provision of the distributed network:

- A host in the dig site sets up a virtual meeting with remote guests in other places. The host wears a VR headset (the UE). The UE registers to the distributed network with the subscription data retrieved from the PLMN of Operator A.

- To create immersive VR experience, substantial data processing tasks are required for the VR headset. To save battery life, part of data processing tasks is offloaded to the edge server in the distributed network. The distributed network provides computing service to the UE.

- Data processing tasks for generating VR videos for remote guests are offloaded to the central server in the PLMN after pre-processing in the edge server, since the central server offers more computation resources than edge server. The PLMN provides computing service to the UE.

- During the virtual meeting, the host approaches the antiques. Other staff members are working nearby, and the number of people exceeds the allowed limit. This is detected via the sensing service and an alarm is trigged. A security guard is promptly dispatched to provide guidance and protect the antiques.

Maintenance of the distributed network:

- Due to excavation mistakes, the connection of the distributed network to the PLMN fails. The distributed network continues to operate independently for a period of time, providing limited services to UEs with subscription data available in the distributed network. UEs without available subscription data cannot access the distributed network. Since the connection of the distributed network to the PLMN is lost, the central server becomes unreachable and offload service to the central server cannot be provided. The video resolution experienced by remote guests decreases.

- The deployed intelligent fault diagnosis service detects and reports network anomalies. Engineers on-site are swiftly assigned to handle this issue that requires manual intervention.

- When the media completes news production and leaves the scene, the temporary distributed network is deleted.

Figure X.1. A network of distributed networks consists of a PLMN and multiple distributed networks.

### 5.x.4	Post-conditions

### The distributed network is deleted and users in the area receive services from the PLMN.

### 5.x.5	Existing features partly or fully covering the use case functionality

Network slicing is supported in 5G. Network operators meet differentiated service requirements by creating different network slices within a network. Each network slice can be customized based on service agreements. However, as the number of slices increases, network complexity also grows. The number of slices that can be accommodated within a single network is limited in practical deployment, which may not be sufficient given the increasing number of verticals requesting customized 6G services.

In addition, a network slice has to be a complete logical network, comprising of all network functions and NF services, configured end-to-end. System procedures in different network slices cannot be customized. The redundant network functions, NF services, and related system procedures degrade operation efficiency and increase energy consumption.

Furthermore, network slicing can only be performed based on the resources and services that have already been deployed. Temporarily adding new functions and services cannot be achieved through network slicing.

Non-public network is supported in 5G. Standalone NPN (SNPN) operates without dependency on a PLMN. This independence is favourable for verticals prioritizing local control over signaling, data, subscription management, high data security, and high resilience. However, SNPN subscribers cannot directly access PLMN services. Public network operators cannot efficiently utilize existing PLMN resources to provide basic communication services (e.g. calling) or services not deployed in SNPN (e.g. sensing) to SNPN subscribers. The high construction and maintenance costs associated with SNPN may be an obstacle for verticals.

Public Network Integrated-NPN (PNI-NPN) operates with dependency on a host PLMN. By utilizing PLMN resources, PNI-NPN is more cost-effective than SNPN. However, its reliance on the host PLMN results in lower data security and lower network resilience, which may not be able to meet verticals’ requirements. If the connection between the PLMN and PNI-NPN fails or the PLMN itself experiences failures, the PNI-NPN cannot function. Currently, 5G supports a fixed level of isolation and collaboration between non-public networks and the PLMN once deployed as either PNI-NPN or SNPN. However, 6G is expected to support a dynamic level of isolation and collaboration which flexibly changes with verticals’ requirements.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] Based on operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism that allows network operator to automatically create a customized (e.g. with minimum set of services, with new services on-demand), distributed network as required.

[PR 5.x.6-2] Based on operator’s policy and agreement with 3rd party, the 6G network shall be able to support lifecycle management (e.g. creation, modification, and deletion) of a distributed network without impact on the PLMN.

[PR 5.x.6-3] Based on operator’s policy and agreement with 3rd party, the 6G network shall allow network operator to provide service by operating a network of distributed networks formed by interconnected PLMN and distributed networks.

[PR 5.x.6-4] Based on operator’s policy and agreement with 3rd party, the 6G network shall be able to support a mechanism that allows distributed networks securely interwork with the PLMN and prevents unauthorized access to other networks.

[PR 5.x.6-5] Based on operator’s policy and agreement with 3rd party, the 6G network shall be able to support a distributed network to discover services provided by the PLMN.

[PR 5.x.6-6] Based on operator’s policy and agreement with 3rd party, the 6G network shall be able to support the PLMN to expose required services to a distributed network.

[PR 5.x.6-7] The 6G network shall support autonomous operation of a distributed network to provide communication service for a limited time period when losing connection to the PLMN.

NOTE 1:	It is assumed that limited services are provided during autonomous operation.

[PR 5.x.6-8] Based on operator policy and agreement with 3rd party, the 6G network shall provide a secure mechanism for distributed network subscribers to temporarily access services from a network (PLMN or distributed network) different to the subscribed distributed network without registered on the network.

NOTE 2:	It is assumed that the subscribed distributed network and the network providing services are of the same network operator.

[PR 5.x.6-9] Based on operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism to enable user subscription data synchronization between a distributed network and the PLMN.

[PR 5.x.6-10] Based on operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism that supports independent operation and maintenance of a distributed network.

* * *End of Change * * * *