3GPP TSG SA WG 1 Meeting #108 	S1-244795

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-244126,4546)

Source:	CATT, ISSDU, China Mobile, Nokia

pCR Title:	Update on 5.3 Use case on Satellite Communication with Resilient Operation Mode for Public Safety

Draft Spec:	3GPP TR 22.887 v0.2.0

Agenda item:	7.3

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes to update existing identified requirements and capture additional potential requirements of use case 5.3 to TR22.887.

1. Introduction

The use case about satellite communication with resilient operation mode for public safety was first proposed in SA1 #106 meeting and then captured to TR22.887 v.0.1.0. With further study, the meaning of resilient operation need to clarified and the wording of PR1 & PR2 need to be modified to clarify the applicability, and more requirements relevant to this use case can be identified and suggest to be included.

2. Reason for Change

Resilient operation needs an explanation to clarify the technical focus behind. Also, the following changes will be included:

(1) modify the pre-condition and service flow to clarify the use case and align with PRs;

(2) remove PR1 and modify PR2 to clarify the role of the operator;

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.887 v0.2.0

* * * First Change * * * *

5.3	Use case on Resilient Satellite Communication for Public Safety

5.3.1	Description

This use case describes a scenario where the resilient satellite communication system for public safety can provide resilient services to UEs with proper operation (a.k.a resilient operation) when either the backhaul link between the LEO satellite and the ground station is temporarily unavailable due to a natural disaster or the core network of LEO satellite was temporally unavailable due to cyberwarfare attack.

In order to preserve service continuity, the resilient operation could eliminate service interruption period before re-establish the backhaul link or restore the core network service.

Figure 5.3.1-1: Scenario for Resilient Satellite Communication for Public Safety

5.3.2	Pre-conditions

The Coast Guard has deployed a smart boat named Virginia for river patrol. Officers Jason and Alice can navigate the boat while reporting their status periodically to the Coast Guard Administration Centre.

Multiple core networks can be selected for disaster scenarios.

The LEO satellite can connect to GEO/MEO via ISLs or directly to the ground 5G core network via feeder link.

5.3.3	Service Flows

1. Connectivity to land 5GC:

- Both officers’ smartphones and their smart boat are connected to the 5G network via a LEO satellite link.

- They periodically report their current status to the Coast Guard Administration Centre.

2. Interruption of backhaul link:

- An EF5 tornado develops and moves toward the satellite ground station.

- The backhaul link between the LEO satellite and the ground station becomes temporarily unavailable because the ground station's antennas are damaged by the EF5 tornado.

3. Resilient operation to maintain service:

- The satellite communication system maintains the service after the LEO satellite loses the backhaul link connection.

- Officer Jason and Alice can still report their status to the Coast Guard Administration Centre through core network, and communicate with each other e.g. via UE-Satellite-UE communication.

4. Re-establish the connectivity to original land 5GC when it is available again.

5.3.4	Post-conditions

During the river patrol,

Officers Jason and Alice can report status to the Coast Guard Administration Centre as expected.

5.3.5	Existing features partly or fully covering the use case functionality

Existing requirements from clause 6.46.9 in TS 22.261 [2]:

Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support UE-Satellite-UE communication regardless of whether the feeder link is available or not.

Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide QoS control of a UE-Satellite-UE communication.

Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support different types of UE-Satellite-UE communication (e.g. voice, messaging, broadband, unicast, multicast, broadcast).

5.3.6	Potential New Requirements needed to support the use case

[PR 5.3.6-001] Subject to operator’s policy, the 5G network with satellite access shall be able to maintain the connection to an available core network with the same service level agreement (SLA) for resilient satellite communication.

NOTE: The resilient communication can be achieved through a single satellite or multiple satellites over different types of orbits.

* * * End of Changes * * * *