3GPP TSG SA WG 1 Meeting #108 	S1-244808

Orlando, US, 18-22 November 2024	(revision of S1-244559)

Source:	China Mobile, China Unicom

pCR Title:	Consideration on Legacy telephony service and supplementary services support of 6G

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.1

Document for:	Approval

Contact:	Xiaonan Shi,shixiaonan@chinamobile.com

Xueqian Bai,baixueqian@chinamobile.com

Qun Wei,weiqun5@chinaunicom.cn

Han Li,han.li@chinaunicom.cn

Abstract: This document provides a proposal on legacy telephony service and supplementary services support of 6G.

1. Introduction

It is a widely discussed that as a basic telephony service, voice should be supported in next generation of the 3GPP network in Day-1[1][2][3]. In the past decades, lots of telephony service and supplementary services related to the voice service have been specified in 3GPP for CS and IMS networks. Only few of these services are widely used in the existing networks. Take the existing telephony service and supplementary services as an exmaple:

- Some of which are outdated for 2/3G sunset.

- Some of which are only defined in the stage 1 of 3GPP work and ignored by the downstream working group and other mainstream implementation organizations for voice:

  - I.E. MCID (Malicious Communicaiton IDentification) and CCNL (Completion of Communications on Not Logged-in) are not specified in 3GPP Release 18 TS 24.173.

  - Only 11 out of 25 services are mandatory to be supported by the UE and the TAS (Telephony Application Server) accroding to GSMA PRD IR.92[4] and NG.114[5].

- Some of which are seldom used due to high costs:

  - Compared with decades, the penetration rate of fixed/mobile terminals has been greatly improved and the communication fee has been greatly reduced. Making calls has become a daily routine for the common people. Services that are designed based on high communication fee, such as Reverse Charging, have been rarely used.

- Some of which are seldom used due to popular alternatives with much better user experience:

  - The USSD service provides the UE or network initiated MMI-mode USSD operations to enable the transparent transport of MMI strings between UE and IMS network as specified in 3GPP Release18 in TS 24.390. It's used for service configuration but required user to remember several complicated USSD number such as #*6100#. Now there're more convinience and efficient ways for online personal service configuration change like service Apps, mini-Apps, hence the USSD have been rarely used.

  - The CCBS (Completion of Communications to Busy Subscriber), CCNR (Completion of Communication on No Reply) and CCNL (Completion of Communications on Not Logged-in) services enable the calling user, when encountering a destination that is busy, does not answer or is not logged-in, to have the communication completed at a later point in time by network reinitiating a new communication attempt as specified in 3GPP Release18 in TS 24.642. Now the operators usually send a SMS to notify the called party of the missed call due to busy or unanswered call. It is the alternative of the CCBS/CCNR/CCNL.

Reference:

[1] https://www.ngmn.org/wp-content/uploads/NGMN_6G_Requirements_and_Design_Considerations.pdf

[2] https://nextgalliance.org/white_papers/6g-applications-and-use-cases/

[3]GSMA proposal in 6G in 3GPP SA1 Workshop on IMT2030 Use Cases

[4] https://www.gsma.com/newsroom/wp-content/uploads/IR.92-v15.0.pdf

[5] https://www.gsma.com/newsroom/gsma_resources/ng-114-ims-profile-for-voice-video-and-messaging-over-5gs-v-8-1/

2. Reason for Change

The inherit of all legacy services supported in 5G system will be a huge burden to 3GPP network and lead to complex network architecture. Therefore, it is necessary to identify what kinds of the legacy multimedia telephony services and supplementary services need to be supported for backwards compatibility.

The following principles are recommended:

-	Interworking and roaming with CS shall not be supported.

-	CS Fall-back shall not be supported.

-	Telephony service which are specific for CS network shall not be supported.

-

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

5.X	Consideration on 6G multimedia services support

5.1.1	Description

Current 3GPP specification provides multimedia service requirements, see TS 22.101, clause 7.2.

It is proposed that 6G will continue to support multimedia services (e.g. voice and video).

5.1.2	Potential New Requirements needed to support the use case

The 6G system shall support multimedia (e.g. voice and video) communication services.

The following are not supported in the 6G system:

-	Interworking and roaming with CS.

-	CS Fallback.

-	Telephony service which are specific for CS network.

Editor's note: further exceptions are FFS.

* * * End Of Change * * * *