3GPP TSG-SA WG1 Meeting #76bis	S1-170007

Spokane, WA, USA, 16-20 January 2017	(revision of S1-17xxxx)

Title:	SMARTER – device clean up

Agenda Item:	5.1

Source:				Nokia, Alcatel-Lucent Shanghai Bell

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes to replace device with UE.  This proposal is contingent upon reaching agreement on a definition for UE that encompasses both ‘traditional’ UEs and IoT devices.

First Change

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Editor's Note: The definitions of active communication and service continuity propose changes to 21.905 and need to be propagated once agreed in SA1. To add to 21.905 we need to consider another note for service continuity, e.g.,

NOTE x: 	Examples of access changes include the following. From pre-5G: CS/PS domain change. From 5G: switching between a direct 3GPP connection and an indirect 3GPP connection.

3GPP connection: The connection between a UE and a 3GPP network.

Active communication: a UE is in active communication when it has one or more connections established. A UE may have any combination of PS connections (e.g., PDP contexts, active PDN connections).

Availability (%): Percentage value of the amount of time the 3GPP system is delivering services divided by the amount of time it is expected to deliver services in a specific area.

Direct 3GPP connection: One mode of 3GPP connection, where there is no relay UE between the remote UE and the 3GPP network.

Direct UE connection: The connection between two UEs without any network entity in the middle.

Hosted Service: A service containing the operator’s own application(s) and/or trusted 3rd party application(s) in the Hosting Service Environment, which can be accessed by the user.

Indirect 3GPP connection: One mode of 3GPP connection, where there is a relay UE between the remote UE and the 3GPP network.

Network slice: A set of network functions and corresponding resources necessary to provide the required telecommunication services and network capabilities.

Reliability (%): The amount of sent network layer packets successfully delivered to a given node within the time constraint required by the targeted service, divided by the total number of sent network layer packets.

Satellite access: Direct connectivity between the user terminal and the satellite.

Service Continuity: The uninterrupted user experience of a service that is using an active communication when a UE undergoes an access change without, as far as possible, the user noticing the change.

NOTE 1: 	In particular Service Continuity encompasses the possibility that after a change the user experience is maintained by a different telecommunication service (e.g. tele- or bearer service) than before the change.

NOTE 2: 	Examples of access changes include the following. From pre-5G: CS/PS domain change. From 5G: switching between a direct 3GPP connection and an indirect 3GPP connection.

Service Hosting Environment: The environment, located inside of 3GPP network and fully controlled by the operator, where Hosted Services are offered from.

Tactile internet: System where a human controls real or virtual objects by tactile input, and receives tactile, audio, or visual feedback of his actions. The feedback has to arrive fast enough to give the impression of zero-latency control.

Wireless Backhaul: provides interconnection link between 3GPP network nodes and/or 3GPP transport network using 3GPP radio access technology.

Second Change

# 4	Overview

Unlike previous 3GPP systems that attempted to provide a 'one size fits all' system, the 5G system is expected to be able to provide optimized support for a variety of different services, different traffic loads, and different end user communities. Various industry white papers, most notably, the NGMN 5G White Paper [2], describe a multi-faceted 5G system capable of simultaneously supporting multiple specific combinations of reliability, latency, throughput, location accuracy, and availability. This technology revolution is achievable with the introduction of new technologies, both in access and the core, such as flexible, scalable assignment of network resources. In addition to increased flexibility and optimization, a 5G system needs to support stringent KPIs for latency, reliability, and throughput. Enhancements in the air interface contribute to meeting these KPIs as do enhancements in the core network, such as network slicing, in-network caching and hosting services closer to the end points.

As outlined in TR 22.891 [3], a 5G system also supports new business models such as those for IoT and enterprise managed networks. Drivers for the 5G KPIs include services such as drone control, augmented reality, and factory automation.  Network flexibility enhancements support self-contained enterprise networks, installed and maintained by network operators while being managed by the enterprise. Enhanced connection modes and evolved security facilitate support of massive IoT, expected to include tens of millions of new UEs sending and receiving data over the 3GPP 5G network.

Flexible network operations are the mainstay of the 5G system.  The capabilities to provide this flexibility have been described in TR 22.864 [4], including network slicing, network capability exposure, scalability, and diverse mobility. Other network operations requirements address the necessary control and data plane resource efficiencies, as well as network configurations that optimize service delivery by minimizing routing between end users and application servers. Enhanced charging and security mechanisms handle new types of UEs connecting to the network in different ways.

MBB enhancements have been identified in TR 22.863 [5] to meet a number of new KPIs. These pertain to high data rates, high density, high user mobility, highly variable data rates, and deployment and coverage. High data rates are driven by the increasing use of data for services such as streaming (e.g., video, music, and user generated content), interactive services (e.g., augmented reality), and IoT (e.g., drone control). These services come with stringent requirements for user experienced data rates (including downlink and uplink) as well as associated requirements pertaining to latency to meet service performance expectations.  Additionally, increased coverage in densely populated areas such as sports arenas, urban areas, and transportation hubs has become essential for pedestrians and users in urban vehicles.  New KPIs on density enable both the transport of high volumes of data traffic per area (traffic density) and transport of data for a high number of connections (UE density or connection density). Many 5G UEs are expected to support a variety of services which exchange either a very large (e.g., streaming video) or very small amount of data (e.g., data burst). The 5G system will handle this variability in a resource efficient manner.  All of these cases introduce new deployment requirements for indoor and outdoor, local area connectivity, wide area connectivity, and UEs travelling at high speeds.

Another aspect of 5G KPIs includes requirements for various combinations of latency and reliability, as well as higher accuracy for positioning. These KPIs, as identified in TR 22.862 [6], are driven by support for both commercial and public safety services. On the commercial side, new use cases are expected for factory automation, industrial process automation, drone and other UAV control, augmented reality, and more.  Services such as UAV controls will require more precise positioning information that includes altitude, speed, and direction, in addition to horizontal coordinates.

Support for MIoT brings many new requirements in addition to those for the enhanced KPIs, as have been identified in TR 22.861 [7]. The expansion of connected things introduces a need for significant improvements in resource efficiency in all system components (e.g., UEs, radio, access network, core network).

Third Change

### 5.1.2	Requirements

#### 5.1.2.1	Interworking between 5G systems

The 5G system shall provide support for a UE with a 5G subscription roaming into a 5G Visited Mobile Network which has a roaming agreement with the UE’s 5G Home Mobile Network.

The 5G system shall enable a Visited Mobile Network to provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

The 5G system shall enable a Visited Mobile Network to provide support for services provided in the home network as well as provide services in the visited network. Whether a service is provided in the visited network or in the home network is determined on a service by service basis.

The 5G system shall provide a mechanism for a network operator to limit access to its services for a roaming UE, (e.g., based on roaming agreement).

The 5G system shall provide a mechanism for a network operator to direct a UE onto a partnership network for routing all or some of the UE user plane traffic over the partnership network, subject to a bilateral agreement between the operators being in place.

#### 5.1.2.2	Legacy Service Support

The 5G system shall support all EPS capabilities (e.g., from TSs 22.011, 22.101, 22.278, 22.185, 22.071, 22.115, 22.153, 22.173) with the following exceptions:

-	CS voice service continuity and/or fallback to GERAN or UTRAN,

-	Seamless handover between the 5G RAN and GERAN or UTRAN, and

-	Access to a 5G core network via GERAN or UTRAN.

Fourth Change

### 6.1.2	Requirements

The 5G system shall allow the operator to create, modify, and delete a network slice.

The 5G system shall allow the operator to define and update the set of services supported in a network slice.

The 5G system shall allow the operator to assign a UE to a network slice based on subscription, UE type, and services provided by the network slice.

The 5G system shall allow the operator to configure the information which associates a UE or a service to a network slice.

The 5G system shall allow the operator to assign a UE to a network slice, to move a UE from one network slice to another, and to remove a UE from a network slice.

The 5G system shall support a mechanism for the VPLMN to assign a UE to a network slice with the needed services and authorised by the HPLMN, or to a default network slice.

The 5G system shall enable a UE to be simultaneously assigned to and access services from more than one network slice of one operator.

Traffic and services in one network slice shall have no impact on traffic and services in other network slices in the same network.

Creation, modification, and deletion of a network slice shall have no or minimal impact on traffic and services in the same network.

The 5G system shall support the adaptation of capacity, i.e. elasticity of capacity of a network slice.

The 5G system shall ensure that elasticity of capacity of one slice has no impact on services provided by other network slices.

The 5G system shall support means by which the operator can add and remove network functions to the network such that they can be used in a network slice.

The 5G system shall support providing connectivity to home and roaming users in the same network slice.

Editor's Note: Use of "UE type" needs to be clarified.

## 6.2	Diverse mobility management

### 6.2.1	Description

A key feature of 5G is support for UEs with different mobility management needs. 5G will support UEs with a range of mobility management needs, including UEs that are

-	stationary during their entire usable life (e.g., sensors embedded in infrastructure),

-	stationary during active periods, but nomadic between activations (e.g., fixed access),

-	mobile within a constrained and well-defined space (e.g., in a factory), and

-	full mobility.

Moreover, some applications require the network to ensure seamless mobility of the UE so that mobility is hidden from the application layer to avoid interruptions in service delivery while other applications have application specific means to ensure service continuity. But these other applications may still require the network to minimize interruption time to ensure that their application-specific means to ensure service continuity work effectively.

With the ever-increasing multimedia broadband data volumes, it is also important to enable the offloading of IP traffic from the 5G network onto traditional IP routing networks via an IP anchor node close to the network edge. As the UE moves, changing the IP anchor node may be needed in order to reduce the traffic load in the system, reduce end-to-end latency and provide a better user experience.

The flexible nature of a 5G system will support different mobility management methods that minimize signalling overhead and optimize access for these different types of UEs.

### 6.2.2	General requirements

The 5G network shall allow operators to optimize network behaviour (e.g., mobility management support) based on the mobility patterns (e.g., stationary, nomadic, spatially restricted mobility, full mobility) of a UE or group of UEs.

The 5G system shall enable operators to specify and modify the types of mobility support provided for a UE.

The 5G system shall optimize mobility management support for a UE or group of UEs that use only mobile originated communications.

### 6.2.3	Service continuity requirements

The 5G system shall enable packet loss to be minimized during inter- and/or intra- access technology changes for some or all connections associated with a UE.

For applications that require a constant IP address, the 5G system shall enable maintaining the IP address assigned to a UE when moving across different cells and access technologies for connections associated with a UE.

The 5G system shall enable minimizing impact to the user experience (e.g., minimization of interruption time) when changing the IP address and IP anchoring point for some or all connections associated with a UE.

The 5G system shall support service continuity for a remote UE, when the remote UE changes from one relay UE to another and both relay UEs use 3GPP access to the 5G core network.

## 6.3	Multiple access technologies

### 6.3.1	Description

The 5G system will support 3GPP access technologies, including one or more 5G RATs (e.g., optimized for different purposes) and E-UTRA as well as non-3GPP access technologies. Interoperability among the various access technologies will be imperative. For optimization and resource efficiency, the 5G system will select the most appropriate 3GPP or non-3GPP access technology for a service, potentially allowing multiple access technologies to be used simultaneously for one or more services active on a UE.  New technology such as satellite and wide area base stations will increase coverage and availability. This clause provides requirements for interworking with the various combinations of access technologies.

Editor's Note: The naming as well as the consistency of usage of the different radio access technologies needs further analysis and alignment.

Editor's Note: The service requirements regarding the need for or level of interworking with earlier generation of 3GPP systems needs to be clarified.

### 6.3.2	Requirements

#### 6.3.2.1	General

Based on operator policy, the 5G system shall enable the UE to select, manage, and efficiently provision services over the 3GPP or non-3GPP access.

Based on operator policy, the 5G system shall support steering a UE to select certain 3GPP access network(s) (e.g., 5G RAT, evolved E-UTRA).

Based on operator policy, the 5G system shall be able to dynamically offload part of the traffic (e.g. from 3GPP RAT to non-3GPP access technology), taking into account traffic load and traffic type.

Based on operator policy, the 5G system shall be able to provide simultaneous data transmission via different access technologies, (e.g., 5G RAT, evolved E-UTRA, non-3GPP), to access one or more 3GPP services.

When a UE is using two or more access technologies simultaneously, the 5G system shall be able to select different access technologies taking into account e.g., service, traffic characteristics, radio characteristics, and UE’s moving speed.

The 5G system shall be able to support data transmissions optimized for different access technologies (e.g., 3GPP, non-3GPP) for UEs that are simultaneously connected to the network via different accesses.

Based on operator policy, the 5G system shall be able to add or drop the various access connections for a UE during a session.

The 5G system shall be able to support inter-system mobility between the supported access networks (e.g., 5G, E-UTRAN, WLAN, fixed broadband).

The 5G system shall support UEs with multiple radio and single radio capabilities.

The 5G system shall support dynamic and static network address allocation of a common network address to the UE over all supported access types.

The 5G system shall support a set of identities for a single user in order to provide a consistent set of policies and a single set of services across 3GPP and non-3GPP access types.

## 6.4	Resource Efficiency

### 6.4.1	Description

5G introduces the opportunity to design a 3GPP system to be optimized for supporting diverse UEs and services. While support for IoT is provided by EPS, there is room for improvement in efficient resource utilization that can be designed into a 5G system whereas they are not easily retrofitted into an existing system. Some of the underlying principles of the potential service and network operation requirements associated with efficient configuration, deployment, and use of UEs in the 5G network include bulk provisioning, resource efficient access, optimization for UE originated data transfer, and efficiencies based on the reduced needs related to mobility management for stationary UEs and UEs with restricted range of movement.

As sensor and monitoring UEs are deployed more extensively, the need to support UEs that send data packages ranging in size from a small status update in a few bits to streaming video increases. A similar need exists for smart phones with widely varying amounts of data. Specifically, to support short data bursts, the network should be able to operate in a mode where there is no need for a lengthy and high overhead signalling procedure before and after small amounts of data are sent. The system will, as a result, avoid both a negative impact to battery life for the UE and wasting signalling resources.

For small form factor UEs it will be challenging to have more than 1 antenna due to the inability to get good isolation between multiple antennas. Thus these UEs need to meet the expected performance in a 5G network with only one antenna.

Additional resource efficiencies will contribute to meeting the various KPIs defined for 5G. Control plane resource efficiencies can be achieved by optimizing and minimizing signalling overhead, particularly for small data transmissions. Mechanisms for minimizing user plane resources utilization include in-network caching and application in a Service Hosting Environment closer to the end user. These optimization efforts contribute to achieving lower latency and higher reliability.

Diverse mobility related resource efficiencies are covered in clause 6.2.

Security related resource efficiencies are covered in clause 8.8.

### 6.4.2	Requirements

#### 6.4.2.1	General

The 5G system shall minimize control and user plane resource usage for data transfer from send only UEs.

The 5G system shall minimize control and user plane resource usage for stationary UE (e.g., lower signalling to user data resource usage ratio).

The 5G system shall minimize control and user plane resource usage for transfer of infrequent small data units.

The 5G system shall optimize the resource use of the control plane and/or user plane for transfer of small data units.

The 5G network shall optimize the resource use of the control plane and/or user plane to support high density connections (e.g., 1 million connections per square kilometre) taking into account, for example, the following criteria:

-	the type of mobility support

-	Communication pattern (e.g., send only)

-	Characteristics of payload (e.g., infrequent or not, small size data payload, etc.)

-	Characteristics of application (e.g., provisioning operation, normal data transfer, etc.)

-	Geographic information of aUE

The 5G system shall efficiently support service discovery mechanisms where UEs can discover, subject to access rights:

-	status of other UEs (e.g., sound on/off);

-	capabilities of other UEs (e.g., the UE is a relay UE) and/or;

-	services provided by other UEs (e.g., the UE is a colour printer).

The 5G system shall be able to minimise the amount of wireless backhaul traffic (e.g., consolidating data transmissions to 1 larger rather than many smaller), when applicable (e.g., providing service in an area subject to power outages).

The 5G system shall support small form factor UEs with single antenna.

NOTE:	Small form factor UEs are typically expected to have the diagonal less than 1/5 of the lowest supported frequency wave length.

#### 6.4.2.2	Efficient bulk operations for IoT

The 5G network shall optimize the resource use of the control plane and/or user plane to support high density connections (e.g., 1 million connections per square kilometre) of grouped UEs.

The 5G network shall optimize the resource use of the control plane and/or user plane to manage (e.g., provide service parameters, activate, deactivate) groups of UEs.

The 5G system shall support a timely, efficient, and/or reliable mechanism to transmit the same information to multiple UEs.

#### 6.4.2.3	Efficient management for IoT

The 5G network shall optimize the resource use of the control plane and/or user plane to manage (e.g., provide service parameters, activate, deactivate) a UE.

#### 6.4.2.4	Efficient control plane

The 5G system shall minimize the signalling that is required prior to user data transmission.

NOTE:	The amount of signalling overhead may vary based on the amount of data to be transmitted, even for the same UE.

## 6.6	Efficient content delivery

### 6.6.1	Description

Video based services (e.g., live streaming, VR), personal data storage applications have been instrumental for the massive growth in mobile broadband traffic. Subject to service agreement between the operator and content provider, the information of content and/or content itself can be aware by operator. In-network content caching including both operator provided and 3rd party provided can improve user experience, reduce backhaul resource usage and utilize limited radio resource efficiently.

The operation of in-network caching includes flexible management of the location of the content cache within the network and efficiently delivery of content from/to the appropriate content caching application. For example: delivering popular video content from a content caching application via broadcast, and securely storing the personal data/files of a user in distributed caching application. Such a service could also provide a student wireless backpack, where students can resume their work through the same or a different UE at a time convenient to them, with very fast response times from the network.

## 6.7		Priority, QoS and Policy Control

### 6.7.1		Description

The 5G network will be supporting many commercial services (e.g., medical) and regional or national regulatory specific services (e.g., MPS, Emergency, Public Safety) with requirements for priority treatment. Some of these priority services share common QoS characteristics such as latency and packet loss rate, but may have different priority requirements.  For example, drone control and air traffic control may have stringent latency and packet loss rate requirements but not necessarily the same priority requirements. In addition, voice based services for MPS and Emergency share common QoS characteristics as applicable for normal public voice communications, yet may have different priority requirements. The 5G network will need to support mechanisms that enable the decoupling of the priority of a particular communication from the associated QoS characteristics such as latency and packet loss rate to allow flexibility to serve different priority services (that need to be configurable to meet operator needs, consistent with operator policies and corresponding national / regional regulatory policies).

The network needs to support flexible means to make priority decisions based on the state of the network (e.g., during disaster events and network congestion) recognizing that the priority needs may change in time during a crisis. The priority of any given service may need to be different (e.g., elevated) for a particular user of that service based on operational needs and regional or national regulations. Therefore, the 5G system should allow a flexible means to make and enforce priority decisions among the different services (e.g., MPS, Emergency, medical, Public Safety) and amongst users of these services.

The network must offer a means to provide the required QoS (e.g., high reliability, latency, bandwidth, etc.) for a service and the ability to prioritize resources when necessary to meet the specific service requirements. Existing QoS and policy frameworks handle latency and improve reliability by traffic engineering. In order to support diverse 5G service requirements, it is necessary for the 5G network to offer QoS and policy control for reliable communication with latency required for a service and enable the resource adaptations as necessary.

Also, as 5G network is expected to operate in a heterogeneous environment with multiple access technologies, multiple types of UEs, etc., it should support a harmonised QoS and policy framework that applies across multiple accesses.

Further, for existing EPC, QoS control only covers RAN and core network, but for 5G network E2E QoS (e.g., RAN, backhaul, core network, network to network interconnect) is needed to achieve the 5G user experience (e.g., ultra-low latency, ultra-high bandwidth).

## 6.9	Connectivity models

### 6.9.1	Description

The UE can connect with the network directly or connect with the network using another UE as a relay UE, or they may be capable of using both types of connections. The scope of such UEs can be from simple wearables, such as a smart watch or a set of sensors embedded in clothing, to a more sophisticated wearable UE monitoring biometrics. They can also be non-wearable UEs that communicate in a Personal Area Network such as a set of home appliances (e.g., smart thermostat and entry key), or the electronic UEs in an office setting (e.g., smart printers), or a smart flower pot that can be remotely activated to provide water to the plant. The relay UE can access the network using 3GPP or non-3GPP access (e.g., WLAN, fixed broadband access). 3GPP and non-3GPP radio technologies can be supported for connectivity scenarios between the remote UE and the relay UE. 3GPP RAT over licensed band or unlicensed band can be supported for connectivity scenarios.

At the time of connecting a remote UE via a relay UE, there might be several relay UEs, supporting discovery and remote UE-to-relay UE connection establishment, available and in proximity of the remote UE. A remote UE may support selection of an appropriate relay UE among the available relay UEs.

### 6.10.2	Requirements

Based on operator policy, a 5G network shall provide suitable APIs to allow a 3rd party to create, modify and delete network slices used for the 3rd party.

Based on operator policy, the 3GPP system shall allow a 3rd party to define and update the set of services supported in a network slice.

Based on operator policy, the 3GPP system shall allow a 3rd party to assign a UE to a network slice based on subscription, UE type, and services provided by the network slice.

The 5G network shall provide a mechanism to expose broadcasting capabilities to 3rd party broadcasters’ management systems.

Based on operator policy, a 5G network shall provide suitable APIs to allow a trusted 3rd party to manage this trusted 3rd party owned application(s) in the operator's Service Hosting Environment.

## 6.14	Subscription aspects

### 6.14.1	Description

With the Internet of Things, it is expected that the diversity of UE types and the usage models will largely vary. Moreover, when the UE is manufactured, the deployment location and specific usage may not be known. Sometimes the UEs will be added to existing subscriptions, other times they may be part of a new subscription for the user. Sometimes the UEs may be leased. During their life cycle these UEs go through different stages, involving the change in ownership when the UE is deployed and possibly afterwards, the activation of the UE by the preferred operator, a possible change of operators, etc. These stages need to be managed securely and efficiently. A method of dynamic subscription generation and management is needed in addition to statically provisioned subscription. Once the subscription is established, subscription management becomes necessary, for example, to modify the subscription when the ownership of the UE changes, to update or refresh credentials due to suspected leakage or theft of security keys or as a preventive measure, etc.

The Internet of Things will also support various connectivity models: The UEs can connect with the network directly or connect with the network using another UE as a relay UE, or they may be capable of using both types of connections. The direct connection between the UE and the relay UE can be using 3GPP or non-3GPP RAT. The relay UE can access the network also using 3GPP or non-3GPP access networks (e.g., WLAN, fixed broadband access). In order to identify and manage the UEs, a subscription with the 5G network is needed, even if the access is done via non-3GPP access.

### 6.14.2	Requirements

A UE which is able to access the 5G network in direct 3GPP connection mode using a 3GPP RAT shall have a 3GPP subscription.

The 5G system shall be able to provide mechanisms to change the association between a subscription and address/number of a UE (e.g., changing the owner and subscription information associated with the UE) within the same operator and in between different operators in an automated or manual way.

The 5G system shall be able to support identification of subscriptions independently of identification of UEs. Both identities shall be secure.

A UE which is able to connect to another UE in direct UE connection mode shall have a 3GPP subscription, if the remote UE needs to be identifiable by the core network (e.g., for UE management purposes or to use indirect 3GPP connection mode).

Based on operator policy, the 5G system shall support a mechanism to provision on-demand connectivity. This on-demand mechanism should enable means for a user to request on-the-spot network connectivity while providing operators with identification and security tools for the provided connectivity.

## 6.15	Energy Efficiency

### 6.15.1	Description

Energy efficiency is a critical issue in 5G. The potential to deploy systems in areas where there is not a reliable energy source requires new methods of managing energy consumption not only in the UEs but throughout all components of the 5G system.

Small form factor UEs also typically have a small battery and this not only puts constrains on general power optimization but also on how the energy is consumed. With smaller batteries it is more important to understand and follow the limitations for the both the max peak and continuous current drain.

### 6.15.2	Requirements

The 5G access network shall support an energy saving mode with the following characteristics:

-	The energy saving mode can be activated/deactivated either manually or automatically.

-	Service can be restricted to a pre-defined set of users (e.g., public safety user, emergency callers).

NOTE:	When in energy saving mode the UE’s and Access transmit power may be reduced or turned off (deep sleep mode), latency and jitter may be increased with no impact on set of users or applications still allowed.

The 5G system shall support mechanisms to improve battery life for a UE over what is possible in EPS.

The 5G system shall optimize the battery consumption of a relay UE via which a UE is in indirect 3GPP connection mode.

The 5G system shall support UEs using small rechargeable and single coin cell batteries (e.g. considering impact on max pulse and continuous current).

## 6.16	Markets requiring minimal service levels

### 6.16.1	Description

A key aspect of 5G system flexibility is the ability to support both the very high end markets as well as very low end markets. Some systems will be deployed in areas where there are constraints on energy resources (e.g., sporadic access to power supplies) and lower end user expectations for availability, reliability, and capacity. In such cases, the system needs additional flexibility to adapt power consumption needs based on fluctuations in power availability. The system should be efficient in order to provide essential services in harsh environments (e.g., far remote rural areas, very large territories) while taking into account the local constraints (adapting resources consumptions to long distances, dealing with variable conditions and possibly disconnections). Content delivery should be optimized in order to reduce constraints on transport networks, on low-end user UEs (e.g., limited screen size, limited energy consumption), variable network conditions, and client profiles.

Fifth Change

# 7	Performance requirements

## 7.1	High data rates and traffic densities

Several scenarios require the support of very high data rates or traffic densities of the 5G system. The scenarios address different service areas: urban and rural areas, office and home, and special deployments (e.g., massive gatherings, broadcast, residential, and high-speed vehicles). The scenarios and their performance requirements can be found in table 7.1-1.

-	Urban macro - The general wide-area scenario in urban area

-	Rural macro - The general wide-area scenario in rural area

-	Indoor hotspot - The scenario for offices and homes, and residential deployments.

-	Broadband access in a crowd - The scenario for very dense crowds, for example, at stadiums or concerts. In addition to a very high connection density the users want to share what they see and hear, putting a higher requirement on the uplink than the downlink.

-	Dense urban - The scenario for pedestrian users, and users in urban vehicles, for example, in offices, city centres, shopping centres, and residential areas. The users in vehicles can be connected either directly or via an onboard base station to the network.

-	Broadcast-like services - The scenario for stationary users, pedestrian users, and users in urban vehicles, for example, in offices, city centres, shopping centres, and residential areas. The passengers in vehicles can be connected either directly or via an onboard base station to the network.

-	High-speed train - The scenario for users in trains. The users can be connected either directly or via an onboard base station to the network.

-	High-speed vehicle - The scenario for users in road vehicles. The users can be connected either directly or via an onboard base station to the network.

-	Airplanes connectivity - The scenario for users in airplanes. The users can be connected either directly or via an onboard base station to the network.

Table 7.1-1 Performance requirements for high data rate and traffic density scenarios.

|  | Scenario | Experienced data rate (DL) (NOTE 4) | Experienced data rate (UL) (NOTE 4) | Area traffic capacity (DL) (NOTE 5) | Area traffic capacity (UL) (NOTE 5) | User density | Activity factor (NOTE 6) | UE speed | Coverage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a | Urban macro | 50 Mbps | 25 Mbps | 100 Gbps/km2 | 50 Gbps/km2 | 10000/km2 | 20% | Pedestrians and users in vehicles (up to 120 km/h | Full network (NOTE 1) |
| 1b | Rural macro | 50 Mbps | 25 Mbps | 1 Gbps/km2 | 0.5 Gbps/km2 | 100/km2 | 20% | Pedestrians and users in vehicles (up to 120 km/h | Full network (NOTE 1) |
| 2 | Indoor hotspot | 1 Gbps | 500 Mbps | 15 Tbps/km2 | 2 Tbps/km2 | 250000/km2 | NOTE 2 | Pedestrians | Office and residential (NOTE 2) (NOTE 3) |
| 3 | Broadband access in a crowd | 25 Mbps | 50 Mbps | [3.75] Tbps/km2 | [7.5] Tbps/km2 | [150000]/km2 | 30% | Pedestrians | Confined area |
| 4 | Dense urban | 300 Mbps | 50 Mbps | 750 Gbps/km2 | 125 Gbps/km2 | 25000/km2 | 10% | Pedestrians and users in vehicles (up to 60 km/h) | Downtown (NOTE 1) |
| 5 | Broadcast-like services | Maximum 200 Mbps (per TV channel) | N/A or modest (e.g., 500 kbps per user) | N/A | N/A | [15] TV channels of [20 Mbps] on one carrier | N/A | Stationary users, pedestrians and users in vehicles (up to 500 km/h) | Full network (NOTE 1) |
| 6a | High-speed train | 50 Mbps | 25 Mbps | 15 Gbps/train | 7.5 Gbps/train | 1000/train | 30% | Users in trains (up to 500 km/h) | Along railways (NOTE 1) |
| 6b | High-speed vehicle | 50 Mbps | 25 Mbps | [100] Gbps/km2 | [50] Gbps/km2 | 4000/km2 | 50% | Users in vehicles (up to 250 km/h) | Along roads (NOTE 1) |
| 7 | Airplanes connectivity | 15 Mbps | 7.5 Mbps | 1.2 Gbps/plane | 600 Mbps/plane | 400/plane | 20% | Users in airplanes (up to 1000 km/h) | (NOTE 1) |
| NOTE 1: For users in vehicles, the UE can be connected to the network directly, or via an on-board moving base station. NOTE 2: A certain traffic mix is assumed; only some users use services that require the highest data rates [2]. NOTE 3: For interactive audio and video services, for example, virtual meetings, the required two-way end-to-end latency (UL and DL) is 2-4ms while the corresponding experienced data rate needs to be up to 8k 3D video [300Mbps] in uplink and downlink. NOTE 4: User experienced data rate is the minimum data rate required to achieve a sufficient quality experience, with the exception of scenario for broadcast like services where the given value is the maximum that is needed. NOTE 5: Area traffic capacity is the product of the user experienced data rate, the user density, and the activity factor. NOTE 6: Activity factor is a percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network. NOTE 7: All the values in this table are targeted values and not strict requirements. |  |  |  |  |  |  |  |  |  |

### 7.2.2	Services and KPIs

Services requiring low latency and high reliability can be categorized in the following groups:

-	Industrial control - Conventional industrial control is characterised by high requirements on the communications system regarding latency, reliability, and availability. Systems supporting industrial control are usually deployed in geographically limited areas but may also be deployed in wider areas (e.g., city- or country-wide networks), access to them may be limited to authorised users, and they may be isolated from networks or network resources used by other cellular customers.

-	Industrial automation - Industrial automation is characterised by high requirements on the communications system regarding reliability and availability. Systems supporting industrial automation are usually deployed in geographically limited areas, access to them may be limited to authorised users, and they may be isolated from networks or network resources used by other cellular customers.

-	Tactile interaction - Tactile interaction is characterised by a human being interacting with the environment or people, or controlling a UE, and relying on tactile feedback.

-	Audio-visual interaction - Audio-visual interaction is characterised by a human being interacting with the environment or people, or controlling a UE, and relying on audio-visual feedback.

-	Remote control - Remote control is characterised by a UE being operated remotely, either by a human or a computer.

Table 7.2.2-1 Performance requirements for low-latency and high-reliability services.

| Service | Latency | Reliability | Experienced data rate | Payload | Traffic density | Connection density | Coverage NOTE 1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Industrial control | 1ms | 99,999 % | 1 Mbps | Small | 1 Gbps/km2 | 1 000/km2 | Very high | NOTE 2 |
| Industrial automation | 10ms | 99,999 % | 1 Mbps | Small | 100 Mbps/km2 | 100/km2 | High |  |
| Tactile interaction | 0,5ms | [99,999 %] | [Low] | [Small] | [Low] | [Low] | [Very high] | NOTE 2 |
| Audio-visual interaction | [10ms] | [99,9 %] | [250 Mbps] | [Big] | [Low] | Low] | [High] | NOTE 3 |
| Remote control | [5ms] | [99,999 %] | [From low to 10 Mbps] | [Small to big] | [Low] | [Low] | [Very high] |  |
| NOTE 1: The coverage requirement applies to the service area, e.g., inside a factory. NOTE 2: Traffic prioritization and hosting services close to the end-user may be helpful in reaching the lowest latency values. NOTE 3: Audio-visual interaction requires very low-delay audio and video coding, and high video frame rates (e.g., 120 fps). NOTE 4: All the values in this table are targeted values and not strict requirements. |  |  |  |  |  |  |  |  |

Editor's Note: All values in this table are FFS.

Sixth Change

# 8	Security

## 8.1	Description

IoT introduces new UEs with different life cycles, including UEs with no user interface (e.g., embedded sensors), long life spans during which the UE may change ownership several times (e.g., consumer goods), and which may not be pre-provisioned (e.g., consumer goods). These drive a need for secure mechanisms to dynamically establish or refresh credentials and subscriptions. New access technologies, including licensed and unlicensed, 3GPP and non-3GPP, drive a need for access independent security that is seamlessly available while the UE is active. High-end smartphones, drones, and factory automation drive a need for protection against theft and fraud.  Expansion into enterprise, vehicular, and public safety markets drive a need for increased end user privacy protection. 5G security addresses all of these new needs while continuing to provide security consistent with prior 3GPP systems.

## 8.3	Authentication

The 5G network shall support a resource efficient mechanism for authenticating groups of UEs.

The 5G system shall support an efficient means to authenticate a user to a UE (e.g., biometrics).

The 5G system shall be able to support authentication over a non-3GPP access technology using 3GPP credentials.

The 5G system shall support operator controlled alternative authentication methods (i.e., alternative to AKA) with different types of credentials for network access for UEs used for IoT to allow new isolated deployment scenarios (e.g., for industrial automation).

## 8.4	Authorization

Based on operator policy, before establishing a direct UE communications using a non-3GPP access technology, UEs may use 3GPP credentials to determine if they are authorized to engage in direct UE communication.

Based on operator policy, the 5G system shall provide a means to verify whether a UE is authorized to use prioritized network access for a specific service.

## 8.5	Identity management

The 5G system shall provide a mechanism for an operator to allow access from a UE using a temporary identifier that hides its subscriber identity.

The 3GPP system shall provide a mechanism for an operator to allow access from a UE connected in an indirect connection using a temporary identifier that hides its subscriber identity.

The 5G HPLMN shall be able to associate a temporary identifier to a UE’s subscriber identity.

The 5G system shall be able to protect subscriber identity and other user identifying information from passive attacks.

Subject to regulatory requirements, the 5G system shall be able to protect subscriber identity and other user identifying information from active attacks.

The 3GPP system shall be able to allow the equipment identifier to be collected by legitimate entity regardless of UE’s user interface, when required.

The 5G system shall be able to support identification of subscriptions independently of identification of equipment.

The 5G system shall support a secure mechanism to collect system information while ensuring end-user and application privacy (e.g., application level information is not to be related to an individual user identity or subscriber identity and UE information is not to be related to an individual subscriber identity).

For a private network using 3GPP technology, the system shall support network access using identities, credentials, and authentication methods provided and managed by a 3rd party and supported by 3GPP.

Editor’s Note: The term “private network” needs to be defined.

## 8.7	Fraud protection

Subject to regulatory requirements, the 5G system shall support a secure mechanism allowing an authorized entity to disable from normal operation of a UE reported as stolen.

Subject to regulatory requirements, the 5G system shall support a secure mechanism allowing an authorized entity to re-enable a recovered stolen UE to normal operation.

The 5G system shall be able to protect user location information from passive attacks.

Subject to regulatory requirements, the 5G system shall be able to protect user location information from active attacks.

## 8.8	Resource efficiency

The 5G system shall minimize security signalling overhead without compromising the security level of the 3GPP system.

The 5G system shall support a secure mechanism to transmit the same data (e.g., service provisioning multiple sensors) to multiple UEs.