3GPP TSG-SA WG1 Meeting #76bis	S1-170075

Spokane, USA, 16-20 JAN 2017	Revision of S1-17XXXX

Title:	eV2X Requirements for SMARTER

Agenda Item:	5.1

Source:	LG Electronics

Contact:	SungDuck Chun (sungduck.chun@lge.com)

- Discussion

Current TS for SMARTER has place holders for eV2X. This document proposes to add latest eV2X requirements into TS 22.261.

The progress made by the end of SA1#76 on TR 22.886 is 95%, which is almost complete. The structure of TR 22.886 contains the following section number regarding consolidated PRs as in the 1st column of Table 1 below.

Table 1:

| TR 22.886 Section Number | Topic | TS 22.261 Section Number |  |
| --- | --- | --- | --- |
| 7.1 | General description | 6.18.1 |  |
| 7.2 | Consolidated requirements | 6.18.2 |  |
| 7.2.1 | General requirements | 6.18.2.1 | Addition of sub-clause |
| 7.2.2 | Requirements for platooning | 6.18.2.2 | Addition of sub-clause |
| 7.2.3 | Requirements for advanced driving | 6.18.2.3 | Addition of sub-clause |
| 7.2.4 | Requirements for extended sensor | 6.18.2.4 | Addition of sub-clause |
| 7.2.5 | Requirements for remote driving | 6.18.2.5 | Addition of sub-clause |

Proposal 1:

To use requirements in the respective sections of TR 22.886 as input for the sections of TS 22.261 as proposed in Table 1.

The eV2X TR also contains tabular form of CPRs (e.g., KPI values).

Table 2:

| TR 22.886 Table Number | Topic | TS 22.261 Section Number |  |
| --- | --- | --- | --- |
| 7.2.2-1 | Performance requirements for platooning | 7.3 eV2X performance requirements |  |
| 7.2.3-1 | Performance requirements for advanced driving | 7.3 eV2X performance requirements |  |
| 7.2.4-1 | Performance requirements for extended sensor | 7.3 eV2X performance requirements |  |
| 7.2.5-1 | Performance requirements for remote driving | 7.3 eV2X performance requirements |  |

Proposal 2:

To put the Tables listed in Table 2 below into section 7.3 of TS 22.261.

Based on the discussion during eV2X session, further update may be needed.

- Proposal

It is proposed to agree on the text proposal below.

------------------------------------------------------------------------------------------------

------------------------------ START of first text Proposal-------------------------------

6.18	eV2X aspects

6.18.1	Description

Different V2X scenarios require the transport of V2X messages with different performance requirements for the 3GPP system.

Vehicles Platoonning enables the vehicles to dynamically form a platoon travelling together. All the UEs in the platoon obtain information from the leading vehicle to manage this platoon. These information allow the vehicles to drive closer (short time or distance inter-vehicle gap) than normal in a coordinated manner, going to the same direction and travelling together. These are expected to be a set of sophisticated application.

Extended Sensors enables the exchange of raw or processed data gathered through local sensors or live video images among  vehicles, road site units, devices’ of pedestrian and V2X application servers. The vehicles can increase the perception of their environemnt beyond of what their own sensors can detect and have have a more broad and holistic view of the local situation. High data rate is one of the key characteristics.

Advanced driving enables semi-automated or full-automated driving. Longer inter-vehicle distance is assumed. Each vehicle and/or RSU shares its own perception data obtained from its local sensors with vehicles in proximity and that allows vehicles to synchronize and coordinate their trajectories or manoeuvres. Each vehicle shares its driving intention with vehicles in proximity, too. The benefits of this use case group are safer traveling, collision avoidance, and improved traffic efficiency.

Remote Driving enables a remote driver or a V2X application to operate a remote vehicle for those passengers who cannot drive by themselves or remote vehicles located in dangerous environments. For a case where variation is limited and routes are predictable, such as public transportation, driving based on cloud computing can be used. Also, access to cloud-based back-end service platform can be considered for this use case group. High reliability and short low latency are the main requirements.

### 6.18.2	Requirements

6.18.2.1	General Requirements

Some requirements in this section are applicable to all V2X scenarios.

The 3GPP system shall be able to control the communication range for a message based on the characteristic of the messages transmitted by a UE supporting V2X application.

The 3GPP system shall be able to optimize the communication between UEs supporting V2X application belonging to the same group and in proximity.

The 3GPP system shall be able to support the message transfer for group management operations as requested by the application layer.

The 3GPP system shall be able to support message transfer among a group of UEs supporting V2X application.

The 3GPP system shall be able to support message transfer between two UEs belonging to the same group of UEs supporting V2X application.

The 3GPP system shall be able to support confidentiality and integrity of message transfer among a group of UEs supporting V2X application.

The 3GPP system shall support relative lateral position accuracy of 0.1 m between UEs supporting V2X application.

The 3GPP system shall support mechanisms to ensure sufficient reliability metrics are reached.

The 3GPP system shall support high connection density for congested traffic.

NOTE:	Example estimate is for worst case US Freeway scenario that does not include arterial roads (i.e. onramps): 5 lanes in each direction or 10 lanes total per highway, for up to 3 highways intersecting = 15,840 cars per mile.

The 3GPP system shall support efficient coordination of radio resources used for transport of V2X communications to maximize the utilisation of the available spectrum and to ensure the required reliability.

The 3GPP system shall be able to control the UL and DL reliability of transport of V2X communications, depending on the requirement of V2X application

Impact to E-UTRA(N) by UE supporting only 5G RAT(s) based V2X communication shall be minimized.

Impact to 5G RA(N) by UE supporting only E-UTRA based V2X communication shall be minimized.

The 3GPP system shall be able to support message transfer between UEs or between a UE and a UE-type RSU, regardless of whether or not they are subscribers of the same PLMN supporting V2X communications. In case they are subscribers to different PLMNs, there shall be no service degradation of the message transfer.

The 3GPP system shall be capable of supporting high reliability without requiring application-layer message retransmissions.

The 3GPP system shall enable discovery and communication between UEs supporting the same V2X application.

The 3GPP system shall be able to support for the operator to select which 3GPP RAT to use for a V2X application.

The 3GPP system shall enable a UE supporting a V2X application to obtain network access via another UE supporting V2X application.

The 3GPP system shall enable a UE supporting a V2X application to discover another UE supporting V2X application that can offer access to the network.

The 3GPP system shall enable a UE supporting a V2X application to switch the network access from direct 3GPP connection to an indirect 3GPP connection via another UE supporting V2X application that is connected via 3GPP access to the 3GPP core network, and vice versa.

The 3GPP system should provide integrity and confidentiality protection (end to end) for the network access traffic of a V2X UE via another such UE.

The 3GPP system shall allow UEs supporting V2X application to use 5G RAT for direct communication when the UEs are not being serviced by a 5G cell.

An RSU shall be able to communicate with up to 200 UEs supporting a V2X application.

The 3GPP system shall support less than 5 ms communication latency for transport of V2V messages between two UEs supporting V2V applications, that are part of a group of UEs supporting V2Vapplications.

NOTE:	The determination of group membership may be done at the upper layers and/or lower layers (application and/or Layer 2).

6.18.2.2	Requirements for platooning

The 3GPP system shall be able to support up to 5 UEs for a group of UEs supporting V2X application.

For Vehicle Platooning, the 3GPP system shall be able to support reliable V2V communications between a specific UE supporting V2X applications and up to 19 other UEs supporting V2X applications.

The 3GPP system shall support relative longitudinal position accuracy of less than 0.5 m for UEs supporting V2X application for platooning in proximity.

6.18.2.3	Requirements for advanced driving

Editor's Note: to be updated.

6.18.2.4	Requirements for extended sensor

Editor's Note: to be updated.

6.18.2.5	Requirements for remote driving

The 3GPP system shall support user experienced data rate up to 1 Mbps at DL and 20 Mbps at UL for UE supporting V2X application between V2X application server and UE for an absolute speed of up to 250 km/h.

The 3GPP system shall support ultra-high UL and DL reliability [99.999 or higher] % for UE supporting safety-related V2X application.

The 3GPP system shall support end-to-end latency 5 ms between V2X application server and UE supporting safety-related V2X application for an absolute speed of up to 250 km/h.

------------------------------------------------------------------------------------------------

------------------------------ START of second text Proposal ----------------------------

7.3	eV2X performance requirements

Table 7.3-1 Performance requirements for platooning

| Communication scenario |  | Payload (Bytes) | Tx rate (Message/ Sec) | Max latency (ms) | Reliability (%) | Data rate (Mbps) | Communication range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Among a group of UEs (or two UEs) supporting V2X application |  | 50-1200 (NOTE 1) | 30 | 10 (NOTE 2) |  |  |  |
|  |  | 300-400 | 30 | 25 | 90 |  |  |
| Between UE supporting V2X application and RSU via another UE supporting V2X application |  | [50-1200] | 2 | 500 |  |  |  |
| Between UEs supporting V2X application | Driver control | 300-400 (NOTE 3) |  | 25 | 90 |  |  |
|  | Fully automated driving | 1200 |  | 10 | 99.99 |  | 80 |
| Between UEs supporting V2X application | Driver control/ Limited automated driving | [6500] | 50 | [20] |  |  | [10] sec * (max. relative speed) [m/s] |
|  | Fully automated driving |  |  | [20] |  | [65] | [5] sec * (max. relative speed) [m/s] |
| Between UE supporting V2X application and RSU | Driver control/ Limited automated driving | [6000] | 50 | [20] |  |  | [10] sec * (max. relative speed) [m/s] |
|  | Fully automated driving |  |  | [20] |  | [50] | [5] sec * (max. relative speed) [m/s] |
| NOTE 1: This value does not including security related messages component. NOTE 2: This value is end-to-end latency. NOTE 3: This value is applicable for both triggered and periodic transmission of data packets |  |  |  |  |  |  |  |

Table 7.3-2 Performance requirements for advanced driving

| Communication scenario |  | Payload (Bytes) | Tx rate (Message/Sec) | Max latency (ms) | Reliability (%) | Data rate (Mbps) | Commu-nication range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Between UEs supporting V2X applications Fully automated driving |  | [2000] |  | [10] | [99.99] | [10] |  |
| Between UEs supporting V2X application | Driver control/ Limited automated driving | [6500] | 10 | [100] |  |  | [10] sec * (maximum relative speed) [m/s] |
|  | Fully automated driving |  |  | [100] |  | [53] | [5] sec * (max. relative speed) [m/s] |
| Between the UE supporting V2X application and the RSU | Driver control/ Limited automated driving | [6000] | 10 | [100] |  |  | [10] sec * (max. relative speed) [m/s] |
|  | Fully automated driving |  |  | [100] |  | [50] | [5] sec * (max. relative speed) [m/s] |
| Between UEs supporting V2X application Fully automated driving |  |  |  | [3] | [99.999] | [30] | [500] |
| Between RSU and UE supporting V2X application |  | 450 | 50 |  |  | DL: [0.5] UL: [50] |  |

Table 7.3-3 Performance requirements for extended sensors

| Communication scenario |  | Payload (Bytes) | Max latency (ms) | Reliability (%) | Data rate (Mbps) | Communication range (meters) |
| --- | --- | --- | --- | --- | --- | --- |
| Between UEs supporting V2X application Fully automated driving |  |  | 10 | 95 | Peak data rate [25] |  |
| Between UEs supporting V2X application | Driver control | [1600] | 100 | 99 |  | 1000 |
|  | Fully automated driving |  | 3 | 99.999 |  | 200 |
|  |  |  | 10 | 99.99 |  | 500 |
|  |  |  | 50 | 99 |  | 1000 |
|  |  |  |  |  | 1000 | 50 |
| Between UEs supporting V2X application | Driver control/ Automated driving |  | [50] | 90 | [10] | [100] |
|  | Fully automated driving |  | [10] | 99.99 | [700] | [500] |

Table 7.3-4 Performance requirements for remote driving

| Communication scenario | Payload (Bytes) | Max latency (ms) | Reliabi-lity (%) | Data rate (Mbps) | Communication range (meters) |
| --- | --- | --- | --- | --- | --- |
| Between a UE supporting V2X application & V2X Application Server. Driver Control |  | [20] | [99.999] | UL: 25 DL: 1 |  |