3GPP TSG-SA WG1 Meeting #92e	S1-204099

Online, 09 November – 18 November 2020

Title:	 FS_Ranging Security and authorization issues

Agenda Item:	7.10.1

Source:	Xiaomi

Contact:	Xiaowei Jiang (jiangxiaowei@xiaomi.com)

1. Background

In last SA1 meeting, some companies have concerns over security & authorization requirement. In this contribution, we address the two issues.

2. Discussion

2.1. Secure ranging

During last meeting email discussion, some companies expressed their concerns over the meaning of secure ranging in relation to the following requirements for smart vehicle key use case and hands free access use case:

The 5G system shall be able to support secure ranging.

Then, the following definition of secure ranging was added to the Definition section of TR22.855:

Secure Ranging: refer to transfer Ranging related data securely during Ranging operation.

Some company thinks it is still unclear. As a result, the secure ranging requirement were removed from the two use cases.

However, security is an very important part of ranging to ensure that:

• Protect privacy of UE and its user by ensuring that no identifiable information can be tracked.

• Protect a UE from being traced. This is to avoid tracing a specific UE by e.g. detecting ranging signal.

• Protect confidentiality, integrity and authenticity of ranging related information (e.g. timestamps, angles).

• Ensure that replay attacks can be detected

• Limit risk of side channel attacks

Thus, we suggest:

Proposal 1: Add the following requirements to smart vehicle key use case:

The 5G system shall be able to protect privacy of a UE and its user, ensuring that no identifiable information can be tracked by undesired entities during Ranging.

Proposal 2: Remove the definition of secure ranging in the definition section of TR22.855.

2.2. Authorization

During last meeting email discussion, the following requirement for smart home TV control use case was set to FFS:

The  5G system shall be able to authorize Ranging for each individual UE.

Editor’s note: this requirement is FFS.

In the overview section of TR22.855, however, the following sentence was added as suggested by operators:

If licensed band is used for ranging, it shall be fully under operator control.

To align these two parts, we suggest to remove the FFS:

Proposal 3: Remove the FFS part::

The  5G system shall be able to authorize Ranging for each individual UE.

3. Proposals

Proposal 1: Add the following requirements to smart vehicle key use case:

The 5G system shall be able to protect privacy of a UE and its user, ensuring that no identifiable information can be tracked by undesired entities during Ranging.

Proposal 2: Remove the definition of secure ranging in the definition section of TR22.855.

Proposal 3: Remove the FFS part:

The  5G system shall be able to authorize Ranging for each individual UE.

**************** First Change ******************

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Key performance indicators and key attributes for Ranging use cases are defined as follows:

…

************* End of First Change ***************

**************** Second Change ******************

### 5.3.6	Potential New Requirements needed to support the use case

The 5G system shall be able to protect privacy of a UE and its user, ensuring that no identifiable information can be tracked by undesired entities during Ranging.

************* End of Second Change ***************

**************** Third Change ******************

### 5.2.6	Potential New Requirements needed to support the use case

The 5G system shall be able toauthorize Ranging for each individual UE.

************* End of Third Change ***************