3GPP TSG-SA WG1 Meeting #92e	S1-204100

Online, 09 November – 18 November 2020

Title:	 FS_Ranging Consolidated Requirements

Agenda Item:	7.10.1

Source:	Xiaomi

Contact:	Xiaowei Jiang (jiangxiaowei@xiaomi.com)

Abstract: consolidated requirements section of FS_Ranging TR22.855  and will be updated based on the email discussion results in this meeting.

Discussion

- Functional requirements

| PR number | Description | Action |
| --- | --- | --- |
| [PR 5.1.6-1] | The 5G system shall be able to support Ranging enabled UEs to discover other Ranging enabled UEs. | Create a new requirement: [PR 8.1-1] The 5G system shall be able to support for a UE to discover other UEs supporting Ranging. |
| [PR 5.2.6-1] | The 5G system shall be able to authorize Ranging for each individual UE. | Create a new requirement: [PR 8.1-2] The 5G system shall be able to authorize Ranging for each individual UE. |
| [PR 5.3.6-1] | The 5G system shall be able to protect privacy of a UE and its user, ensuring that no identifiable information can be tracked by undesired entities during Ranging. | Create a new requirement: [PR 8.1-3] The 5G system shall be able to protect privacy of a UE and its user, ensuring that no identifiable information can be tracked by undesired entities during Ranging. |
| [PR 5.5.6-1] | The 5G system shall support identification of other ranging capable UE/UEs with which a ranging capable UE performs Ranging. | This is a duplicate of [PR 8.1-1]. |
| [PR 5.6.6-1] | The 5G system shall be able to enable or disable the Ranging service. | Create a new requirement: [PR 8.1-4] The 5G system shall be able to enable or disable the Ranging service. |
| [PR 5.7.6-1] | The 5G system shall be able to support for a UE to discover other UEs supporting Ranging. | Included by [PR 8.1-1] |
| [PR 5.8.6-1] | The 5G system shall be able to support for a UE to discover other UEs supporting Ranging. | Included by [PR 8.1-1] |
| [PR 5.9.6-1] | The 5G system shall be able to support for a UE to discover other UEs supporting Ranging. | Included by [PR 8.1-1] |
| [PR 5.11.6-1] | The 5G system shall support mutual ranging, i.e. the two UE’s involved shall be able to range each other. | Create a new requirement: [PR 8.1-5] The 5G system shall support mutual ranging, i.e. the two UE’s involved shall be able to range each other. |
| [PR 5.12.6-1] | The 5G system shall support energy efficient UE ranging services and operation. | Create a new requirement: [PR 8.1-6] The 5G system shall support energy efficient UE ranging services and operation. |
| [PR 5.13.6-1] | The 5G system shall be able to support change of ranging configurations (e.g., ranging interval, or other KPIs) for one ranging service during the ranging service operation and continuous operation with new ranging configurations. | Create a new requirement: [PR 8.1-7]The 5G system shall be able to support change of ranging configurations (e.g., ranging interval, or other KPIs) for one ranging service during the ranging service operation and continuous operation with new ranging configurations. |
| [PR 5.13.6-2] | The 5G system shall be able to support suitable APIs to enable the ranging based service provider to manage ranging operation and configurations. | Create a new requirement: [PR 8.1-8]The 5G system shall be able to support suitable APIs to enable the ranging based service provider to manage ranging operation and configurations. |
| [PR 5.2.6-2] | The 5G system shall be able to ensure that the use of ranging in licensed spectrum is under full continuous control of the network. | Create a new requirement: [PR 8.1-9]The 5G system shall be able to ensure that the use of ranging in licensed spectrum is under full continuous control of the network. |

- Performance requirements

Except for museum tour use case (as there is only one KPI value), all other use cases are merged into one KPI table.

**************** First Change ******************

6	Consolidated requirements

6.1	Functional requirements

[PR 8.1-1] The 5G system shall be able to support for a UE to discover other UEs supporting Ranging.

[PR 8.1-2] The 5G system shall be able to authorize Ranging for each individual UE.

[PR 8.1-3] The 5G system shall be able to protect privacy of a UE and its user, ensuring that no identifiable information can be tracked by undesired entities during Ranging.

[PR 8.1-4] The 5G system shall be able to enable or disable the Ranging service.

[PR 8.1-5] The 5G system shall support mutual ranging, i.e. the two UE’s involved shall be able to range each other.

[PR 8.1-6] The 5G system shall support energy efficient UE ranging services and operation.

[PR 8.1-7] The 5G system shall be able to support change of ranging configurations (e.g., ranging interval, or other KPIs) for one ranging service during the ranging service operation and continuous operation with new ranging configurations.

[PR 8.1-8] The 5G system shall be able to support suitable APIs to enable the ranging based service provider to manage ranging operation and configurations.

[PR 8.1-9] The 5G system shall be able to ensure that the use of ranging in licensed spectrum is under full continuous control of the network.

6.2	Performance requirements

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Distance based smart device control | 10cm | - | 99 % | 50ms | 20m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | 20 | - |
| Distance based smart home TV control | 10cm up to 3 meter separation | ±2° horizontal direction accuracy at 0.1 to 3 meter separation and AoA coverage of (-60°) to (+60°); ±2° Elevation direction accuracy at 0.1 to 3 meter separation and AoA coverage of (-45°) to (+45°) | 99 % | 10ms | 10m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | - | - |
| Smart Vehicle Key | 10 cm | - | 99 % | 50ms | 30m | IC/PC/OOC | - | Static/ Moving (<2m/s) | - | - | 50UEs/ (104m2) |
| Finding Items in a supermarket | 50 cm | 5 degree | 95 % | 500ms | 100m | IC/PC/OOC | - | Static/ Moving (<1m/s) | - | - | 100 UEs/ (3.14*104m2) |
| Touchless Self-checkout Machine Control | 10cm | - | 99% | 50ms | 1m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | - | - | = |
| Hands Free Access | 10cm | - | [99 %] | 100ms | 10 m | IC/PC/OOC | - | Static/ Moving (1 m/s) | - | - | 20 UEs/3.14*100m2 |
| Smart Transportation Metro/Bus Validation | 10cm | - | 99 % | 50ms | 2m | IC/PC/OOC | LOS | Static/ Moving (3km/h) | 50ms | 20 | 100 in the area of 8 m2 |
| distance based intelligent perception for public safety | 50cm | - | 99 % | - | 20m | IC/PC/OOC | LOS | Static/ Moving (<20km/h) | - | 100 | - |
| Picture and video sharing based on ranging results | 10cm | 2° | 99 % | 50ms | 10m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | - | - |
| Ranging of UE’s in front of vending machine | 20cm | 10° | - | 150ms | 5m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | - | 10 |
| Search and rescue | 20m | 5° | 99 % | 1s | 100m-1km | IC/PC/OOC | LOS | Static/ Moving (up to 10m/s) | 5s | - | - |

************* End of First Change ***************