3GPP TSG-SA WG1 Meeting #92e 	S1-204157r1

Electronic Meeting, 9 November - 18 November 2020

Title:	Requirement of touchless self-checkout machine control

Agenda Item:	7.10.1

Source:	OPPO

Contact:	Wenqiang Tian (tianwenqiang@oppo.com)

Abstract: To clarify the number described in the potential requirement of Touchless Self-checkout Machine Control Use Cases of FS_Ranging

- Discussion

As described in this use cases, traditional public touch mode for a self-checkout machine would be risky, especially during the outbreak of a pandemic. For example, if an infected user leaves virus that may cause disease on a self-checkout machine through touching, then there would be a great health risk for subsequent users who may touch the public control device. To deal with this problem, self-devices such as mobile phones could be utilized to obtain the control authority from a public device, e.g. a public self-checkout machine, and perform interactive operations to indirectly control the public device.

In this case, Dorina’s smartphone needs to obtain an authority when it is close to a public self-checkout machine. So timely and reliable ranging would be required to trigger the authority procedure and to improve user experience. For example, when a device finds there is a self-checkout machine nearby (already within an authority area of a public self-checkout machine, e.g. the distance between the UE and the machine is less than 1m with ±10cm accuracy and availability >99%), the UE can trigger a corresponding procedure to obtain the authority to get control interface and indirectly control the machine.

Besides, Dorina also wants to enjoy a smooth service without obvious interruption. In the process of self-service checkout and payment, 1s seems to be a reasonable delay from the time Dorina enter the authorization area to the interactive interface displayed on her smartphone. Because the range based authorization process is only a small part of the whole service, So the ranging service should be carried out with a delay of 50ms, and the rest of the time can be used to complete the transmission and display of the shopping info.

Given that people occasionally go to supermarkets, this type of Ranging service need to be turned on and off on demand. For example, when Dorina enters a supermarket, a touchless self-checkout service can be enabled. When the customer leaves the supermarket, the service is disabled. In this way, the unnecessary ranging service can be avoided and the power consumption of UE can be reduced.

- Proposal

With a clear clarification, the square brackets can be removed in clause 5.6.6 of TR 22.855.

**************** First Change ******************

5.x.6	Potential New Requirements needed to support the use case

The 5G system shall be able to enable or disable the Ranging service.

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Coverage, environment of use, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation support in area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  |  | Effective ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation support in area |
| distance based intelligent perception for public safety | 10 cm | - | 99% | 50 ms |  | 1m | IC/PC/OOC | LOS | - | - | - | = |

************* End of First Change ***************