3GPP TSG-SA WG1 Meeting #92e 	S1-204169r6

Electronic Meeting, 10 November – 19 November 2020	(revision of S1-20xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.104 | CR | 0060 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding energy efficiency related requirements for industrial IoT positioning scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Dummy |  |  |  |  |  | Date: |  |  | 2020-11-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In industrial environment, high precision positioning service may be applied to power-constrained IoT devices. In TS 22.104, two use cases “plant asset management” and “Flexible, modular assembly area” already specified the possible scenarios and pointed out “high energy-efficiency is required for these battery-driven sensors”. However for both use cases, so far there has not proposed any requirements related to energy efficiency aspects in TS 22.104. We aims Energy efficient high precision position is essential and critical for industrial IoT device. On one hand, the IoT device would consume more power to achieve the required high positioning accuracy, in particular when the IoT device may move with a high speed from time to time, for example, a typical use case be some material, carried by an AGV moves from one warehouse to another. When the UE moves, it is necessary to perform very frequent positioning update while when it is stationary, the postioning update could be relaxed. In addition, it is confirmed by various utility sector stakeholders that in areas of their operation environment labelled as Danger Zones (i.e. nuclear power plant, HV/MV/LV substations, etc.), it is necessary to perform very frequent positioning update even when the UE does not have high mobility.. On the other hand, considering recharging massive IoT devices would be a great burden for the industry operator, it shall be possible for the IoT device to be able to work for a quite long time period with an embedded battery. This factor should be considered, so that the energy efficiency for high precision positioning service has a clear market demand. Hence, energy efficiency related requirements for industrial IoT positioning scenarios shall be specified accordingly in SA1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add energy efficiency related requirements for industrial IoT positioning scenarios |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The energy efficiency apects for industrial IoT positioning scenarios are not addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the Change---

## 5.7	Positioning performance requirements

High accuracy positioning is becoming essential for Factories of the Future. The reason for this is that tracking of mobile devices as well as mobile assets is becoming increasingly important in improving processes and increasing flexibility in industrial environments.

The 5G system shall provide positioning information for a UE that is out of coverage of the network, with accuracy of < [1 m] relative to other UEs that are in proximity and in coverage of the network.

Table 5.7-1 below lists typical scenarios and the corresponding high positioning requirements for horizontal and vertical accuracy, availability, heading, latency, and UE speed.

NOTE:	The column on "Corresponding Positioning Service Level in TS 22.261" maps the scenarios listed in Table 5.7-1 to the service levels defined in TS 22.261 [2].

Table 5.7-1: Positioning performance requirements

| Scenario | Horizontal accuracy | Vertical accuracy | Availability | Heading | Latency for position estimation of UE | UE Speed | Corresponding Positioning Service Level in TS 22.261 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mobile control panels with safety functions (non-danger zones) | < 5 m | < 3 m | 90 % | n/a | < 5 s | n/a | Service Level 2 |
| Process automation – plant asset management | < 1 m | < 3 m | 90 % | n/a | < 2 s | < 30 km/h | Service Level 3 |
| Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) | < 1 m (relative positioning) | n/a | 99 % | n/a | 1 s | < 30 km/h | Service Level 3 |
| Augmented reality in smart factories | < 1 m | < 3 m | 99 % | < 0.17 rad | < 15 ms | < 10 km/h | Service Level 4 |
| Mobile control panels with safety functions in smart factories (within factory danger zones) | < 1 m | < 3 m | 99.9 % | < 0.54 rad | < 1 s | n/a | Service Level 4 |
| Flexible, modular assembly area in smart factories (for autonomous vehicles, only for monitoring purposes) | < 50 cm | < 3 m | 99 % | n/a | 1 s | < 30 km/h | Service Level 5 |
| Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of indoor autonomous driving systems)) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | < 3 m | 99.9 % | n/a | 10 ms | < 30 km/h | Service Level 6 |
| Inbound logistics for manufacturing (for storage of goods) | < 20 cm | < 20 cm | 99 % | n/a | < 1 s | < 30 km/h | Service Level 7 |

## 5.7.1 Energy efficiency requirements for industrial IoT positioning

Table 5.7.1-1 below lists typical industrial IoT positioning scenarios and the corresponding energy effciency requirements, including positioning accuracy, positioning interval and bettery life time.

Table 5.7.1-1: Energy efficiency requirements for industrial IoT positioning

| Scenario | Corresponding Positioning Service Level in TS 22.261 (Note 1) | Positioning interval (Note 2) | Battery life time (Note 4) |
| --- | --- | --- | --- |
| Process automation – plant asset management | Service Level 3 | > 5 seconds (on average) | >1 year |
| Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) | Service Level 3 | > 5 seconds (on average) | >1 year |
| Note 1: The positioning service level is based on table 5.7-1 Note 2: This value applies to at least two types of scenarios. First, not all the use cases of Service level 3 applied to the requirement, but the value of >5 seconds applies for UEs which may moving with 30km/h. Another compelling additional scenario applies for very frequent positioning fix, even when the UE does not have high mobility. Example IIoT operation environment could be areas labeled as Danger Zones (ie. nuclear power plant, HV/MV/LV substations, etc.) of a power energy company and electrical utilities. Note 3: It is confirmed by various utility sector stakeholders that, it is necessary to perform. Note 4: This requirement assume the devices are equipped with typical button battery with less than 1650 mWh of capacity dedicated for positioning, which is for sheer energy consumption pertinent to positioning fix. |  |  |  |

## ---End of the Change---