3GPP TSG-SA WG1 Meeting #92e	S1-204172r1

Electronic Meeting, 10 - 19 November 2020	(revision of S1-20xxxx)

Title:	Clarification on efficient routing for UE-to-UE communications via residential gateway

Agenda Item:	7.11

Source:	Huawei, Interdigital

Contact:	Yuan Wang (wangyuan5@huawei.com)

Abstract: This document proposes clarifications on the use case of efficient routing for UE-to-UE communications via residential gateway.

**************** First Changes ****************

5.5	Clarification on efficient routing for communications between two UEs via residential gateway

5.5.1	Description

This use case assumes that multiple indoor small base stations were already deployed in individual rooms behind a residential gateway, in order to provide better cellular coverage at home. This use case is to enable efficient routing for the communications between two UEs via the residential gateway.

5.5.2	Pre-Conditions

The following pre-conditions and assumptions apply to this use case:

- Multiple indoor small base stations were deployed in individual rooms inside a residential home.

- The indoor small base station provides cellular access to UEs

- The indoor small base station is connected to a Residential Gateway via wireline or wireless link

- The residential gateway is connected to the same 5G system (and has a subscription to the same 5G system) as the indoor small base station belongs to.

In particular, one indoor small base station is deployed at the attic and another one is deployed at the basement. The connections between the indoor small base stations and the residential gateway could have multiple options, e.g., fiber, WiFi, or 3GPP licensed spectrum.

A security sensor is deployed at the basement which is capable of detecting smoke or fire. The sensor having a SIM is a 3GPP-capable UE which is connecting to the indoor small base station in the basement.

5.5.3	Service Flows

Alice is working alone at home, staying in her office in the attic. Her smartphone is connecting to the indoor base station deployed in the attic.

Suddenly there is a fire in the basement.

The security sensor detects the fire and smoke, and then immediately sends an alarm to Alice’s smartphone.

The alarm message first reaches the residential gateway via the indoor small base station in the basement, and then routed by the residential gateway to Alice smartphone via the indoor small base station in the attic.

5.5.4	Post-Conditions

Alice reads the alarm message from her smartphone. After double-check the situations in the basement, Alice makes the call to the fire policy.

5.5.5	Existing features partly or fully covering the use case functionality

The 5G system shall support on-demand establishment of UE to UE, multicast, and broadcast private communication between members UEs of the same 5G LAN-VN. Multiple types of data communication shall be supported, at least IP and Ethernet.

3GPP TS 22.261 specifies the requirements for routing efficiency, e.g., private communication for 5G LAN-type service.

5.5.6	Potential New Requirements needed to support the use case

[PR. 001] The 5G system shall be able to provide E2E QoS control for the communication path between a UE and a residential gateway via an indoor small base station.

[PR. 002] The 5G system shall be able to support efficient routing for the data traffic between two UEs  via a residential gateway (i.e., without going through the 5GC), where both UEs are connected to the residential gateway via a same or different indoor small base stations.

[PR. 003] The 5G system shall ensure the use of a residential gateway does not compromise the security of any PLMN or broadband access network.

[PR.004] The 5G system shall ensure the use of a residential gateway does not compromise the security of the UE.

[PR. 005] The 5G system shall enable the network operator associated with a residential gateway to control the security policy of the residential gateway.

[PR 006] The 5G system shall ensure a residential gateway does not compromise user privacy for UEs that are using the residential gateway, including communication confidentiality, location privacy and identity protection.

Editor’s Note: the name of residential gateway need further study.

Editor’s Note: Definition of indoor small base station needs further study.

************* End of Changes ***************