3GPP TSG-SA WG1 Meeting #93e	  S1-210176r2

Online, 22 February – 04 March 2021

Title:		Use Case of power distribution grid power control service

Agenda Item:	7.9.1

Source:	Ericsson

Contact:	Robert Farac (robert.farac@ericsson.com)

Abstract: This contribution proposes a new use case of power distribution grid power control service.

Acknowledgment

This contribution proposes a new use case of power distribution grid power control service. This contribution has been prepared in collaboration with RWTH University of Aachen Prof. Monti, Antonello. The use case has been implemented and tested in several European power distribution grids in EU project SOGNO (https://www.sogno-energy.eu/).

Discussion

-

Proposal

It is proposed to agree to add the following change to TR 22.867.

Begin First Change

2		References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	                       3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	SunWeiqing etc.  Generalized Energy Storage Control Strategies on User Side in Power Ancillary Service Market.  Automation of Electric Power System Vol.44 No.2 Jan.25, 2020

[3]	China Southern Power Grid, China Mobile, HuaWei: "White Paper: 5G enables smart grid applications".http://www.sgcio.com/technology/anli/2018/0716/86656.html

[4]  	GB/T 1936-2018 Guidelines of cyber security protection for electric power system supervision and control http://www.howtoabc.com/book8/14078-0.html, https://www.secrss.com/articles/24761

[5]	IEEE C37.2-2008 IEEE Standard Electrical Power System Device Function Numbers, Acronyms, and Contact Designations.

[6]	IEC TR 61850-90-1:2010, Communication Networks and Systems for Power Utility automation – Part 90-1: Use of IEC61850 for the communication between substations.

[7]	IEEE 1588-2019 – IEEE Standard for a precision clock synchronization protocol for networked measurement and control systems.

[8]	IEEE Guide for Application of Digital Line Current Differential Relays Using Digital Communication.

[9]	61850-9-3-2016 – IEC/IEEE International Standard - Communication Networks and Systems for Power Utility automation – Part 90-3: Precision time protocol profile for power utility

[10]                     3GPP TR 22.804: "Study on Communication for Automation in Vertical domains (CAV)".

[11]	Sendin, A., et. al., “Telecommunication Networks for the Smart Grid,” Artech House, 2016.

[12]	Goel, S., S. F. Bush, and D. Bakken, (eds.), IEEE Vision for Smart Grid Communications: 2030 and Beyond, New York: IEEE, 2013.

[13]	US Department of Energy, “Communications Requirements of Smart Grid Technologies, 2010”, accessed 14.08.20, http://energy.gov/sites/prod/files/gcprod/documents/Smart_Grid_Communications_Requirements_Report_10-05-2010.pdf

[14]	IT Process Wiki – The ITIL Wiki:. Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[15]	Incident Management: https://wiki.en.it-processmaps.com/index.php/Incident_Management Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[16]	Change Management: https://wiki.en.it-processmaps.com/index.php/Change_Management Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[17]	Service Asset and Configuration Management: https://wiki.en.it-processmaps.com/index.php/Service_Asset_and_Configuration_Management Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[18]	China Innovation Alliance for Power Distribution Internet of Things, “Technical Specifications of Smart Distribution Transformer Terminal”

[19]	Open PLC European Research Alliance (OPERA). https://cordis.europa.eu/project/id/026920

[20]	“Applications of ITU-T G.9960, ITU-T G.9961 transceivers for Smart Grid applications: Advanced metering infrastructure, energy management in the home and electric vehicles”, 06/2020. https://www.itu.int/dms_pub/itu-t/opb/tut/T-TUT-HOME-2010-PDF-E.pdf

[21]	M. Amin and A. Giacomoni, "Toward more secure, stronger and smarter electric power grids," IEEE PES’11, 2011.

[22]	Eric D. Knapp, Raj Samani, "Smart Grid Network Architecture", "Applied Cyber Security and the Smart Grid", 2013

[23]	5G DNA White Paper: "5GDN@Smart Grid White Paper: Requirements, Technologies, and Practices" https://www.5gdna.org/

[24]	IEEE Std 1344™-1995 (R2001), IEEE Standard for Synchrophasors for Power Systems

[25]                  	IEEE Std C37.118™-2011, IEEE Standard for Synchrophasors for Power Systems

[26]	IEEE Std 1588-2008, Precision Clock Synchronization Protocol for Networked Measurement and Control Systems

[27]	IEEE Std C37.238-2011, IEEE Standard Profile for Use of IEEE Std 1588™ Precision Time Protocol in Power System Applications.

[28]	IEEE Std C37.224-2013

  - [29]	IEC 61850-90-5:2012, use of IEC 61850 to transmit Synchrophasors information according to IEEE C37.118

[30]	IEEE Std C37.118.1a-2014: amendment 1 with modification of selected performance requirements.

[31]	IEEE Std C37.118.2-2011

[32] 	61850-90-3-2016 – IEC/IEEE International Standard - Communication Networks and Systems for Power Utility automation – Part 90-3: Precision time protocol profile for power utility

[33]	Budka, K, et. al., “Communication Networks for Smart Grids”, Springer Verlag, 2014.

[x1]	SOGNO project, Deliverable 4.6, "Description of the integration and testing of the solution including the 5G based advanced communication", https://www.sogno-energy.eu/global/images/cms/Deliverables/774613_deliverable_D4.6.pdf, June 2020.

End First Change

Begin Second Change

### 5.x	 Use case of power distribution grid power control service

#### 5.x.1	Description

The Power Control (PC) service [x1] optimises the management of the distribution grid power flows (at both medium voltage and low voltage level) for preventing possible contingencies, such as violation of the voltage limits, and overloading of grid components. The service also fosters a more efficient and reliable system operation. This is obtained through the smart control of the active and reactive power injected (or consumed) by converter-based components connected to the grid, such as Distributed Generation (e.g. photovoltaic plants, wind turbines) and energy storage units. For the management of the Distributed Generation based on renewable energy sources, an additional objective is to maximize the use of green energy while respecting the operational constraints of the electric grid, thus minimizing as much as possible the power curtailment of renewable generation. The use of smart power control service in the power grid with high penetration of renewable energy sources is expected to significantly improve the efficient operation of the grid, the power quality, and the grid reliability. This service can be defined as an “active service”, since it actively acts on some of the power system components to modify their operation.

#### 5.x.2	Pre-conditions

The State Estimation service (see Chapter X “Use case of power distribution grid state estimation” above) is deployed and active in the power distribution grid. Output of the State Estimation service can be used by the PC service.

#### 5.x.3	Service Flows

The developed PC service is based on the operating conditions of the grid and processes this information to determine the optimal setting of active and reactive power generation for the distributed generation or, when available, of storage units. Different goals can be pursued to enhance the operation of the distribution grid, such as: the reduction of power losses during the operation with normal conditions; the minimization of voltage unbalance in case of high differences in the status of the three phases of the system; the minimization of possible active power generation curtailments during circumstances with over-voltage issues in the grid. The output of the PC service is a list of set points of active and reactive power (PQ set points) for the converter-based components connected to the grid. Consequently, the service needs bi-directional communication with the field to ensure proper delivery of the generated control commands.

The design of the PC service is coupled with the State Estimation service. According to this idea, the PC service takes as input the results of the State Estimation service, namely the voltage profile at the different buses and the information about the branch power flows and the power consumption (or injection) at the nodes. Together with this information, the service needs as input also the model of the electric grid, expressed in terms of network topology and line characteristics. More specifically, the list of inputs needed to enable the PC service includes the following data.

Static grid data:

- Electric grid topology (information about the nodes and the connection among the different nodes in the grid).

- Electric component parameters (e.g. impedances of the lines, impedances of transformers, etc.).

Real-time data:

- Estimation of the bus voltages, power flows in the branches, power consumption or injection at the nodes (as provided by the State Estimation service).

- Real-time notification of a change in a switch or breaker status.

The output of the PC service is the pair of active and reactive power set points for each converter-based component in the grid. The output of the power control service has to be sent to the converters in the field through communication technologies available in the field for control commands. Results can also be sent to the control room of the Distribution System Operator (DSO) for monitoring and can be stored in the database for possible a posteriori analysis.

Since the PC service relies on the results of the State Estimation service, some of the power system requirements for this service are in common with the State Estimation service. First, measurement devices with real-time communication capabilities are needed to guarantee meaningful monitoring of the electric grid. The accuracy of the estimated results provided by the State Estimation service can also affect the performance of the PC service. Similar to the case of the State Estimation service, a critical challenge for the application of PC at distribution level is the large size of the distribution grids. This calls for the design of distributed approaches where PC services act on small areas, possibly sharing some data with an upper level controller to ensure the coordination of the power control strategy in the whole grid.

Due to the active nature of this service, a bi-directional communication is needed with the field. Data are collected from the grid to enable the State Estimation service, while the output of the PC service is sent to the converter-based components for the control of their power generation (or consumption). This also implies the need to have converters able to communicate remotely, both for sending measurement data (if available) and for receiving the actuation commands. While converters available today do not always guarantee this behaviour, this requirement can be considered as realistic in future scenarios where the availability of communication capabilities is essential to enable the smart grid.

The PC service runs continuously and synchronously with the State Estimation service. In particular, a new iteration of the service is triggered by the collection of a new set of estimation results provide by the State Estimation service. Similar to the case of State Estimation service, there are not strict requirements on the reporting rate for the PC service. However, due to the highly intermittent behaviour of renewable energy sources and the possibility of sudden changes in the operating conditions of the distribution grid, it is recommended to run the PC service with at intervals shorter than one minute and possibly close to a few seconds.

#### 5.x.3.1	Power system requirements of PC service

Needed components in the field: inverters (controllable from remote), sensors and meters.

Information about the electrical grid: topology of the network; grid parameters (line impedance, transformer data, etc.); nominal power of connected loads and generators; location of the meters.

Type of measurements: the State Estimation service results are used in input to the PC service, so the same requirements as to the State Estimation service apply for this point.

Measurement number: the State Estimation service results are used in input to the PC service, so the same requirements as to the State Estimation service apply for this point.

Data volume: limited. In typical operations, one set of setpoints (i.e., active and reactive power, P and Q) is transmitted per minute with volt-reactive volt ampere curve; as an alternative, dynamic voltage control is recommended in case of disturbances. In such situations, the solution will transmit approximately 1 set of setpoints per second. P and Q are floating point numbers. The communication should be scalable, i.e., supporting changing grid topology with an increasing number of actors in the electrical grid.

Measurement accuracy: the State Estimation service results are used in input to the PC service, so the same requirements as to the State Estimation service apply for this point.

Measurement synchronization: the State Estimation service results are used in input to the PC service, so the same requirements as to the State Estimation service apply for this point.

Additional data needed from the field: notification of switching events leading to a change in the network topology.

Real-time communication capability: yes, required

Bi-directional communication: yes required; needed to collect measurement and switch notifications in one direction (uplink) and to apply the control signals to the inverter in the other direction (downlink).

Communication reliability and security: very high. PC service uses only a limited amount of command messages per actor, typically one actor for each switch, transformer, or major power generation unit. Since there will be only a limited set of such actors, the reliability of the data communication becomes particularly important, because losing just a single actor would impact the impact of the PC service. Likewise, it is essential that the data are transmitted in a secure way ensuring full data integrity because of automatic modification of grid edge settings; and in this context, higher security than for monitoring services expected.

#### 5.x.4	Post-conditions

The management of the distribution grid power flows is optimised in terms of preventing possible contingencies and overloading of grid components. The efficient operation of the power grid, the power quality and the grid reliability are significantly improved.

#### 5.x.5	Existing features partly or fully covering the use case functionality

None

#### 5.x.6	Potential New Requirements needed to support the use case

| Characteristic parameter (KPI) |  |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability | Communication service reliability: mean time between failures | Max Allowed End-to-end latency (note 1) (note 2) | Service bit rate: user-experienced data rate |  | Message size [byte] (note 2) | Survival time | UE speed | # of UEs | Service area |  |
| 99.99 % |  | < 30 ms | UL: < 100 Kbit/s |  | < 1000 |  | Stationary | < 300 per km2 | several km2 up to 100,000 km2 |  |
| NOTE 1: Unless otherwise specified, all communication includes 1 wireless link (UE to network node or network node to UE) rather than two wireless links (UE to UE). NOTE 2: It applies to both UL and DL unless stated otherwise. |  |  |  |  |  |  |  |  |  |  |

Editor note: this table should be revisited whether they are existing requirements

End Second Change