3GPP TSG-SA WG1 Meeting #93-e	S1-210268r01

Electronic Meeting, 23 February – 4 March 2021	(revision of S1-21aaaa)

Title:	FS_PIN: Overview update – add diagrams

Agenda Item:	7.12.1 FS_PIN

Source:	vivo Mobile Communications Ltd

Contact:	Adrian Buckley <adrian dot buckley at vivo dot com>

Abstract

This document provides some diagrams for the overview section.

# 4	Overview

[Editor’s Note: This clause provides a high-level overview of the feature. This section will be updated based on the study outcome.]

IoT capabilities have been designed for devices that communicate using the traditional cellular network, including battery constrained devices where it is expected that the battery should last in the order of years. Recently standards have been extended to support more vertical IoT devices for factory based, audio visual, medical, mission critical and vehicular solutions.  In some contexts, e.g. factory based solutions the concept of a private network has been introduced, this has the added benefit for devices that generate very little user plane traffic (e.g. sensors etc.) the traditional cellular operator might not have to dedicate resources to them in the network and the traffic can stay within the local “factory (private network) environment”. There are 2 consumer segments that have similar traffic characteristics where private networks provide an advantage, where communications are predominately within the constraints of a localized IoT network:

i)	Wearable devices;

ii)	Home automation

For the purpose of this discussion these will be called “Personal IoT networks” (PINs). These types of networks are very different to commercial IoT device, they are usually less rugged, most highly battery constrained and lifespan of the battery typically a couple of days or weeks. User plane traffic typically stays with a constrained environment, around the body or in the home i.e. within the PIN.

PINs have been around for a long time using others standards however their take up / adoption rate has been low compared to the general smartphone UE.

a)	An example of a home automation PIN can be seen in Figure 4-1 where there are a number of devices in the home that either communicate directly with the hub or indirectly via a relay to the hub.  A smartphone in the 5G system can receive notifications regarding events (e.g. door opens) from the home automation PIN.

Figure 4-1: Home automation PIN

b)	Wearables can use a multitude of different access technologies, battery life can in some situations be severely constrained just by the physical dimension limitations e.g. glasses frames, earbuds and rings.  Space is also at a premium, capabilities are limited (memory, processing power, even USIM functionality might not be available).  Location requirements on wearable devices, especially those with Uu interfaces can contribute to battery drain. Earbuds / Rings are very small, even an ESIM chip takes up valuable space, battery consumption and adds weight to the device. Figure 4-2 shows two wearable networks, one where all the devices communicate via the smartphone and another where glasses act as a relay for smart earbuds.

Figure 4-2: Wearable PINs

In summary, it is considered beneficial for 3GPP specification to address 5G system support of different use cases for PINs.