3GPP TSG-SA WG1 Meeting # 94bis-e	S1-212018r3

# Electronic Meeting, No Country,  5 July 2021 - 12 July 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.867 | CR | 0008 | rev |  | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Consolidated PR and KPI tables in Section 7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom, CEPRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSEI |  |  |  |  |  | Date: |  |  | 2021-06-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In SA1 94e meeting, the consolidated PR section of TR22.867 has been constructed. After review the section, some clarification and update are needed to align with latest TR contents. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is suggested: 1. “relays” in [CPR.002] is revised to “UEs” because ‘relay’ is a kind of grid terminology and it is better to map it to communication terminology; 2. to remove [x] in Table 7.2-1 and Table 7.2-2 to align with latest approved content; 3. to move the KPI related to accuracy fee control of advanced metering into Table 7.2-2 because the advanced metering data collection belongs to periodic communication while accuracy fee control belongs to aperiodic communication; 4. to add the transfer interval values into line 5.3 of Table 7.2-1 because of the description in section 5.3.1: During the feeder system normal working phase, the heartbeat packet is periodic transmitted with 1s. When a fault occurs, it performs incremental periodic transmission with 2ms, 2ms, 4ms or 8ms time interval; 5. to add one line “5.8 Smart Distribution Transformer Terminal” in Table7.2-1 to align with latest version which add a new KPI table in section 5.8.6 and also add related notes; 6. to add KPI values in Table 7.2-1 e.g. Density, latency range, data rate range, etc. to align with clause 5.12.3.6; 7. to correct the ‘note1’ to ‘note3’ in line 5.19 of Table 7.2-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent with latest TR content and some content in the clause 7 is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1,7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************** Begin Change *******************

# 7	Consolidated potential requirements and KPIs

## 7.1	Consolidated potential requirements

[CPR. 001] The 5G system shall support an end-to-end latency of less than 5ms or 10ms, as requested by the UE initiating the communication.

[CPR. 002] The 5G system shall support communication channel symmetry in terms of latency (latency from UE1 to UE2, and latency from UE2 to UE1) between the two UEs, with the max asymmetry < 2ms.

[CPR 003] Based on MNO policy, the 5G network shall provide suitable means to allow a trusted third party to monitor LAN-VN performance parameters, to configure and receive information for conditions relevant to a specific UE, network and specific configuration aspects of the UE in the VN.

[CPR 004] The 5G system shall provide a means by which an MNO informs 3rd parties of network events (failure of network infrastructure affecting UEs in a particular area, etc.).

[CPR 005] The 5G system shall provide means by which an MNO informs 3rd parties of changes in UE subscription information. The 5G system shall also provide a means for 3rd parties to request this information at any time from the MNO.

NOTE 1: 	Examples of UE subscription information include IP addressand APN network.

Editor’s Note: This requirement is FFS.

[CPR 005a] Based on operator policy, the 5G system shall provide means for the 3rd parties to request changes to UE subscription parameters for access to data networks, e.g. static IP address and APN.

[CPR 006] The 5G system shall provide a means by which an MNO can inform 3rd parties of RAN information with a suitable frequency to aid in identifying performance anomalies and problems.

Editor’s Note: This requirement is FFS.

[CPR 007] The 5G system shall provide a means for an MNO to inform 3rd parties of changes in a UE’s IP configuration (e.g. IP address, 5G LAN-VN membership.)

Editor’s Note: This requirement is FFS.

[CPR 008] The 5G system shall provide a means for an MNO to inform 3rd parties of USIM card related events (USIM opens or closes context) according to their service level agreement.

[CPR 009] The 5G system shall enable support of a mechanism to support authentication and secured communication between the 5G system Core Network and a 3rd party’s application function, in order to provide secure end to end communication service.

[CPR 010] The 5G system shall provide a mechanism for a 3rd party to report to an MNO service degradations, communications loss and sustained connection loss. These reports use a standard form. The specific values, thresholds and conditions upon which alarms occur could include e.g. the measured values for latency, data rate, availability, jitter, etc. for a UE, its location, and the time(s) in which the degradation occurred.

NOTE 2: What the MNO does with such reports is out of scope of 3GPP specifications.

NOTE 3: The above potential requirement expresses the need for reporting by a third party to the 5GS and leaves it to downstream groups (in this case SA5) to work out the implications.

[CPR 011] The 5G System should support the IEC 61850-9-3 [29] profile and IEEE Std C37.238-2017 [24].

[CPR 012] 5G system should support at least one of the two profiles for synchrophasor communications: IEC 61850-90-5:2012 [26], or IEEE Std C37.118.2-2011 [28].

[CPR 013] The 5G system should support the IEEE 802.1Q QoS profile as defined IEC 61850-90-5 [26].

[CPR 014] The 5G system shall support delivery of the same UE originated data in a resource-efficient manner in terms of bandwidth to UEs distributed over a large geographical area.

[CPR 015] The 5G system shall allow a UE to request a communication service to send data to different groups of UEs at the same time.

[CPR 016] The 5G system shall allow a UE to request different QoS for the communication in each of those groups.

[CPR 017]The 5G system shall provide a mechanism for an MNO, based on MNO policy, to automatically report to 3rd parties’ service degradations, communications loss and sustained connection loss in a specific geographic area (e.g. a cell sector, a cell or a group of cells.) These reports use a standard form. The specific values, thresholds and conditions upon which alarms occur could include e.g. the measured values for latency, data rate, availability, jitter, etc. for a UE, its location, and the time(s) in which the degradation occurred.

[CPR 018] Subject to regulatory requirements and operator policy, the 5G system shall support a mechanism by which an MNO can identify the uninterruptable power supply status of the MNO’s infrastructure, specifying which physical regions would be affected in terms of physical topology, as this information will facilitate energy system recovery operations

[CPR 019] Subject to regulatory requirements, the 5G system shall support a mechanism by which a third party can communicate the energy system recovery status in terms of location and time table to the MNO, as this information will facilitate MNO operations to facilitate energy system recovery.

NOTE 4: It is assumed that once aware of the proximity and duration of the energy outage, the MNO may manage the 5G nodes affected by the outage to make use of the power autonomy remaining in the 5G nodes, e.g. prioritizing the delivery of resources to support the energy system operations communications. Power consumption for energy system operations must be optimized so that the service recovery can be remotely orchestrated by the energy utility.

## 7.2	Consolidated potential KPIs

Editor’s Note: 	This section summarizes the available KPI values from the various approved use cases based on v1.0.0. The KPI values in the tables may be corrected/modified aligned with the newly-added use-cases and the updates to the approved use-cases.

Table 7.2-1 KPI Table of Periodic Communication Services

| # Use case | # Experienced data rate | # availability | # Transfer interval: target value | # Message size | # Service area | # Max Allowed End-to-End latency | # density |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # 5.1 # Distributed Energy Storage -monitoring | UL: > 16 Mbit/s (urban), 640 Mbit/s (rural) DL: > 100 kbit/s (see note 1) | # DL: >99.90% | # UL: 10 ms | UL: 50 x 16 kbyte | # - | # DL:<10 ms UL:<10 ms | # >10 /km2 (urban), >100 /km2 (rural) # (storage node density, see note 2) |
| # 5.1 # Distributed Energy -Storage # Data collection | UL: > 128 kbit/s (urban), 10.4 Mbit/s (rural) DL: > 100 kbit/s (see note 1) | # DL: >99.90% | # UL: 1000 ms | UL: 50 x 26 kbyte DL: >100 kbyte | # - | # DL:<10 ms UL:<1000 ms | # >10 /km2 (urban), >[100 /km2 (rural) # (storage node density, see note 2) |
| # 5.2 # advanced metering | UL:<2 Mbit/s DL:<1 Mbit/s | # >99.99% | # - | - | # - | # General information data collection: <3000 ms # (note 3) | # <10000/km2 (connection density, note4) |
| # 5.3 # Distributed Feeder Automation | 2 Mbit/s to 10 Mbit/s | # 99.999% | # Normal:1s; # Fault:2ms | - | # - | <10 ms # (see NOTE 6)- # Latency jitter <50 µs # (see note 5) | # 54/km² # (see note 7) # 78/km2 (connection density, note 8) |
| 5.8 Smart Distribution Transformer Terminal | >2 Mbit/s (note9) |  |  |  | 100 m ~ 500 m, outdoor, indoor / deep indoor | 10 ms, 100 ms, 3 s (note10) | 500 /distribution area (note 11) |
| # 5.12 # Distribution Intelligence – FLISR # High speed current differential protection (see NOTE 13 ) | 1,2Mbit/s~2,5 Mbit/s | # > 99,999 % | # ≤ 1 ms~2ms | <245 byte | # - several km2 | # 5 ms~15ms | # ≤ 100/km2 |
| # 5.15 # Distributed Energy Resources and Micro-Grids(see NOTE 12 ) | 5,4 Mbit/s | # 99,9999 % | # ≤ 1 ms | 140 byte | # - | 3 ms | # - |
| # 5.22 # ensuring uninterrupted MTC service availability during emergencies | < 1 kbit/s per DER | # 99.9999 % | # 100 ms # (see note 13-14 ) | - | # - | # - | # - |

NOTE 1: This KPI is to require data rate in one Energy storage station which may provide via one or more 5G connections and via one or more 3GPP UE(s) at the same time. NOTE 2: It is used to deduce data volume in an area which has multiple energy storage stations. The data volume can be deduced through follow formula ( Current + other data) data rate per storage station * (Storage node density /km2) * (Active factor/km2) + video data rate per storage station * (Storage node density /km2). In general, the Active factor is 10% NOTE 3: It is one way latency from 5G IoT device to backend system while the distance between them is no more than 40 km i.e. city range. The command implementation need 100 ms. NOTE 4: It is the typical connection density in today city environment. With the evolution from meter centralization collection to sockets in home directly collection, the connection density is expected to increase 5-10 times. NOTE 5: The latency jitter is required for the switch off between the active and standby communication links NOTE 6: It is the one way delay from a distributed terminal to 5G network. NOTE 7: When the distributed terminals are deployed along overhead line, about 54 terminals will be distributed along overhead lines in one square kilometre with the power load density is 20MW/km2. NOTE 8: When the distributed terminals are deployed in power distribution cabinets, and considering the power load density is 20 MW/km2, there are about 78 terminals in one square kilometre. NOTE 9: It is the smart metering application data rate between the Smart Distribution Transformer Terminal and energy end equipment. Once there are multiple smart grid applications, it is required more data rate. NOTE 10: It depends on different applications supported by the Smart Distribution Transformer Terminal. The less the latency is, the more applications can be supported. NOTE 11: The distribution area is circular with range between 100 m and 500 m (0.031 km2 to 0.785 km2). NOTE 12: UE to UE communication is assumed. NOTE 13: Unless otherwise specified, all communication includes 1 wireless link (UE to network node or network node to UE) rather than two wireless links (UE to UE). NOTE 14: It applies to both UL and DL unless stated otherwise.

Table 7.2-2 KPI Table of Aperiodic Communication Services

| # Use case | # Max allowed # end-to-end # latency | # Experienced data rate | # Communication service availability | # Message size | # Service area | # Reliability | # Storage node density # /km2 (Note2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # 5.1 # Distributed Energy Storage # Energy storage station: video | DL:<10 ms UL:<1000 ms (rural) | UL: >5 Gbit/s DL: >100 kbit/s (see note 1) | # - | # - | # - | # DL: >99.90% | # >100 |
| # 5.2 # Advanced metering | Accuracy fee control: < 100 ms (note2); | DL:<1 Mbit/s |  |  |  | # >99,99% |  |
| # 5.12 # Distribution Intelligence – FLISR # Feeder automation # (note 3) | 20 ms | - | # > 99,999 % | < 100 byte | # several km2 | # - | # - |
| # 5.15 # Distributed Energy Resources and Micro-Grids(note 3) | <3 ms | - | # > 99,9999 % | 160 byte | # - | # - | # - |

NOTE 1: The required data rate in one Energy storage station which may provide via one or more 5G connections and one or more 3GPP UE(s) at the same time. It can be calculated with following formula:12.5 Mbytes/s * 50(containers) * 8 = 5 Gbit/s NOTE 2: The accuracy fee control latency here is for communication one way latency from backend system to 5G IoT device while the distance between them is no more than 40 km i.e. city range. NOTE 3: UE to UE communication is assumed.

Table 7.2-3 Clock Synchronization Service Performance Requirements

| # Use case | # User-specific clock synchronicity accuracy level [65] | # Number of devices in one Communication group for clock synchronisation | # Clock synchronicity requirement | # Service area | # 5GS synchronicity budget requirement |
| --- | --- | --- | --- | --- | --- |
| # 5.3 # Distributed Feeder Automation | - | # 54/km² (see note 1) 78/km2 (see note 2) | - | several km² | <10 µs |
| # 5.12 # Distribution Intelligence – FLISR # feeder automation aperiodic deterministic communication # and # High speed current differential protection periodic deterministic communication | - | ≤ 100/km2 | - | several km² | ≤ 10 µs |
| # 5.19 # Applications Using IEC 61850-9-2 Sampled Values # Smart Grid: Synchronicity between sync master and PMUs This range covers the extreme cases where the PTP clock in the end device uses 5G sync modem as direct time-source (1 µs) # The 5G sync modem acts as PTP GM or 5G sync modem provides PPS output to PTP GM at the top of the Ethernet based synchronization chain with up to 15 transparent clocks or 3 boundary clocks (250 ns). | 4 | Up to 100 UEs | <250 ns-1 µs [7] (see note3) | < 20 km² | - |
| # 5.19 # Applications Using IEC 61850-9-2 Sampled Values # Smart Grid: Power system protection in digital substation with merging units, line differential protection and synchronization | 4a | Up to 100 UEs | <10-20 µs [64] (see note3) | < 20 km² |  |
| # 5.19 # Applications Using IEC 61850-9-2 Sampled Values # Smart Grid: Event reporting and Disturbance recording use-cases | 4b | Up to 100 UEs | <1 ms [64] (see note 3) | < 20 km² |  |

NOTE 1: When the distributed terminals are deployed along overhead line, about 54 terminals will be distributed along overhead lines in one square kilometre with the power load density is 20MW/km². NOTE 2: When the distributed terminals are deployed in power distribution cabinets, and considering the power load density is 20MW/km², there are about 78 terminals in one square kilometer. NOTE 3: The clock synchronicity requirement refers to the clock synchronicity budget for the 5G system, as described in Clause 5.19.6.1.

***************** End of Change *******************