3GPP TSG- SA WG1 Meeting #94-bis-e	S1-212021draftr1

# Electronic Meeting, 5 – 9 July 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.858 | CR | 0003 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clause5.5 routing via residential IP network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Resident |  |  |  |  |  | Date: |  |  | 2021-July-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | [PR. 5.5.6-002] in the current TR 22.858 requires the 5G system to support local data routing within a Customer Premise Network for UEs. From an operator’s perspective, when UEs are authenticated and authorized by an operator to access the services the operator has deployed in the 3GPP network, the data deems to traverse the operator’s core network for the following (not exhaustive) to be possible: - To access operator depoloyed services; - To manage spectrum usage; - To enable charging; - To enable LI, subject to regulations. The local data routing within the customer’s own premise network described in clause 5.5 resembles approaches like routing in the LAN, where, among other things, the LAN routing is locally managed by the user itself, and the operator has nothing to do with it. Therefore, it is proposed to rewording this requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | [PR. 5.5.6-002] the “Subject to regulatory requirements and operator policy” is added and the accompanying Note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement will be kept that is out of the 3GPP specification scope. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - |  |  |  |  |  |  |  |  |

***************** First Change *******************

5.5.6	Potential New Requirements needed to support the use case

[PR. 5.5.6-001] The 5G system shall be able to provide QoS control for the communication path between a UE and an Evolved Residential Gateway via a Premises Radio Access Station.

[PR. 5.5.6-002] Subject to regulatory requirements and operator policy, the 5G system shall support an efficient user plane path for data traffic between two UEs through an Evolved Residential Gateway

[PR. 5.4.6-001]	The 5G system shall support routing efficiency for data traffic between a UE and a non-3GPP device through an Evolved Residential Gateway.

Editor’s Note: This requirement can be merged during potential requirement consolidation.

NOTE: For services an operator deploys in the 5G network (i.e. not in the CPN), local data routed via eRG does not apply.

[PR. 5.5.6-003] The 5G system shall support a mechanism to minimize the security impact on any PLMN or broadband access network when using an Evolved Residential Gateway.

[PR. 5.5.6-004] The 5G system shall support a mechanism to minimize the security impact on the UE when using an Evolved Residential Gateway.

[PR. 5.5.6-005] The 5G system shall enable the network operator associated with an Evolved Residential Gateway to control the security policy of an Evolved Residential Gateway.

[PR 5.5.6-006] The 5G system shall ensure an Evolved Residential Gateway does not compromise user privacy for UEs that are using the Evolved Residential Gateway, including communication confidentiality, location privacy and identity protection.

***************** End of Change *****************