3GPP TSG-SA1 Meeting #94e-bis	S1-212030r3

Electronic Meeting, 5 – 12 July 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.859 | CR | 0007 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for potential requirements of UCs 5.3, 5.5, 5.8, 5.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, vivo Mobile Communications Co. LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_PIN |  |  |  |  |  | Date: |  |  | 06/28/2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The TR identifies a number of use cases (5.3, 5.5, 5.8, 5.9) that identify a PIN Element can be a member of more than 1 PIN. In such situations the PIN Element could be using more than 2 PEGC, this is not clear in the current text and needs clarification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Considering one PIN Element can be members of more than one PIN, the following aspects are proposed for clarification and consideration of consolidation: - UC 5.9: A PIN Element can access to 5G network via PEGC or directly via direct network connection (as a UE). - UC 5.3: One PIN can have one or more PEGC; at any point of time, one PIN Element can only access to 5G network via one PEGC. - UC 5.5: 5G system support for identifying a target PIN for desired communication bewteen two PIN Elements which are both members of one or more PIN. (new) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear service requirmeents and missing general requirements for consolidation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.6, 5.5.6, 5.8.6, 5.9.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes will also have to be reflected in consolidated requirements. This is done in a separate CR for the consolidated requirements. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****Start Changes ****

### 5.3.6	Potential New Requirements needed to support the use case

[PR 5.3.6-1] For intra-PIN communications, a PIN Element shall be able to transmit media to one or more PIN Element at the same time.

[PR 5.3.6-2] A PIN Element shall support service continuity when a PIN Element changes the communication path from one PIN Element to another PIN Element. The communication path between PIN devices may include both 3GPP and non-3GPP access.

[PR 5.3.6-3] The 5G system shall be able to support a PIN which has more than one PIN Element with Gateway Capability.

Editor’s Note:	SA3 need to be consulted on the security aspects of having more than one PIN Element with Gateway Capability in the PIN.

****2nd Change ****

### 5.5.6	Potential New Requirements needed to support the use case

[PR-5.5.6-1]: The 5G system shall support secure mechanisms for a PIN Element using direct PIN connection or via PIN Element with Gateway Capability to access and communicate with another PIN Element for a PIN.

Editor's Note: evolved residential gateway (eRG) defined in 3GPP TR 22.858 [6] is assuming with PIN element gateway capability which needs further clarification in both studies.

[PR-5.5.6-2]: The 5G system shall be able to support “User Identity and Authentication” requirements (as defined in 3GPP TS 22.101 [3] clause sec 26.a) for PIN Elements of a PIN.

[PR-5.5.6-3]: The 5G system shall be able to identify a PIN and allow a PIN Element to communicate with another PIN Element of a specific PIN when they are both members of one or more PIN.

****3rd Change ****

### 5.9.6	Potential New Requirements needed to support the use case

[PR 5.9.6-1] The 5G system shall support an authorized PIN Element to access the 5G network and its services via a PIN Element with Gateway Capability, via non-3GPP access when the PIN Element is associated to a 3GPP subscription and pre-configured with credentials or via direct network connection when the PIN Element is a UE).

****End Changes ****