3GPP TSG-SA WG1 Meeting #94e 	S1-211467

Electronic Meeting, 10 – 20 May 2021	(revision of S1-210140, S1-211198r2)

Title:	Use case for Operator managed PIN

Agenda Item:	7.12.1

Source:	Lenovo, Motorola Mobility, vivo mobile communications limited, Intel

Contact:	Apostolis Salkintzis (salki@motorola.com)

Abstract: This use case describes the required support from operator to manage PIN Elements including IoT devices or non-3GPP devices in a PIN in a home or in an office.

****All New Text****

## 5.x	Operator managed PIN

### 5.x.1	Description

This use case describes the required support from operator to manage PIN Elements including IoT devices or non-3GPP devices in a PIN, in a home or in an office.

### 5.x.2	Pre-conditions

Bob has a mobile subscription with MNOa.

Bob has a smart home network using a gateway (i.e. a PIN Element with Gateway Capability) that is provided and managed by MNOa.

Bob’s smart home network supports several wireless technologies, including 3GPP direct device connection, WLAN, Bluetooth®, wireline, etc.

The 5G network of MNOa has a list (also referred to as a collection or group) of PIN Elements in Bob’s smart home network, which can be managed by the 5G network. This list is called the list of managed PIN Elements. This list contains information for each PIN Element, such as an identity, connectivity capabilities, credentials, communication restrictions (e.g. other PIN Elements it cannot communicate with), etc.

### 5.x.3	Service Flows

Step1: Bob buys a new PIN Element, e.g. a smart plug with WLAN capability.

Step2: Bob connects this PIN Element to the WLAN of his smart home network and the PIN Element obtains an IP address.

Step3: The 5G network of MNOa detects that a new PIN Element is connected to Bob’s PIN.

Step4: The 5G network acting as an Identity provider creates a User Identity and User Profile for the PIN Element based on UIA Framework in TS 22.101.

Step5: The 5G network of MNOa sends a notification to Bob’s UE (a member of the PIN) requesting permission to add this device to the list of managed PIN Elements of his PIN.

Step6: Bob grants permission and the 5G network of MNOa adds the PIN Element to the list of "managed" PIN Elements.

### 5.x.4	Post-conditions

Bob is now able to monitor and control the PIN element (and any other PIN element in the list of "managed" PIN Elements), e.g. by using a web application provided by MNOa.

### 5.x.5	Existing features partly or fully covering the use case functionality

The service requirements in TS 22.101 clause 26a provide the principle for user centric identifiers and authentication by the 3GPP system:

-  	The 3GPP System shall support operators to act as User Identity provider and to authenticate users for accessing operator and non-operator deployed (i.e. external non-3GPP) services.

This use case and requirements consider that the application running on the PIN Element is a non-operator deployed (i.e. external non-3GPP) services behind a PIN Element with Gateway Capability in PIN.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 5G system shall support a mechanism for a network operator or authorized 3rd party to create a PIN and manage the PIN Elements for the PIN.