3GPP TSG SA WG 1 Meeting 96e		 S1-214048

Electronic Meeting, 8 –18 November 2021

Title:	Reply LS on Indication of country of UE location and its use in PLMN selection

Response to:	LS C1-214778 on Indication of country of UE location and its use in PLMN selection from CT1

Release:	Rel-17

Work Item:	5GSAT_ARCH-CT

Source:	SA1

To:	CT1

Cc:	SA2, SA3-LI

Contact person:	Betsy Covell

Betsy.covell@Nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

SA1 thanks CT1 for their LS on Indication of country of UE location and its use in PLMN selection. After reviewing the questions, SA1 has the following responses.

- Should the indication of UE location represent a "country" in the manner a "country" is defined in TS 23.122, or should the "country" be the TR 22.926 definition of "a country is defined as the area embedded within a set of borders and for which a unique set of regulations applies for the provision of communication services through mobile networks"?

A1)  The indication of UE location should represent a country in the manner a country is defined in TS 23.122

- that the UE is not allowed in the current geographical position to select any PLMN which does not correspond to the indicated country of UE location; or

A2)  Yes, a UE may select a PLMN that corresponds to the indicated country of UE location while belonging to another country, e.g., NTN MCC, shared/global PLMN.

- the UE might select available and allowable PLMNs of that indicated country and other countries.

A3)  The UE can select a PLMN which corresponds to the indicated country of UE location and belongs to a different country than the one indicated as the country of UE location. In addition, as indicated in 2), the UE can select a shared/global PLMN. For other cases, the UE shall not select a PLMN of other countries.

It is up to CT1 to specify how the UE knows which PLMNs correspond to the indicated country of UE location.

# 2	Actions

To CT1

ACTION: 	SA1 asks CT1 to take these answers into account.

# 3	Dates of next TSG SA WG 1 meetings

SA1#97e	14-24 Feb 2022			Electronic Meeting

SA1#98e	09-13 May 2022			Asia – Korea or Electronic Meeting