3GPP TSG-SA1 Meeting #96e	S1-214053

Electronic Meeting, 8 – 18 November 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.878 | CR | 0003 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update CPR tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5TRS |  |  |  |  |  | Date: |  |  | 2001-11-08 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adoption of new CPR table format to facilitate traceability between use cases, PRs, and CPRs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CPRs are updated to the new table format. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Less traceability as PRs are modified into CPRs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1, 7.1.2, 7.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 7	Consolidated potential requirements and KPIs

## 7.1	Consolidated potential requirements

### 7.1.1	General

Table 7.1.1-1 – General Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.1-1 | The 5G system shall be able to receive accurate timing signals from one or more independent timing source(s), which can offer a timing alternative to GNSS, e.g. TBS/MBS [7] [8]], Sync over Fiber [7]. | [PR 5.1.6-1] |  |
| CPR 7.1.1-2 | The 5G system shall be able to maintain accurate time synchronization as appropriate for the supported applications in the event of degradation or loss of GNSS timing signals. | [PR 5.1.6-3] |  |

### 7.1.2	Monitoring and Reporting

Table 7.1.2-1 – Monitoring and Reporting Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.2-1 | The 5G system shall monitor for timing source failure. | [PR 5.2.6.1-1] |  |
| CPR 7.1.2-2 | The 5G system shall be able to detect when reference timing signals (e.g., from GNSS or other timing source) are no longer viable for network time synchronization. | [PR 5.1.6-2] |  |
| CPR 7.1.2-3 | The 5G system shall support a mechanism to determine the time uncertainty of the 5G time synchronization. | [PR 5.4.6.1-1] |  |
| CPR 7.1.2-4 | The 5G system shall be able to indicate to devices (e.g., UEs, applications) that they need to use an alternate time source (e.g., to 5G system internal holdover capability, atomic clock, Sync over Fiber, TBS, GNSS), taking into account the holdover capability of the devices. | [PR 5.2.6.1-2] |  |
| CPR 7.1.2-5 | The 5G system shall be able to detect when a timing source fails or is restored for network time synchronization. | [PR 5.2.6.1-4] |  |
| CPR 7.1.2-6 | The 5G system shall support mechanisms to monitor different time sources and adopt the most appropriate. | [PR 5.4.6.1-2] |  |
| CPR 7.1.2-7 | The 5G system shall support a mechanism to report timing resiliency information (e.g., divergence from UTC, time uncertainty) to 3rd party applications. | [PR 5.4.6.1-3] |  |

### 7.1.3	Service Exposure

Table 7.1.3-1 – Service Exposure Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.3-1 | The 5G system shall support a mechanism for a 3rd party application to request resilient timing with specific KPIs (e.g., accuracy, interval, coverage area). | [PR 5.3.6-3] |  |