3GPP TSG-SA1 Meeting #96

# Electronic Meeting, 8 Nov.-18 Nov. 2021													 S1-214082

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.847 | CR | - | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements consolidated on tactile and multi-modality service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_TACMM |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of consolidated requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of consolidated requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No consolidated requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Discussion

| PR.NO | CPR.NO | Comments |  |
| --- | --- | --- | --- |
| PR5.1.6-1 PR5.2.6-1 PR5.3.6-1 PR5.4.6-1 PR5.5.6-3 | CPR 001 | The 5G system shall be able to support tactile and multi-modality communication service with KPIs, which would be detailed in table 1. |  |
| PR5.1.6-3 PR5.3.6-4 PR5.5.6-4 | CPR 002 | The 5G system shall be able to support the synchronization thresholds to support tactile and multi-modality communication service, which would be detailed in table 2. |  |
| PR5.5.6-2 PR5.6.6-2 | CPR 003 | The 5G system shall support a mechanism to apply QoS policy for flows of multiple UEs associated with an application received from an authorized 3rd party. |  |
| PR5.2.6-2 PR5.3.6.2 PR5.5.6-1 PR5.6.6-1 | CPR 004 | The 5G system shall support a mechanism to allow an authorized 3rd party to provide QoS policy for coordination between flows of multiple UEs associated with an application. The policy may contain e.g. the set of UEs and data flows, the expected 5GS QoS handling(s) and associated triggering events, expected coordination assistance provided by 5G system between those multiple flows for different traffic types (e.g., haptic, audio and video). |  |
| PR5.1.6-2 PR5.3.6-3 | CPR 005 | The 5G system shall enable means to meet a synchronization threshold for flows of multiple UEs associated with an application based on input received from an authorized 3rd party. |  |
| PR5.7.6-1 | CPR 006 | The 5G system shall support a mechanism to ensure users’ QoE of the multi-modal communication service involving one or multiple devices at either end of the communication. QoE refers to the difference of the physical interaction across the 5G network and the same manipulation carried out locally. |  |
| PR5.7.6-2 | CPR 007 | The 5G system shall support a mechanism for a 3rd party application server to provide real-time feedback on the traffic characteristics and service requirements of the multiple streams of a multi-modal communication session. |
| PR5.7.6-3 | CPR 008 | The 5G system shall support a mechanism to assist the synchronisation between the multiple streams (e.g., haptic, audio and video) of a multi-modal communication session in order to avoid the negative impact on the user experience. |
| PR5.8.6-1 | CPR 009 | The 5G system shall be able to support the interaction with applications on UEs or data flows grouping information within one tactile and multi-modality communication service. |
| PR5.8.6-2 | CPR 010 | The 5G system shall be able to provide a dynamic mechanism to transfer different data flows with different latency to achieve a certain transmission time difference within one tactile and multi-modality communication service. |

6	Consolidated requirements

Editor’s note: This clause contains the consolidated requirements of tactile and multi-modality communication services based on analysing the requirements from use cased in clause 5.

[CPR 001] The 5G system shall be able to support tactile and multi-modality communication service with KPIs, which is detailed in table 1.

Table 1-Key performance requirements for tactile and multi-modality communication service

| Link | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | Message size (byte) | # of UEs | UE Speed | Service Area |  |
| UL: device application sever | 5 ms | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | [99.9%] (without haptic compression encoding) [99.999%] (with haptic compression encoding) | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 More DoFs may supported by the haptic device | - | Stationary or Pedestrian | TBD | Haptic feedback |
|  | 5 ms | < 1Mbit/s | [99.99%] | MTU | - | Stationary or Pedestrian | TBD | Sensing information e.g. user poisoning and view |
| DL: application sever device | 10 ms | 1-100 Mbit/s | [99.9%] | 1500 | - | Stationary or Pedestrian | TBD | Video |
|  | 10 ms | 5-512 kbit/s | [99.9%] | 50 | - | Stationary or Pedestrian | TBD | Audio |
|  | 5 ms | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | [99.9%] (without haptic compression encoding) [99.999%] (with haptic compression encoding) | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 | - | Stationary or Pedestrian | TBD | Haptic feedback |

[CPR 002] The 5G system shall be able to support the synchronization thresholds to support tactile and multi-modality communication service, which would be detailed in table 2.

Table 2- Key performance requirements for synchronization thresholds for tactile and multi-modality service

|  | synchronisation threshold |  |
| --- | --- | --- |
| audio-tactile | audio delay: [50 ms] | tactile delay: [25 ms] |
| visual-tactile | visual delay: [15 ms] | tactile delay: [50 ms] |

[CPR 003] The 5G system shall support a mechanism to apply QoS policy for flows of multiple UEs associated with an application received from an authorized 3rd party.

[CPR 004] The 5G system shall support a mechanism to allow an authorized 3rd party to provide QoS policy for coordination between flows of multiple UEs associated with an application.

[CPR 005] The 5G system shall support a mechanism to ensure users’ QoE of the multi-modal communication service involving one or multiple devices at either end of the communication.

[CPR 006] The 5G system shall enable means to meet a synchronization threshold for flows of multiple UEs associated with an application based on input received from an authorized 3rd party.

[CPR 007] The 5G system shall support a mechanism for a 3rd party application server to provide real-time feedback on the traffic characteristics and service requirements of the multiple streams of a multi-modal communication session.

[CPR 008] The 5G system shall support a mechanism to assist the synchronisation between the multiple streams of a multi-modal communication session in order to avoid the negative impact on the user experience.

[CPR 009] The 5G system shall be able to support the interaction with applications on UEs or data flows grouping information within one tactile and multi-modality communication service.

[CPR 010] The 5G system shall be able to provide a dynamic mechanism to transfer different data flows with different latency to achieve a certain transmission time difference within one tactile and multi-modality communication service.