3GPP TSG-SA1 Meeting #96e	S1-214117

Electronic Meeting, 8 – 18 November 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.847 | CR | 0009 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clean up EN and TBD in KPI table in TR22.847 clause 5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_TACMM |  |  |  |  |  | Date: |  |  | 2021-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There’s EN left in clause 5.8 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deleting EN and reword PR 5.8.6-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR 22.847 will left some unsolved problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------- The 1st change ---------

## 5.8	Virtual factory

### 5.8.1	Description

Virtual factory is an important feature in industry 4.0, which provides people on remote site easy access to factory. Virtual factory could support immersive monitor of real scene, simulation and analysis on production line planning and practical production adjustment.

Figure 5.8.1-1. Virtual Factory and a photo of the real factory floor under construction [19].

Also it can provide an approach in managing distributed realistic factory when considering establishing a complex production line which need to combine the capability of several factories.

Figure 5.8.1-2. A service-oriented virtual factory [20]

### 5.8.2	Pre-conditions

- Factory is covered by 5G signals. An industrial robot and a monitoring camera are connected to 5G network.

- The remote site where the virtual factory will be displayed is also covered by 5G signals.

### 5.8.3	Service Flows

- The factory first tells 5G network that the data flows of motion information collector of robot and monitor video need coordination.

- The motion information of the robot is collected and transferred through dataflow 1. A monitor camera captures the real movement of the industrial robot and transmit video signal through dataflow 2. Network will mark these two data flows according to the information factory provided.

- Data flow1 which carries the motion information signal will be transferred to the VR server and produce VR video according to the motion information. Dataflow 2 will be transferred directly to a screen in the remote site.

- On the remote site, there’s a VR glasses which will receive the video data from the VR server. Alongside the VR glasses there’s the monitor screen of the real robot motion.

- As the processing from motion information to VR video need time, dataflow 2 should be hold by network for a certain period. The dataflow of VR and monitor video will arrive at the remote site within a certain time window, so that users will not feel the delay between these two flows.

Figure 5.8.3-1. Figure of service flow on virtual factory

### 5.8.4	Post-conditions

Due to the coordination processing network has done, users can see the virtual factory through VR glasses and the real scene in factory on screen without feeling any delay.

### 5.8.5	Existing features partly or fully covering the use case functionality

none

### 5.8.6	Potential New Requirements needed to support the use case

[PR 5.8.6-1] 5G system shall be able to support the interaction with applications on UEs or data flows grouping information within one tactile and multi-modality communication service.

[PR 5.8.6-2] The 5G system shall support mechanisms to assist a 3rd party application to coordinate the transmission of multiple UEs’ flows (e.g., haptic, audio and video) of a multi-modal communication session and enable presenting the related tactile and multi-modal data to the user within a certain time.