3GPP TSG-SA WG1 Meeting #96-e 	S1-214130

Electronic Meeting, 08-18 Nov 2021	(revision of S1-21xxxx)

Source:	Huawei

pCR Title:	FS_TACMM resolving the ENs in clause 5.1

Draft Spec:	3GPP TR 22.847 V18.0.0

Agenda item:	7.11.1

Document for:	Approval

Contact:	Alice Li, alice.li1@huawei.com

Abstract: There are several editor’s notes in clause 5.1 “Immersive multi-modal Virtual Reality (VR) application”. This document discusses the remaining issues and proposes the way forward to resolve these ENs.

Discussion

There are several editor’s notes in clause 5.1 “Immersive multi-modal Virtual Reality (VR) application”.

Service Area

The service area for the immersive multi-modal VR application is set as “several km2” with a note:

NOTE 3:	In practice, the service area depends on the actual deployment. In some cases a local approach (e.g. the application servers are hosted at the network edge) is preferred in order to satisfy the requirements of low latency and high reliability.

Considering the end-to-end latency requirement of one way < 5ms, it is suggested to update the service area to “typically < 100 km2”

Proposal 1: To update the service area as “typically < 100 km2”.

Requirements related to “synchronization threshold”

There are a couple of ENs related to the synchronization threshold. During the course of the consolidation, it seems reasonable to update the related requirements as following.

Proposal 2:

Service exposure –

[1] The 5G network shall support mechanisms to allow an authorized 3rd party to provide QoS policy for multiple flows (e.g., haptic, audio and video) of multiple UEs associated with a multi-modal application. The policy may contain e.g. synchronization threshold(s).

[2-op#1] The 5G system shall support mechanisms to assist a 3rd party application to coordinate the transmission of multiple flows (e.g., haptic, audio and video) of a multi-modal communication session to enable presenting the related tactile and multi-modal data to the user within a certain time.

[2-op#2] The 5G system shall support mechanisms to coordinate the transmission of multiple flows (e.g., haptic, audio and video) of a multi-modal communication session based on QoS policy for the UEs associated with the application.

Performance KPIs –

[op#1] The 5G system shall enable mechanisms to meet a synchronization threshold for flows of multiple UEs associated with an application based on input received from an authorized 3rd party. The typical synchronization thresholds, see [22], [23], [24] and [25], are summarised in table 5.1.6-2.

Table: Typical synchronization thresholds for immersive multi-modality VR applications

|  | synchronization threshold (note 1) |  |
| --- | --- | --- |
| audio-tactile | audio delay: [50 ms] | tactile delay: [25 ms] |
| visual-tactile | visual delay: [15 ms] | tactile delay: [50 ms] |
| NOTE 1: For each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |

[op#2] To support immersive multi-modal VR applications, synchronization and coordination may be required between the flows (e.g. between audio/visual and tactile components) in order to avoid having a negative impact on the user experience (i.e. detecting lack of synchronization), for which the typical synchronization thresholds (see [22], [23], [24] and [25]) are summarised in table 5.1.6-2.

Table: Typical synchronization thresholds for immersive multi-modal VR applications

|  | synchronization threshold (note 1) |  |
| --- | --- | --- |
| audio-tactile | audio delay: [50 ms] | tactile delay: [25 ms] |
| visual-tactile | visual delay: [15 ms] | tactile delay: [50 ms] |
| NOTE 1: For each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |

A CR (with [2-op#1] and [op#2]) is provided to update the TR accordingly.