3GPP TSG-SA1 Meeting #96e	S1-214315

Electronic Meeting, 8-18 November 2021	(revision of S1-214287-was-S1-214027r1)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0008 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Triggering a change of communication privileges for a particular FRMCS User based on identities and/or talker status |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eFRMCS |  |  |  |  |  | Date: |  |  | 2021-11-18 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Alignment between latest revisions of UIC FRMCS Specifications and 3GPP TR 22.989 including the need to enable and disable communication privielges of FRMCS/MCX Users |  |  |  |  |  |  |  |  |
| Summary of change: |  | Triggering a change of communication privileges for a particular FRMCS User based on identities and/or talker status |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between 3GPP TR 22.9xx series and UIC FRMCS Specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.1, 9.9.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************************************Change**************************************************

### 9.9.1	Introduction

In this chapter the use cases related to authorisation of application are described, the following use case is identified

- Enabling/Disabling applications

- Robust mission critical group communication services

- Enabling/Disabling communication privileges per FRMCS User based on identities/talker status

*************************************End of Change***********************************************

****************************************Change**************************************************

### 9.9.3	Use case: Enabling/Disabling communication privileges per FRMCS User based on identities/talker status

#### 9.9.3.1	Description

In addition to enable the control of use of of FRMCS Applications, the FRMCS System shall allow the network operators and/or the FRMCS Users to control the use of FRMCS Applications with communication privileges per FRMCS User.

FRMCS User authorization shall be based on the FRMCS User Identity and/or the functional identity/ies of the FRMCS User, the functional identity of the FRMCS Equipment, and/or the talker status for ongoing communications.

Regarding this last parameter (i.e., communication privileges based on talker status), as an example, if an FRMCS User A who is involved as a listener in a voice communication, and FRMCS User B is talking in this voice communication, FRMCS User A is allowed or not to put on hold/leave/terminate the call, and/or to kick out FRMCS User B based on talker status of both FRMCS Users A and B.

#### 9.9.3.2	Pre-conditions

The FRMCS User equipment is powered on and the FRMCS Application on the FRMCS User equipment has started.

#### 9.9.3.3	Service flows

Change of FRMCS User authentication and authorisation status

Upon a change in FRMCS User authentication and authorisation status, the FRMCS System triggers a change of communication privileges (e.g., FRMCS User profiles) for that particular FRMCS User.

The following use cases are concerned:

- An FRMCS User is authorised or not to initiate a private or group voice, video or data communication;

- An FRMCS User is authorised or not to put a group voice communication on hold;

- An FRMCS User is authorised or not to leave a group voice communication;

- An FRMCS User is authorised or not to terminate a private or group voice communication;

- An FRMCS User is authorised or not to merge Railway Emergency Alerts;

- An FRMCS User is authorised or not to kick out another FRMCS User from a group voice communication;

- An FRMCS User is authorised or not to invite another FRMCS User to a group voice communication.

Change of FRMCS User registration status

Upon a change in registration status by registration / deregistration to / from a functional identity, the FRMCS System triggers a change of communication privileges (e.g., FRMCS User profiles) for that particular FRMCS User.

The following use cases are concerned:

- An FRMCS User is authorised or not to initiate a private or group voice, video or data communication;

- An FRMCS User is authorised or not to put a group voice communication on hold;

- An FRMCS User is authorised or not to leave a group voice communication;

- An FRMCS User is authorised or not to terminate a private or group voice communication;

- An FRMCS User is authorised or not to merge Railway Emergency Alerts;

- An FRMCS User is authorised or not to kick out another FRMCS User from a group voice communication;

- An FRMCS User is authorised or not to invite another FRMCS User to a group voice communication.

Change of FRMCS User’s talker status during ongoing group voice communication

Upon a change of talker status when an FRMCS User is involved in an ongoing group voice communication (i.e., the current talker becomes listener and vice-versa), the FRMCS System may be able to trigger a change of communication privileges (e.g., FRMCS User profiles) for all FRMCS Users affiliated to that group.

The following use cases are concerned:

- An FRMCS User is authorised or not to put an ongoing group voice communication on hold;

- An FRMCS User is authorised or not to leave an ongoing group voice communication;

- An FRMCS User is authorised or not to terminate an ongoing group voice communication;

- An FRMCS User is authorised or not to merge Railway Emergency Alerts;

- An FRMCS User is authorised or not to kick out another FRMCS User from an ongoing group voice communication;

- An FRMCS User is authorised or not to invite another FRMCS User to an ongoing group voice communication.

#### 9.9.3.4	Post-conditions

The FRMCS User is authorized or not to use FRMCS Applications.

#### 9.9.3.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-9.9.2-001] | When an FRMCS User performs MCX authentication and authorisation, the FRMCS System shall be able to trigger a change of communication privileges for that particular FRMCS User. | A | Partly covered (refer to communication privileges below) | Identification of the targeted user of the communication privilege status change: partly covered by MCX authentication and authorisation mechanism (MC ID only, no FA). Change of user profiles on MCX authentication and authorisation: covered MCX User Service Authorization is the function that validates whether or not a MCX user has the authority to access certain MCX services. |
| [R-9.9.2-002] | When an FRMCS User activates or deactivates Functional Alias(es), the FRMCS System shall be able to trigger a change of communication privileges for that particular FRMCS User. | A | Partly covered (refer to communication privileges below) | Identification of the targeted user of the communication privilege status change: covered by FA mechanism (MC ID + list of FA(s)). Change of user profiles on FA activation or creation of new profiles for FA: not covered. Editor’s note: CR is needed |
| [R-9.9.2-003] | When the floor is granted or released for a particular FRMCS User (including multi-talker floor control), the FRMCS System shall be able to trigger a change of communication privileges for all FRMCS Users affiliated to that group. | A | Partly covered | Identification of the targeted users of the communication privilege status change: Partly covered. Only MCPTT ID(s) and list of FA(s) of the requester(s) (floor control request & multi-talker) are known by the system. Editor’s note: CR is needed Change of user profiles when floor granted/released: not covered. Editor’s note: CR is needed |
| [R-9.9.2-004] | The FRMCS System shall support Communication privilege as (1) authorisation to put a group voice communication on hold. | A | Covered | Put a group voice communication on hold: covered by the affiliation mechanism which is considered sufficient to mimic the desired behaviour. Authorisation mechanism: authorization of changing affiliation based on registration status by registration / deregistration to / from a functional identity or by talker status:not covered. Editor’s note: CR is needed |
| [R-9.9.2-005] | The FRMCS System shall support Communication privilege as (2) authorisation to leave a group voice communication. | A | Covered | Leave a group voice communication: covered by the affiliation mechanism which is considered sufficient to mimic the desired behaviour. Authorisation mechanism: authorization of changing affiliation based on registration status by registration / deregistration to / from a functional identity or by talker status:not covered. Editor’s note: CR is needed |
| [R-9.9.2-006] | The FRMCS System shall support Communication privilege as (3) authorisation to terminate a group voice communication. | A | Not covered | Terminate a group voice communication: strictly performed by the system. Authorisation mechanism: not covered. Editor’s note: CR is needed |
| [R-9.9.2-007] | The FRMCS System shall support Communication privilege as (4) authorisation to merge Railway Emergency Alerts. | A | [Need analysis] | [Need analysis – finalize potential requirements for Merging of Railway Emergency Alerts first] |
| [R-9.9.2-008] | The FRMCS System shall support Communication privilege as (5) authorisation to kick out another FRMCS User from a group voice communication. | A | Covered by 22.280 | Kick out another user: covered by remote change of affiliation (mandatory mode) 22.280 6.4.6.1. Authorisation mechanism: authorization of changing affiliation based on registration status by registration / deregistration to / from a functional identity or by talker status:not covered. Editor’s note: CR is needed |
| [R-9.9.2-009] | The FRMCS System shall support Communication privilege as (6) authorisation to invite another FRMCS User to a group voice communication. | A | Covered by 22.280 | Invite a user to join: covered by remote change of affiliation (negotiated mode) 22.280 6.4.6.2. Authorisation mechanism: authorization of changing affiliation based on registration status by registration / deregistration to / from a functional identity or by talker status:not covered. Editor’s note: CR is needed |
| [R-9.9.2-010] | The FRMCS System shall support Communication privilege as (7) authorisation to initiate private voice, video or data communication. | A | Covered by 22.280 | Initiate private voice, video or data communication: covered by MC private call request mechanism. Authorisation mechanism: covered by authorisation check based on user profile. |
| [R-9.9.2-011] | The FRMCS System shall support Communication privilege as (8) authorisation to initiate group voice, video or data communication. | A | Covered by 22.280 | Initiate group voice, video or data communication: covered by group call request mechanism. Authorisation mechanism: covered by authorisation check based on user profile. |

*************************************End of Change***********************************************