3GPP TSG-SA1 Meeting #96e	S1-214111r3

Electronic Meeting, 8 – 18 November 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.847 | CR | 0006 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TACMM Consolidation of KPIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, Interdigital, LG Electronics, China Southern Power Grid |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_TACMM |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The existing TR is missing clause 6: Consolidation of KPIs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding clause 6: Consolidation of KPIs. The consolidated KPI table consolidates the existing KPI tables in TR22.847, and also take into account related CR regarding updating KPI tables on several use cases, including four CRs on clause 5.1, 5.2, 5.4, 5.5 respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TR will be imcomplete and there’ll be no KPIs reference for downstream groups. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-T, "Technology Watch Report: The Tactile Internet", August 2014.

[3]	O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards, " Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[4]	3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications".

[5]	S. K. Sharma, I. Woungang, A. Anpalagan and S. Chatzinotas, "Toward Tactile Internet in Beyond 5G Era: Recent Advances, Current Issues, and Future Directions," in IEEE Access, vol. 8, pp. 56948-56991, 2020

[6]	3GPP TS 22.261: "Service requirements for the 5G system ".

[7]	Kwang Soon Kim, et al., " Ultrareliable and Low-Latency Communication Techniques for Tactile Internet Services", PROCEEDINGS OF THE IEEE, Vol. 107, No. 2, February 2019

[8]	SAE Maneuver Sharing and Coordinating Service Task Force, https://www.sae.org/servlets/works/committeeHome.do?comtID=TEVCSC3A.

[9]	SAE Sensor-Sharing Task Force, https://www.sae.org/servlets/works/committeeHome.do?comtID=TEVCSC3B.

[10]	M. During and K. Lemmer, "Cooperative maneuver planning for cooperative driving," IEEE Intell. Transp. Syst. Mag., vol. 8, no. 3, pp. 8–22, Jul. 2016.

[11]	D. Soldani, Y. Guo, B. Barani, P. Mogensen, I. Chih-Lin, S. Das, "5G for ultra-reliable low-latency communications". IEEE Network. 2018 Apr 2; 32(2):6-7.

[12]	O. Dubuisson and P. Fouquart, "ASN.1: Communication between Heterogeneous Systems", Morgan Kaufmann Publishers Inc., San Francisco, CA, USA, 2001.

[13]	IEEE SA, "P1918.1 - Tactile Internet: Application Scenarios, Definitions and Terminology, Architecture, Functions, and Technical Assumptions", https://standards.ieee.org/project/1918_1.html

[14]	M. Eid, J. Cha, and A. El Saddik, "Admux: An adaptive multiplexer for haptic-audio-visual data communication", IEEE Tran. Instrument. and Measurement, vol. 60, pp. 21–31, Jan 2011.

[15]	 K. Iwata, Y. Ishibashi, N. Fukushima, and S. Sugawara, "QoE assessment in haptic media, sound, and video transmission: Effect of playout buffering control", Comput. Entertain., vol. 8, pp. 12:1–12:14, Dec 2010.

[16]	N. Suzuki and S. Katsura, "Evaluation of QoS in haptic communication based on bilateral control", in IEEE Int. Conf. on Mechatronics (ICM), Feb 2013, pp. 886–891.

[17]	E. Isomura, S. Tasaka, and T. Nunome, "A multidimensional QoE monitoring system for audiovisual and haptic interactive IP communications", in IEEE Consumer Communications and Networking Conference (CCNC), Jan 2013, pp. 196–202.

[18]	A. Hamam and A. El Saddik, "Toward a mathematical model for quality of experience evaluation of haptic applications", IEEE Tran. Instrument. and Measurement, vol. 62, pp. 3315–3322, Dec 2013.

[19]	M. Back et al., "The virtual factory: Exploring 3D worlds as industrial collaboration and control environments," 2010 IEEE Virtual Reality Conference (VR), 2010, pp. 257-258

[20]	S. Schulte, D. Schuller, R. Steinmetz and S. Abels, "Plug-and-Play Virtual Factories," in IEEE Internet Computing, vol. 16, no. 5, pp. 78-82, Sept.-Oct. 2012

[21]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains"

[22]	Altinsoy, M. E., Blauert, J., & Treier, C., "Inter-Modal Effects of Non-Simultaneous Stimulus Presentation," A. Alippi (Ed.), Proceedings of the 7th International Congress on Acoustics, Rome, Italy, 2001.

[23]	Hirsh I.J., and Sherrrick C.E, 1961. J. Exp. Psychol 62, 423-432

[24]	Altinsoy, M.E. (2012). "The Quality of Auditory-Tactile Virtual Environments," Journal of the Audio Engineering Society, Vol. 60, No. 1/2, pp. 38-46, Jan.-Feb. 2012.

[25]	M. Di Luca and A. Mahnan, "Perceptual Limits of Visual-Haptic Simultaneity in Virtual Reality Interactions," 2019 IEEE World Haptics Conference (WHC), 2019, pp. 67-72, doi: 10.1109/WHC.2019.8816173.

[26]	Arnon, Shlomi, et al. "A comparative study of wireless communication network configurations for medical applications." IEEE Wireless Communications 10.1 (2003): page 56-61.

[27]	K. Antonakoglou et al., “Toward Haptic Communications Over the 5G Tactile Internet”, IEEE Communications Surveys & Tutorials, 20 (4), 2018.

========= Second Change ==========

# 6	Consolidated requirements

## 6.2	Consolidated potential KPIs

The 5G system shall support tactile and multi-modal communication services with the following KPIs.

Table 6.2-1: Multi-modal communication service performance requirements

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | Message size (byte) | UE Speed | Service Area |  |
| Immersive multi-modal VR (UL: device application sever) | 5 ms (note 2) | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.9% (without haptic compression encoding) 99.999% (with haptic compression encoding) [3] | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 More DoFs can be supported by the haptic device | Stationary or Pedestrian | typically < 100 km2 (note 5) | Haptic feedback |
|  | 5 ms | < 1Mbit/s | 99.99% [3] | 1500 | Stationary or Pedestrian | typically < 100 km2 (note 5) | Sensing information e.g. position and view information generated by the VR glasses |
| Immersive multi-modal VR (DL: application sever device) | 10 ms (note1) | 1-100 Mbit/s | 99.9% [3] | 1500 | Stationary or Pedestrian | typically < 100 km2 (note 5) | Video |
|  | 10 ms | 5-512 kbit/s | 99.9% [3] | 50 | Stationary or Pedestrian | typically < 100 km2 (note 5) | Audio |
|  | 5 ms (note 2) | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.9% (without haptic compression encoding) 99.999% (with haptic compression encoding) [3] | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 | Stationary or Pedestrian | typically < 100 km2 (note 5) | Haptic feedback |
| Remote control robot | 1-20ms | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.999% [3] | 2-8 (1 DoF) | high-dynamic (≤ 50 km/h) | ≤ 1 km2 | Haptic feedback |
|  | 20-100ms | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.999% [3] | 2-8 (1 DoF) | Stationary or Pedestrian | ≤ 1 km2 | Haptic feedback |
|  | 5 ms | 1-100 Mbit/s | 99.999% [3] | 1500 | Stationary or Pedestrian | ≤ 1 km2 | Video |
|  | 5 ms | 5-512 kbit/s | 99.9% [3] | 50-100 | Stationary or Pedestrian | ≤ 1 km2 | Audio |
|  | 5 ms | < 1Mbit/s | 99.999% [3] | - | Stationary or Pedestrian | ≤ 1 km2 | Sensor information |
| Skillset sharing low- dynamic robotics (including teleoperation) Controller to controlee | 5-10ms | 0.8 - 200 kbit/s (with compression) | 99,999% [3][27] | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 | Stationary or Pedestrian | 100 km2 | Haptic (position, velocity) |
| Skillset sharing low- dynamic robotics (including teleoperation) Controlee to controller | 5-10ms | 0.8 - 200 kbit/s (with compression) | 99,999% [3][27] | 1 DoF: 2-8 10 DoFs: 20-80 100 DoFs: 200-800 | Stationary or Pedestrian | 100 km2 | Haptic feedback |
|  | 10ms | 1-100 Mbit/s | 99,999% [3] [27] | 1500 | Stationary or Pedestrian | 100 km2 | Video |
|  | 10ms | 5-512 kbit/s | 99,9% [3] [27] | 50 | Stationary or Pedestrian | 100 km2 | Audio |
| Skillset sharing Highly dynamic/ mobile robotics Controller to controlee | 1-5ms | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99,999% (with compression) 99,9% (w/o compression) [3] [27] | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 | high-dynamic | 4 km2 | Haptic (position, velocity) |
| Skillset sharing Highly dynamic/ mobile robotics Controlee to controller | 1-5ms | 0.8 - 200 kbit/s | 99,999% (with compression) 99,9% (w/o compression) [3] [27] | 1 DoF: 2-8 10 DoFs: 20-80 100 DoFs: 200-800 | high-dynamic | 4 km2 | Haptic feedback |
|  | 1-10ms | 1-10 Mbit/s | 99,999% [3] [27] | 2000-4000 | high-dynamic | 4 km2 | Video |
|  | 1-10ms | 100-500 kbit/s | 99,9% [3] [27] | 100 | high-dynamic | 4 km2 | Audio |
| Immersive multi-modal navigation applications Remote Site Local Site (DL) | 50 ms [39] | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.999 % [3] | 1 DoF: 2 to 8 10 DoF: 20 to 80 100 DoF: 200 to 800 | Stationary or Pedestrian | ≤ 100 km2 ( note 5) | Haptic feedback |
|  | <400 ms [39] | 1-100 Mbit/s | 99.999 % [3] | 1500 | Stationary/ or Pedestrian, | ≤ 100 km2 (note 5) | Video |
|  | <150 ms [39] | 5-512 kbit/s | 99.9 % [3] | 50 | Stationary or Pedestrian | ≤ 100 km2 (note 5) | Audio |
|  | <300 ms | 600 Mbit/s | 99.9 % [3] | 1500 | Stationary or Pedestrian | ≤ 100 km2 (note 5) | VR |
| Local Site Remote Site (UL) | <300 ms | 12 kbit/s [26] | 99.999 % [3] | 1500 | Stationary or Pedestrian | ≤ 100 km2 (note 5) | Biometric / Affective |
|  | <400 ms [39] | 1-100 Mbit/s | 99.999 % [3] | 1500 | Workers: Stationary/ or Pedestrian, UAV: [30-300mph] | ≤ 100 km2 (note 5) | Video |
|  | <150 ms [39] | 5-512 kbit/s | 99.9 % [3] | 50 | Stationary or Pedestrian | ≤ 100 km2 (note 5) | Audio |
|  | <300 ms | 600 Mbit/s | 99.9 % [3] | 1500 | Stationary or Pedestrian | ≤ 100 km2 (note 5) | VR |
| NOTE 1: Motion-to-photon delay (the time difference between the user’s motion and corresponding change of the video image on display) is less than 20 ms, and the communication latency for transferring the packets of one audio-visual media is less than 10 ms, e.g. the packets corresponding to one video/audio frame are transferred to the devices within 10 ms. NOTE 2: According to IEEE 1918.1 [3] as for haptic feedback, the latency is less than 25 ms for accurately completing haptic operations. As rendering and hardware introduce some delay, the communication delay for haptic modality can be reasonably less than 5 ms, i.e. the packets related to one haptic feedback are transferred to the devices within 10 ms. NOTE 3: Haptic feedback is typically haptic signal, such as force level, torque level, vibration and texture. NOTE 4: The latency requirements are expected to be satisfied even when multimodal communication for skillset sharing is via indirect network connection (i.e., relayed by one UE to network relay). NOTE 5: In practice, the service area depends on the actual deployment. In some cases a local approach (e.g. the application servers are hosted at the network edge) is preferred in order to satisfy the requirements of low latency and high reliability. |  |  |  |  |  |  |  |