3GPP TSG-SA WG1 Meeting #97e 	S1-220105

Electronic Meeting, 14 – 24 Feb. 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0633 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of KPI in TS22.261 clause 7.10 for AMMT use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMMT |  |  |  |  |  | Date: |  |  | 2022-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the discussion and change in S1-220103, the round-trip latency for robitic control needs to be clarified in 22.261. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the round-trip latency with certain UL and DL payload size for use case “split control for robotics” in 7.10 TS22.261. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some requirements are not clarified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 22.874 CR 0007 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** Start of Change ******************

7.10	KPIs for AI/ML model transfer in 5GS

The 5G system shall support split AI/ML inference between UE and Network Server/Application function with performance requirements as given in Table 7.10-1.

Table 7.10-1 KPI Table of split AI/ML inference between UE and Network Server/Application function

| # Uplink KPI |  |  |  |  | # Downlink KPI |  |  |  |  | # Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Max allowed UL end-to-end latency | # Experienced data rate | # Payload size | # Communication service availability | # Reliability | # Max allowed DL end-to-end latency | # Experienced data rate | # Payload size | # Reliability | # Round-trip latency |  |
| 2ms | 1.08Gbit/s | # 0.27 MByte | # 99.999 % | # 99.9% |  |  |  | # 99.999% |  | # Split AI/ML image recognition |
| 100ms | 1.5Mbit/s |  |  |  | # 100ms | # 150 Mbit/s | 1.5 MByte /frame |  |  | # Enhanced media recognition |
| # 1ms # (see NOTE2) | # 4.7Mbit/s (see NOTE2) | 592 Byte |  |  | 12ms (see NOTE 2) | 320Mbit/s (see NOTE 2) | 40kByte |  | 13ms (UL+DL) | Split control for robotics |
| NOTE 1: Communication service availability relates to the service interfaces, and reliability relates to a given system entity. One or more retransmissions of network layer packets may take place in order to satisfy the reliability requirement. NOTE 2: The UL and DL latency and data rate can be adjusted as long as the transfer of data payload (UL: 592Bytes, DL: 40kBytes) is completed within the round-trip latency (13ms). |  |  |  |  |  |  |  |  |  |  |

The 5G system shall support AI/ML model downloading with performance requirements as given in Table 7.10-2.

*******************End of Change*****************