3GPP TSG SA WG 1 Meeting 97e		 S1-220186

Electronic Meeting, 14 – 24 February 2022

Title:	Reply LS on ad hoc group communication

Response to:	LS S6-212694 (S1-220058)

Release:	Release 18

Work Item:

Source:	SA1

To:	SA6

Cc:

Contact person:	Erik Guttman

erik.guttman@samsung.com

+49 172 916 6662

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA1 thanks SA6 for their LS on ad hoc group communication, and provides answers to the questions asked.

Q1: Is the Group ID required for MCX Service or an authorized MCX user to establish Ad hoc group communication?

SA1 observes that Ad hoc group communication is similar to group communication in most of the aspects except that the Group doesn’t exist beforehand. Many operations rely on the Group ID, for example upgrade the ongoing call to emergency, for logging purposes, etc. All these aspects are not addressed at stage 1.

Answer: Ad hoc group communication is another type of group communication and so SA1 confirms that a Group ID is required for establishment of Ad hoc group communication.

Whether the MCX Service is required to enable the MCX users participating in the Ad hoc group communication to receive the Ad hoc group related information (e.g. Group ID) prior to or during the Ad hoc group communication?

Answer: The MCX Service is required to enable the MCX users participating in the Ad hoc group communication to receive Ad hoc group related information including the Group ID during the Ad hoc group communication. Whether the group ID provided by the MCX service system or by the initiator of the call must is not determined at stage 1 as this is a stage 2 aspect.

Q2: 3GPP TS 22.280 sub clause 6.15.5.2 and 6.15.5.3 capture the below requirements:

[R-6.15.5.2-004] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure additional conditions under which MCX Service Ad hoc Group Communication shall be terminated (e.g., last Participant leaving, second last Participant leaving, initiator leaving).

[R-6.15.5.3-001] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure which MCX Users, within their authority, are authorized to initiate a MCX Service Ad hoc Group Communication.

[R-6.15.5.3-002] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure the maximum number of MCX Users who can participate in a MCX Service Ad hoc Group Communication.

[R-6.15.5.3-003] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure which MCX Users are authorized to participate in a MCX Service Ad hoc Group Communication. [TS 22.280 R-6.7.2-003]

[R-6.15.5.3-004] The MCX Service shall provide a mechanism for an MCX Service Administrator to define the default parameters for MCX Service Ad hoc Group communication (e.g., priority, hang time, broadcast mode).

[R-6.15.5.3-005] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure whether MCX Service Ad hoc Group Communication is allowed on the MCX system regardless of individual MCX User authorizations.

Clarify whether the MCX Service is required to enable the MCX users participating the Ad hoc group communication to receive the above configurations or default parameters prior to or during the ad hoc group communication,

Answer: The requirements listed above provide specific configurations that must be available in order for Ad hoc group communication to occur. It is up to SA6 to decide which parameters must be provided to the MCX users, the MCX system, or both. It is also up to SA6 to determine when parameters are provided (prior or during the ad hoc group communication.)

and whether the MCX Service is required to provide configurations for Ad hoc group communication differently than providing the normal group configurations for other type of MC service group communication.

Answer: SA1 responds that from a functional perspective, the configurations for Ad hoc group communication differ from other forms of group communication. How this configuration is to be specified, interpreted or provisioned is a matter for SA6 to decide.

# 2	Actions

To the SA6 group

ACTION:  SA1 asks that SA6 take the answers provided above into account.

# 3	Dates of next TSG SA WG 1 meetings

SA1#98e	09-19 May 2022			Electronic Meeting

SA1#99e	22-26 August 2022 or TBD	Goteborg, SE or Electronic Meeting