3GPP TSG SA WG 1 Meeting 98e		 S1-221043

Electronic Meeting, 9 – 19 May 2022

Title:	[Draft] Reply LS on MINT and Higher priority PLMN Selection

Response to:	LS C1-220816 on MINT and Higher priority PLMN Selection from CT1

Release:	Rel-17

Work Item:	MINT

Source:	Apple [SA1]

To:	CT1

Cc:

Contact person:	Mona Mustapha at Apple dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	n/a

# 1	Overall description

SA1 thanks CT1 for their LS on MINT and Higher priority PLMN Selection in C1-220816.

Question 1) Is the UE allowed or even mandated to scan for all available PLMNs in order to obtain service on an allowable PLMN when registered for disaster roaming service?

SA1 response:

For Case 1 in relation to question 1: 
As the UE is already receiving services from the Forbidden PLMN providing disaster roaming and there is no higher priority PLMN other than the PLMNs under disaster, the UE is not mandated to scan for other allowable PLMNs and the UE shall remain registered to the Forbidden PLMN.

Question 2) Is the UE allowed or even mandated to attempt service on such other allowable PLMNs when registered for disaster roaming service or shall the UE continue to use disaster roaming service on the FPLMN?

SA1 response:

For Case 2 in relation to question 2: 
If the UE has performed a PLMN scan to attempt to select a higher priority PLMN and could not register to a higher priority PLMN, the UE shall remain registered to the Forbidden PLMN.

Question 3) The interval of time between searches for a higher priority PLMN is configured by HPLMN. Can SA1 kindly provide the reasoning whether this interval of time needs be different/adjusted for the UE in disaster roaming condition?

SA1 response:

The purpose of the timer is to ensure that the UE moves to a higher priority PLMN and is independent of whether the UE is under disaster roaming. Therefore, the interval of time does not need to be different / adjusted.

# 2	Actions

To CT1

ACTION: 	SA1 asks CT1 to take the above clarification into account.

# 3	Dates of next TSG SA WG 1 meetings

SA1#99		22-26 Aug 2022			Goteborg, Sweden (fallback: Electronic Meeting)

SA1#100	07-11 Nov 2022			TBD, North America (fallback: Electronic Meeting)