3GPP SA WG1 Meeting #98e 	S1-221036r3

Electronic Meeting, 09 - 19 May 2022

Source: 	IIT Bombay

Title:		New SID on treating (UE) signalling as a user service

Document for:	Approval

Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: SID on treating (UE) signalling as a user service

Acronym: FS_SigAsServ

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No |  |  |  |  |  |
| Don't know |  | X |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  |  | Feature |
| --- | --- | --- |
|  |  | Building Block |
|  |  | Work Task |
| X | Study Item |  |  |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | N/A |

Dependency on non-3GPP (draft) specification: N/A

# 3	Justification

High Level Scope of the Study

The control plane (CP) network functions (AMF, gNB-CU-CP etc.) of the 5G system (5GS) exchanges signalling messages with UEs (TS 22.261, Clause 6.4.2.4  Efficient Control Plane), for example, Non Access Stratum (NAS) messages related to UE “Registration”, “Identity”, or “Authentication” etc. and AS messages such as “RRCSetup” or “MeasurementReport”.

The exchange of signalling messages between the UEs and the 5GS requires a path to be established through the 5G network, e.g., SRB(s) over the radio link. Even though these signalling paths are very similar to data paths such as DRBs for PDU Sessions, the 5GS treats UE signalling traffic differently from the user data traffic.

Scope: As part of this SID, we would like to study if it is possible to treat “UE signalling” as a user (data) service, i.e., if the signalling traffic and the user data traffic can be treated in a similar manner by the 5GS and if yes then are there any advantages thereof?

While we understand that a significant part of the study may deal with modifications in 5G system architecture however the proposal has been brought to SA1 since the study has direct impact on some of the stage 1 requirements such as the ones identified in “TS 22.261, Clause 6.4.2.4  Efficient Control Plane”.

Explanation of the Proposal and Potential Impact on 5GS architecture

A key difference between the signalling traffic and the data traffic in the 5GS is that their end points are different on the network side. While the end points for the signalling traffic (in the 5G network) are the control plane network functions such as gNB-CU-CP, AMF or AUSF as shown in figure 1 below, the end points for the data traffic may be network functions in the Data Network (DN) beyond the N6 interface.

Figure 1: Signalling and Data Paths in existing 5GS

The data network may contain network functions like Application Functions (AF) and/or Application Servers (AS), which may be used to provide different  types of data services to UEs. An example AF/AS are 5G Downlink Media Streaming (5GMSd) AF and AS [TS 26.501]. The 5GMSd AF and AS are data network functions and used for media delivery to UEs in downlink direction. The 5GMSd AF and AS communicate with the UEs via N6 interface and the 5G network user plane as has been shown in figure 2. AF may also communicate with the 5GS CP via N5/N33 interfaces to provide the service delivery related requirements to the 5GS CP. Along with the streaming application data, the AF/AS may also exchange some application plane signalling with UEs. This application plane signalling is also exchanged via the 5G netork user plane (N6 interface) similar to streaming data, i.e., the application signalling is also treated as data by the 5G network.

Figure 2: Application Provider Services (Media Streaming) in 5GS (TS 26.501]

Borrowing from the AF/AS based service delivery architecture, we provide a scheme (an architecture) on how (a modified) 5GS may be able to support similar treatment for UE signalling and user (data) services. It should be noted that the scheme has been provided for the purpose of illustration only and is by no means a proposal for consideration and approval.

Similar to how AF/AS acts as the end points to provide data services to users, it may be possible to define dedicated signalling service functions (e.g., Authentication server or NAS server etc.) as end points in the 5GS. These signalling service functions are responsible for exchange of signalling messages such as NAS “Registration” or “Authentication” messages with UEs via signalling paths established through the 5GS user plane. These signalling service functions may also be independent of the CP network functions and the modified 5GS CP network functions may not contains the UE signalling functionality. The CP network functions may primarily be responsible for control and management of user plane functions such as UPF, gNB-CU-UP, gNB-DU, i.e., for setting up the data paths (Radio Bearers, GTP Tunnels) through the user plane of the 5G network, including the paths for the signalling messages. As mentioned above the modified CP network functions may not be responsible for the exchange of signalling messages with UEs in this scheme unlike the existing 5GS CP network functions. The scheme (architecture) has been shown in figure 3.

Figure 3: Signalling as a Service in 5GS, modelled on AF/AS based service delivery architecture

Summary of the impact of the proposal on the 5G system archtecture (The architecture details has been provided here for the purpose of illustration only.)

Control plane network function in the existing 3GPP 5G system essentially comprise two types of functionalities

- UE Signalling functionality - Signalling exchange with UE, e.g., exchange of NAS or RRC messages

- User Plane Control and management functionality – Control and management of user plane functions, such as UPF in Core, and gNB- CU-UP and gNB-DU in RAN

- Both these functionalities are tightly coupled, e.g., as in gNB-CU-CP, AMF and SMF.

The proposal intends to decouple the “UE signalling” and “User Plane control” functionalities in the control plane (network functions) and move the “UE signalling functionality” out of the control plane and place them as “signalling service functions” in the user plane. As a result of this proposal the existing control plane (network side) of the 5GS shall be split into separate “Signalling service functions” and “User Plane control network functions”.

- Signalling service functions (signalling end points):

1.Responsible for Signalling exchange with UEs and UE control & state management.

2. An example here is that part of AMF & SMF functionality, which handles NAS messages. This NAS message handling part of AMF & SMF can be moved out of CN CP and become a "signalling end 	point or “NAS signalling server" and be placed as a service function in the network.

3. Similarly in RAN CP, the UE “RRC message handling part" of gNB-CU-CP can be moved out of gNB-CU-CP and be placed as a signalling service function in RAN, a “RRC signalling server”.

4. There can be additional signalling service functions, e.g., “Authentication server” in the core handling UE Authentication.

5. These functions shall be moved out of the control plane and placed as service functions in user plane. These signalling service functions can use the existing (or even new) user plane functions e.g. gNB-DU and possibly gNB-CU-UP and UPF to exchange the signalling messages with UEs. The signalling messages can be carried as flows over the user plane with specific QoS requirements. It is similar to the usage of SRBs for carrying UE specific RRC messages via gNB-DU. The SRBs are not much different from DRBs.

- User Plane control network Functions:

- To control user plane functions such as UPF in Core and gNB-CU-UP and gNB-DU in RAN respectively through protocols like PFCP, E1AP and F1AP.

- These network functions do not exchange signalling messages with UEs, that is the job of signalling service functions.

- These functions remain part of the control plane.

The proposed changes are not expected to have any impact on the UE side.

Figure 1 shows the existing 5G network archiecture and how UE signalling and the user data flow through the 5G network. It also shows the signalling end points (CP NFs) in the existing 5G network such as AMF etc. Figure 3 shows the modified 5G network architecture (as per the scheme discussed above) which can support the SID proposal, i.e., can treat UE signalling as a user service. Figure 1 shows that the signalling paths terminate in the control plane network functions (at signalling end points like AMF) in the existing 5GS, whereas figure 3 shows that both the signalling end points as well as the signalling paths are part of the user plane. There may not be any difference between the signalling messages and the user data pertaining to various data services in this scheme. They both can be viewed as bits of information, flowing through the 5G network, albeit with different quality of service requirements.

Benefits of the Proposal

- Apparently, treating UE signalling messages as a user (data) service may have certain advantages. A few of these have been listed below:

Enhanced Modularity & Disaggregation of Control Plane - The approach brings enhanced modularity to the control plane and also further disaggregation to control plane functions, e.g., as a result of this proposal, the gNB-CU-CP may be split in two parts:

  - One part handles the configuration and management of gNB-DU and gNB-CU-UP via F1AP and E1AP protocols, this is essentially the “user plane control” functionality.

  - The other part handles the signalling exchange with UEs (RRC signalling server). This can be moved out of gNB-CU-CP and placed as a service function in user plane.

  - Similar impact may be there in the CN CP

- Ease of support to a set of requirements – This functionality split brings enhanced flexibility and ease of support to many 5GS requirements, e.g. the following requirements:

  - 5G system expects different signalling overheads in different scenarios such as

    - For different network slices - “TS 22.261, Clause 6.1 Network Slicing” - “5G system shall minimize signalling exchange and service interruption time for a network slice…”

    - For UEs with different mobility management needs - “TS 22.261, Clause 6.2 Diverse mobility management” - “The flexible nature of a 5G system will support different mobility management methods that minimize signalling overhead and optimize access for these different types of UEs.”

  - Such requirements can be supported with ease through deployment of different signalling servers, which can support different signalling needs for different types of UEs, slices etc. For example, it may be possible to have slice specific signalling service functions which can support slice specific minimization of signalling exchange with ease. Signalling servers may also be deployed dynamically to take care of the dynamically changing signalling overhead requirements.

  - Similarly, if there is a need to support different signalling latencies for different types of UEs, that may also be supported quite easily with flexible placement of signalling servers, e.g., an “RRC signalling server” can be placed close to gNB-DU to support lower signalling latency to specific types of UEs. This is different from the existing 5GS where the “RRC signalling functionality” is necessarily a part of gNB-CU-CP and all UEs connected to the gNB may experience the same signalling latency.

- Flexible function placement and chaining for signalling service

  - Different signalling servers such as RRC signalling server, NAS signalling server, Authentication signalling server… can be placed and chained together flexibly to serve UEs with different signalling requirements. As highlighted above such a deployment may allow deployment of slice specific “signalling servers” with ease and in different locations in the network (e.g., near gNB-DU, near gNB-CU-UP etc.), if needed.

- Simpler Protocols

  - With decoupling of “user plane control” functionality from “UE signalling” handling, some of the protocols may become simpler, for example, F1AP - is used for UE context management, interface management of gNB-DU and also for RRC message transfer to UEs. With the proposed decoupling of functionality, F1AP protocol can be simplified and would not need to support RRC message transfer as that can be handled separately between the RRC signalling servers and the user plane functions.

  - Similarly, the NAS message transport functionality would not be required in NGAP protocol, simplifying the NGAP protocol as NAS messages can directly be exchanged by the NAS signalling server with the RRC signalling server.

This proposal is being submitted to SA1 as there are requirements in the 5GS Stage 1 specifications that need a relook as part of this study, e.g., the requirements such as “The 5G system shall minimize the signalling that is required prior to user data transmission” under the clause “6.4.2.4 Efficient control plane” of TS 22.261[2]. The requirement implies that control plane is responsible for signalling exchange with UEs. Please refer to the specification TS 22.261 (e.g., clause 6.4 Resource efficiency) for more such requirements and assumptions.

References:

3GPP TS 26.501, "5G Media Streaming (5GMS); General description and architecture", V17.1.0 (2022-03)

3GPP TS 22.261, "Service requirements for the 5G system; Stage 1", V18.6.0 (2022-03)

# 4	Objective

The objective of the proposed study is to:

- Analyze the feasibility of treating the signalling message exchange with UEs as a user (data) service, i.e., if the signalling messages can be treated in a manner similar to user data transfer by the 5GS and if this approach to signalling exchange is feasible.

- Analyze if it is possible to separate the signalling exchange functionality from the control plane functionality in the 5GS.

- Identify the benefits of the proposed approach both in terms of quantitative and qualitative performance parameters such as flexibility, modularity, scalability, resource efficiency etc.

- Study the impact of the proposed approach on other services offered by the 5GS such as “network slicing”.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| External TR | TBD | Study on treating (UE) signalling as a user service | TSG#102 | TSG#103 | Jha, Pranav, IIT Bombay, pranavjha@ee.iitb.ac.in |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Jha, Pranav, IIT Bombay, pranavjha@ee.iitb.ac.in

# 7	Work item leadership

SA1

# 8	Aspects that involve other WGs

None identified yet

# 9	Supporting Individual Members

Supporting IM name

IIT Bombay

Tejas Networks

IIT Kanpur

Saankhya Labs

CEWiT