3GPP TSG-SA WG1 Meeting #98e 	S1-221054r5

Electronic Meeting, 9 – 19 May 2022	(revision of S1-22xxxx)

Source:	Korea Railroad Research Institute (KRRI)

pCR Title:	Pseudo-CR on use case of multiple trains’ stops at the same platform

Draft Spec:	3GPP TR 22.890

Agenda item:	7.1 FS_RAILSS

Document for:	Approval

Contact:	Woojin Ahn <woojin.ahn@krri.re.kr>, Ilmu Byun <ilmubyun@krri.re.kr>

Abstract: This contribution proposes the requirements of multiple trains’ stops at the same platform in chapter 6 of TR22.890

1. Introduction

This pCR suggests the use case of multiple trains’ stops at the same platform.

2. Reason for Change

This pCR provides a new use case for TR22.890.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

We propose to add the following feature to 3GPP TR22.890.

* * * First Change * * * *

## 6.x	Multiple trains’ stops at the same platform

### 6.x.1	Description

It is needed to reduce train intervals to increase the track capacity. If the train intervals are less than some threshold, multiple trains should stop at the same platform. This is because we can reduce the interval between trains, but for safety reasons, reducing the time for passengers to get on and off and increasing the speed of trains entering the station is limited. In general, it takes more than 1 minute for the train to enter and exit the platform, including the time for passengers to get on and off, so if the train interval becomes shorter than 1 minute, there could be two trains on the platform.

The scenario when two trains stop at the same platform is as follows:

- The passenger information system (PIS) of a smart station displays that Train 1 will stop in front of the platform, and Train 2 will stop at the back of the platform. Train 1 & 2 may have different routes.

- Step 1:  A previous train is departing the platform, and Train 1 enters the platform while a part of the previous train is still at the platform. According to the previous train's location, Train 1 slows down its speed and goes in front of the platform.

- Step 2: After the previous train left the platform, Train 1 stops in front of the platform.

- Step 3: While Train 1 is stopping for passengers to get on and off, Train 2 enters the platform

- Step 4: Train 2 stops behind Train 1 and opens its doors. While the passengers of Train 2 are getting on and off, Train 1 start to depart the platform.

Figure 6.x. 1. Two trains’ stops at the same platform

### 6.x.2	Pre-conditions

- Each train has at least one onboard UE (i.e., FRMCS UE), supporting both on-network and off-network communications.

- There is an edge server per station and the edge server can determine which platform trains stop and transmit/receive data to onboard UEs of Trains 1&2 through the 3GPP network.

- Each edge server can transmit/receive information to the PIS.

- Onboard UEs know the identities of the edge server on the train route.

### 6.x.3	Service Flows

1. Train 1 is stopping at Platform A of the smart station, and Train 2 is approaching. Trains 1 & 2 are connected and authorized to transmit/receive information to the server at the smart station.

2. The server determines that Train 2 stops at Platform A. The server informs Train 2 of the platform where Train 2 will stop, the stop location within the platform, and the existence of Train 1 at the platform. Also, the server informs Train 1 that Train 2 will stop behind Train 1.

3. Train 2 establishes a connection with Train 1 through on-network and then notifies the server of the connection establishment with Train 1. Trains 1&2 can share information such as acceleration/deceleration, braking, location, etc., through the connection to stop at the same platform.

4. The server allows Train 2 to enter the platform.

5. For redundancy, Train 2 can add a connection with Train 1 through off-network before entering the platform.

6. Train 2 stops behind Train 1 at Platform A and opens its doors for passengers to get on and off. Train 1 starts to depart.

### 6.x.4	Post-conditions

- Train 1 establishes a connection with the server at the next station while Train 1 has a connection with the server at the current station.

- When the distance between Trains 1 and 2 becomes longer, Trains 1 and 2 stop sharing the information and disconnect from each other.

### 6.x.5	Existing features partly or fully covering the use case functionality

TR 22.990 covers utilizing off-network and on-network communications at the same time and the traffic characteristic of off-network communications.

### 6.x.6	Potential New Requirements needed to support the use case

### 6.x.6.1	Requirements related to the Service layer

[PR 6.x.6.1-1] FRMCS UEs shall connect multiple edge servers simultaneously which are located along rail tracks.

Note: The above requirement is intended to be included in Section 5.5 of TS22.282[x].

[Editor’s note: The potential requirements for FRMCS UEs to identify the edge servers are FFS]

[Editor’s note: The potential requirements for edge servers to authorize the FRMCS UEs are FFS]

### 6.x.6.2	Requirements related to the Transport layer

[PR 6.x.6.2-2] The FRMCS System shall support the following traffic characteristics of data transfer:

Note: This table is intended to be included in Section 6.2 of TS 22.289[y].

| Scenario (Note 5) | End-to-end latency | Reliability (Note 1) | UE speed | UE Relative Speed | User experienced data rate | Payload size (Note 2) | Area traffic density | Overall UE density | Service area dimension (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Multiple trains’ stops at the same platform (Korea, urban railway) | ≤10 ms | 99.9999% | ≤100 km/h | ≤50km/h | ≤1Mbps | Small to large | ≤ 1 Mbps/km | ≤ 5 (100m) | ≤ 15 km along rail tracks including bad weather conditions (Note 4) |
|  | NOTE 1: Reliability as defined in TS 22.289 sub-clause 3.1. NOTE 2: Small: payload ≤ 256 octets, Medium: payload ≤512 octets; Large: payload 513 -1500 octets. NOTE 3: Estimates of maximum dimensions. NOTE 4: Non-Line-of-Sight (NLOS) between UEs shall be supported NOTE 5: Off-network traffic characteristics are not addressed in this table since it can be covered by TR22.990. |  |  |  |  |  |  |  |  |  |

Table 6.x.6.2-1: Traffic characteristics for multiple trains’ stops at the same platform