3GPP TSG-SA1 Meeting #98e 	S1-221066r4

Electronic Meeting, 09 - 19 May 2022	(revision of xx-yyxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.874 | CR | 0008 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use Case of AI model transfer management through direct device connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AIML_Ph2 |  |  |  |  |  | Date: |  |  | 2022-05-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A new use case that 5GS may further support AI model transfer management through direct device connection and it may bring new potential requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new use case and related potential requirements to be included in FS_AIML_Ph2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GS may mot have capabilities for AI model transfer management through direct device connection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is a new use case addition to an approved TR. Even though the CR is category B and there are tick boxes checked on the header, there is no functional change. There are no downstream standardization consequences of this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** First Change ******************

11.x	AI Model Transfer Management through Direct Device Connection

11.x.1	Description

Based on the earlier study in phase one, operators can provide services to help manage and distribute the AI/ML models especially in the edge server so that the UE can acquire a proper model immediately. However, when a lot of UEs requesting for the same model at the same time or the UE is blocked by barriers with poor connection with the base station, the model transfer process will become longer than expected.

To overcome this difficulty, as shown in Fig.1, a volunteer UE which is well connected to the base station can help receive and store AI/ML models first. Then, the other UEs can download AI/ML models from the volunteer UE through direct device connection. In this way, all UE can have a stable and reliable model transfer process while the radio resource of the base station can be saved. Besides, the volunteer UE can transfer the stored models to other volunteer UEs under operator’s control.

The selection of volunteer UE can be realized by local network policies and strategies. And it also can be exposed as a capability to the 3rd party company when the company wants to choose one or a few certain UEs to be volunteer UEs in an activity. For example, a travel company may assign the tour guides’ Augmented Reality (AR) headsets as volunteer UEs in a carnival. The travel company may sign a higher quality plan for tourist guides’ devices to provide better user experience for following tourists. Meanwhile, operator can benefit from the alternative open service based on AI/ML model management capabilities and may avoid low Quality of Service due to crowding direct connections to base stations during the carnival.

Fig.1 AI/ML Model management through direct device connection

11.x.2	Pre-conditions

The operator’s MEC near the Jurassic Park stores a variety of AI/ML models according to the the park company’s requirements. And it is capable to transfer the stored model to the device such as AR headset.

The operator rolls out a new high quality plan which can allow the user customizes own Service Level Agreement (SLA) for specific network address access and data (e.g. AI/ML Models) download. As a trade-off, the user’s device will help transfer the same data through direct device connection to nearby devices sharing common aspiration.

The AR headset can transfer the stored AI/ML model to the other AR headsets. However, the AR headset cannot store all models for different scenarios due to limited storage. Indeed, a model needs to be downloaded when or a few seconds before the UE first appears in the certain area.

Alice and Bob are tour guide hired by Jurassic Park.

All of AR headsets should in the coverage of the base stations serving for the Jurassic Park.

11.x.3	Service Flows

- 1. Jurassic Park provides panorama AR tour guide services in a commercial area and a tropical rainforest area. AR headsets need to download Model A and B (both are 32 bits VGGnet, 536MByte) respectively.

- 2. To provide high quality user experience, Jurassic Park company indicates to the operator that AR headsets require to download model A in area A and model B in area B.

- 3. The Jurassic Park company signs a high quality plan for tour guide Alice and Bob’s AR Headsets for providing better service to the tour group using direct device connection.

- 4. When Bob and his tour group enter area A, their headsets request for the Model A. The operator network finds they requested the same model and Bob is a signed volunteer UE, then triggers to establish a QoS acceleration for Bob’s model downloading timely. Meanwhile, the operator network indicates Bob to help transfer the model to all other UE near Bob. Also, the operator network informs all other UE near Bob that Bob can provide the model as well. The UE which is a little far from Bob (e.g. out of Bob’s coverage) will still download the model through the base station directly.

- 5. Alice and her group are 10 meters far from Bob and also move towards to area A. The operator network predicts their desire model based on their movement and finds Bob has already downloaded it based on the model transferring records. The operator network indicates Alice can request model from Bob. Meanwhile, the operator network indicates all other UE near Alice to download the model from Alice.

11.x.4	Post-conditions

1. The tourists can enjoy the continuous AR services with smooth model switchover when their location and responding models change.

- 2. Tour group’s AR headsets provides user experience of the panorama AR tour guide services that can help retrain and improve AI/ML models in operator’s MEC by Jurassic Park company (e.g. Federated/Distributed Learning).

3. the operator network performs analytics, based on network statistics and quality of experience reported by Jurassic Park company, to improve and optimized the model transfer process (e.g. setting constraints for maximum direct device connection for one volunteer UE and choose a temporary volunteer UE for sharing model transfer task).

11.x.5	Existing features partly or fully covering the use case functionality

22.261 v18.6.0 6.27

The 5G system shall be able to make the position-related data available to an application or to an application server existing within the PLMN, external to the PLMN, or in the User Equipment.

Editor's Note:	it is FFS whether other existing features will be identified.

11.x.6	Potential New Requirements needed to support the use case

[P.R.11.x-001] Subject to user consent and operator policies, the 5G system shall be able to expose capabilities to an authorized third-party to choose one or a group of UEs for AI/ML model transfer through direct device connection.

[P.R.11.x-002] Subject to user consent and operator policies, the 5G system shall be able to support a direct device connection capable UE to discover one or a group of direct device connection capable UEs when those UEs are going to enter or have entered a certain area.

Editor's Note:	it is FFS whether other potential new requirements or KPIs can be identified.

************* End of First Change ***************