3GPP TSG-SA WG1 Meeting #98e 	S1-221104r1

Electronic Meeting, 9 – 19 May 2022

Source:	Huawei, China Telecom, vivo

pCR Title:	New use case: pedestrian/animal intrusion detection in highway

Draft Spec:	3GPP TR 22.837

Agenda item:	7.2

Document for:	Approval

Contact:	     < Fangyuan Zhu (zhufangyuan@huawei.com), Chuting Yao (yaochuting@huawei.com)>

Abstract: This document proposes a new use case about pedestrian/animal intrusion detection in highway.

* * * First Change * * * *(All new changes)

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]                         W. Favoreel, "Pedestrian sensing for increased traffic safety and efficiency at signalized intersections," 2011 8th IEEE International Conference on Advanced Video and Signal Based Surveillance (AVSS), 2011, pp. 539-542, doi: 10.1109/AVSS.2011.6027406.

[y]                         Advances in Wildlife Crossing Technologies: https://highways.dot.gov/public-roads/septoct-2009/advances-wildlife-crossing-technologies.

[z]                         Protection Detection: Making Roads Safe for Drivers and Wildlife: https://onlinepubs.trb.org/onlinepubs/webinars/201118.pdf.

* * * Second Change * * * *(All new changes)

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

sensing measurement: obtaining sensing measurement data about a target object.

sensing result: the derived features and information about a target object after processing, such as being present and object dimension, which is related to a particular sensing service.

* * * Third Change * * * *(All new changes)

5	Use cases

5.x	Use case of pedestrian/animal intrusion detection in highway

5.x.1	Description

Transportation as a basic and essential industry plays one of the important roles in a human’s life. Making transportation smarter can make life more convenient and benefit economic development. Highways are an important part of smart transportation. Due to the strong road safety demand on smart transportation, it is necessary to monitor the road situation so as to make appropriate management of road traffic, give guidance or assistance information to vehicles and/or highway traffic safety administration [x].

For example, major accidents caused by pedestrians or animals crossing highways occur frequently [y] [z]. Currently, the highway supervision systems are mainly based on traditional sensors (e.g. radars, cameras) equipped in the roadside infrastructure, but there are still many problems in road supervision system, e.g. it only has partial coverage along the roadside, and the radar may be dedicated for a single usage which requires deploying different types of transportation radars in the same place to satisfy the respective sense use cases and requirements in the area of interest.

Base stations on the roadside are already used to provide 5G coverage for communication, and the radio signals can also be used to sense the environment for object detection. The base station as transmitter and receiver knows the detailed structure of the transmitted sensing signal, such as the allocation of sensing resource, the beam direction, the time/phase/amplitude differences between the sent and received signals. The distance, angles and Doppler information (i.e. velocity) can be extracted once sensing signals are received. Thanks to the native network-wide sensing capability and feasible resource cooperation, the cellular networks can become a ubiquitous radio sensing infrastructure, while providing simultaneous communication service.

5.x.2	Pre-conditions

Good partnership and cooperation are established between the road supervision department and Mobile Operator#A in City#B. Requested by the supervision departments, the base stations around/along a highway enable Mobile Operator#A to constantly sense the road situation including moving objects (e.g. vehicles and pedestrians). The sensing signal emitted from the base station arrives at vehicles/pedestrians/objects on the road and is bounced (reflected) back to the transmitting base station.

5.x.3	Service Flows

Figure 5.x.3-1: Pedestrian/animal intrusion detection

- Fei is a tourist, who is taking a taxi to enjoy the view around the highway in City#B. The base stations around/along the highway constantly sense the road situation. While the taxi is driving on the highway, Fei rolls down the window to take some pictures. Suddenly his mobile phone falls out the window.

- Fei tells the driver to stop and cautiously gets out of the taxi. He crosses the highway and wants to find his mobile phone. Meanwhile, some animals (e.g. sheep and deer) from a farm near the highway approach the road. More and more surrounding vehicles are passing at very high speed. The pedestrian and animals are detected and closely tracked with sufficient accuracy in the sensing area of a base station, and then the sensing measurement is transferred to the core network from the base station and further processed into the sensing results in the core network.

- The sensing results are exposed to the road supervision departments and map provider by the Mobile Operator#A. The map provider adds the position of the vulnerable pedestrian and animals into the HD dynamic maps and transmits warning messages to the vehicles approaching them. The staff working for supervision departments immediately responds to the emergency by warning Fei off the highway, launching temporary traffic management, and rushes to the emergency site to fetch the mobile phone for Fei and drive the animals away from the highway.

- Finally, Fei leaves the highway safely. Potential road accident(s) caused by the pedestrian/animal intrusion are avoided.

5.x.4	Post-conditions

Thanks to the area-coverage, long-distance sensing capability of the base station (which provides a bird’s-eye-view for monitoring the highway environment) the precision and efficiency of highway management and safety supervision is improved. The network-based sensing can provide timely, continuous, accurate, and comprehensive sensing results, which is a reliable basis for highway safety services.

5.x.5	Existing features partly or fully covering the use case functionality

None.

5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 5G system shall be able to provide sensing services.

[PR 5.x.6-2] The 5G system shall be able to support a base station to efficiently perform sensing.

[PR 5.x.6-3] The 5G system shall be able to support means to discover and select a suitable base station to efficiently perform sensing, e.g. based on the base station’s location, sensing capability, and request from an authorized third-party.

[PR 5.x.6-4] The 5G system shall be able to support means for an operator to configure and control the sensing operation of a base station, including authorization and management (e.g. sensing activation and/or deactivation, sensing duration, sensing accuracy, target sensing area).

[PR 5.x.6-5] The 5G system shall be able to support means to enable a base station to transfer sensing measurement data to the core network.

[PR 5.x.6-6] The 5G system shall support means to enable the core network to process the sensing measurement data and expose sensing results to an authorized third-party.

[PR 5.x.6-7] The 5G system shall support charging for the sensing services (e.g. considering service type, sensing accuracy, target area, duration) requested by an authorized third-party.

Editor’ note: The performance requirements for this use case are FFS.

* * * End of Change * * * *