3GPP SA WG1 Meeting #98e 	S1-221107r1

Electronic Meeting, 9 - 19 May 2022	(revision of S1-220199)

# Source:	China Telecom, ZTE, CEPRI, Xiaomi

# Title:	New SID on Multi-hop Multi-path Relay

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study of Multi-hop Multi-path Relay

Acronym:	FS_MultiRelay

Unique identifier:	xxxxx

Potential target Release:	Rel-19

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 720005 | New Services and Markets Technology Enablers (SMARTER) | Previous normative work includes one-hop relay |
| 840034 | Enhanced Relays for Energy Efficiency and Extensive Coverage (REFEC) | Pervious normative work includes multi-hops relay |
| 800007 | Service requirements for cyber-physical control applications in vertical domains (cyberCAV) | Previous normative work includes one-hop relay |

Dependency on non-3GPP (draft) specification:

None

3	Justification

Relay is beneficial for extending communication range. It could be used for eMBB scenarios and vertical industry scenarios. It can not only work for indirect network connection, but also for direct device connection.

- In the case of indirect network connection: Indirect network connection has been introduced in the earlier releases for IoT devices, vertical scenarios, and public safety use cases. Indirect network connection could be further enhanced for eMBB scenarios under operator’s control.

Additional use cases are proposed to study for multi-hop multi-path indirect network connection:

- Device transmits same data over different hops via different paths: When the same traffic is transmitted via different paths, there are redundant paths available if the relay of one of the paths is down or out of coverage.

- Network-assisted indirect network connection path management: considering that there may be many relays that are available, indirect network connection under operator’s control is introduced for communication performance optimization. Two examples are shown as below:

- stationary relay UE could be placed at some shops for further coverage extension, when selecting paths, based on the operator’s policy, prioritization could be placed to the stationary relay UE, and different charging rate could be applied to the stationary relay UE,

- if there is congestion of one of the paths, remote UE switches to a different path or dynamically schedules the packets among different paths based on the information provided by the network.

- Energy-effective indirect network connection: Since the remote UE and the relay UE are both on mobility, it is hard to guarantee that they are always in proximity. Hence, it might cause frequent registration update procedure, incurring unnecessary signalling and power consumption. On-demand network association with remote UE via the relay could provide the energy-effective indirect network connection.

- Aggregated QoS (e.g., bandwidth) for massive UEs behind relay: when there are massive remote UEs connected to the relay UE, some of the remote UEs require high bandwidth services (e.g., media, XR/VR services) while the relay UE could not offer the QoS for the remote UEs or while the relay UE’s QoS is limited by its subscription.

It is noted that in TS 22.261, service requirements for multi-hop relaying and different traffic flows of a remote UE to be relayed via different indirect paths are specified. However, the existing requirements does not cover the cases for same traffic flows of a remote UE through different paths, the service requirements for multi-hop multi-path network-assisted indirect network connection path management are not covered. Moreover, mobility and QoS management enhancement could be further studied for eMBB scenarios.

- In the case of direct device connection: It has been studied for cloud gaming, future factory, asset tracking etc. Considering further vertical scenarios e.g. smart mine, a lot of diverse mine equipments need to be connected and exchange information in the underground long mine tunnel, it is proposed to study multi-hop and multi-path relay for direct device connection. These relays could assist direct device connection to extend communication distance and improve reliability whether in coverage and out of coverage. The typical use cases examples are:

- To extend communication distance and improve reliability for vertical applications. For example, in smart mine scenario, the communication between the mining vehicle underground and its controller on site spans long distance. With the assistance of multiple hop relays, the long distance direct communication can be provided. And with the relays based multi-path traffic forwarding, it may provide high reliability for the direct device connection.

- For the multi-hop and multi-path assisted direct device connection serving for vertical applications, it can be provided only to the specific set of UEs according to Operators and Industry demand. For UEs not belonging the group, this kind of communication service is not applied. In this way, the data traffic can be confined in the industrial park/factory area with enhanced security.

- When multiple traffic forwarding paths applied for a given UE pair, the QoS assurance need to be considered.

- The routing path for the direct traffic between two UEs may be switched with the mobility of UE pairs. When one of the path failure or system fault happens, the connection can be dynamically switched to other data path to minimize service interruption. Taking the UE mobility and potential link failure into account, the routing path reconfiguration should be considered for service continuity.

Among existing functionalities and requirements, some partially address the direct device connection related use cases for cyber-physical control applications, V2X services and mission critical services, which are summarized as follows.

| TS No. | Title | UE-to-UE relay service requirements |
| --- | --- | --- |
| 22.278 | Service requirements for the Evolved Packet System (EPS) | ProSe Communication can also take place over a ProSe UE-to-UE Relay, a form of relay in which a Public Safety ProSe-enabled UE act as ProSe E-UTRA Communication relay between two other Public Safety ProSe-enabled UEs. |
| 22.104 | Service requirements for cyber-physical control applications in vertical domains | 5G system shall be able to support direct device connection via an UE to UE relay with respective service performance requirements related to cooperative carrying between UEs out of transmission range of each other. |
| 22.185 | Service requirements for V2X services | [R-5.2.1-001] The E-UTRA(N) shall be capable of transferring messages between two UEs supporting V2V/P application, directly or via an RSU, with a maximum latency of 100ms. NOTE: RSU is a term frequently used in existing ITS specifications, and the reason for introducing the term in the 3GPP specifications is to make the documents easier to read for the ITS industry. RSU is a logical entity that supports V2X application logic using the functionality provided by either a 3GPP network or an UE (referred to as UE-type RSU). |
| 22.280 | Mission Critical Services Common requirements | [R-7.15.1-001] The Off-Network MCX Service shall provide a means by which the MCX User of a (source) UE can make a Private Communication to the MCX User of a (target) UE via a ProSe UE-to-UE Relay. [R-7.15.2-001] An MCX Service Administrator or authorized user shall be able to configure a ProSe-enabled UE, authorized to act as a ProSe UE-to-UE Relay, to relay any received MCX Service transmissions for one (or more) specified MCX Service Groups. |

The multi-hop and multi-path direct device connection has not yet been studied. With regard to the TS 22.261, the relay can only support the indirect network connection.

So, it is proposed to investigate the use cases and potential requirements to support the multi-hop and multi-path relay for indirect network connection and direct device connection.

4	Objective

The objectives of this study are to study use cases and identify potential new requirements, including:

- Relay for indirect network connection:

  - Same traffic flows of a remote UE to be relayed via different indirect network connections paths;

  - Multi-hop multi-path relay connection, including network-assisted indirect network connection path management;

  - Mobility management enhancement for UE behind relay to avoid frequent relay discovery;

  - Aggregated QoS (e.g., bandwidth) management for massive devices using relay to access the network considering relay capability;

- Relay for direct device connection:

  - Multi-hop scenarios related with direct device connection to support extensive range communication;

  - Multi-path scenarios related with direct device connection to support high reliability;

  - Network managed configuration and management of multi-hop and multi-path assisted direct device communication service;

- Management specific UE groups which are permitted to use the multi-hop and multi-path based communication service;

- Multi-hop and multi-path configuration for QoS assurance and service continuity;

- Other aspects, including charging and security;

- Gap analysis between the identified requirements and the existing requirements.

NOTE: Multi-hop and multi-path direct device communication needs to be authorized and managed by the network.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 22.XXX | Study of Multi-hop Multi-path Relay | TSG#98 (Dec 2022) | TSG#99 (Mar 2023) | Yuying Zhang, China Telecom, zhangyy45@chinatelecom.cn |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| N/A | N/A | N/A | N/A |

6	Work item Rapporteur(s)

Yuying Zhang, China Telecom, zhangyy45@chinatelecom.cn, Primary Rapporteur

Chen Lin, ZTE, chen.lin23@zte.com.cn, Secondary Rapporteur is responsible for the objectives related with direct device connection

7	Work item leadership

SA1

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Telecom

CATT

ZTE

AT&T

China Electric Power Research Institute

CALTTA

OPPO

Tencent

Tencent Cloud

Xiaomi