3GPP TSG-SA1 Meeting #98-e	S1-221117r1

# Online, , 9th May 2022 - 19th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.268 | CR | 0068 | rev | 3 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Device based geo-fencing for EU-alert |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TNO, KPN, one2many, MINEA, Netherlands Police |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-04-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Device based geofencing is specified in 3GPP TS 22.268 as a CMAS specfic requirement from Release 15 onwards. As EU-ALERT is based on the same specifications as CMAS, the functionality technically also works for EU-ALERT. However, device based geofencing is not yet listed in the EU-ALERT specific section as functionality that applies for EU-ALERT. At TSG-SA#95-e this CR was approved for Rel-18. However, now a follow-on 23.041CR needs to be approved in CT1. That can be done for Rel-17, but only if the 22.268CR is also done for Rel-17, to keep alignment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Copy text on device based geofencing from CMAS specific section to the EU-ALERT specific section (with some modifications). Fixing editorial issues in the CMAS specific section. Text changes are exactly the same as the Rel-18 version agreed at TSG-SA#95-e. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A new Dutch law on NL-alert/EU-ALERT will come into effect at the start of 2023. It will refer to ETSI TS 122 268, the ETSI transposed version of 3GPP TS 22.268. However, an ETSI transposed version for Rel-18 will only be produced in Dec 2023 at the freezing of Rel-18, too late for the Dutch law. Dutch networks will start using geofencing by Jan 2024. UEs vendors that implement geofencing for EU-ALERT based on a Rel-18 schedule may not be ready on time and will not be able to implement Dutch regulated functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.041 CR 231 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The intention is to have this CR and 23.041CR0231 both implemented for Rel-17 to keep alignment. |  |  |  |  |  |  |  |  |

## 6.2	Additional PWS Requirements Specific to CMAS

In addition to the General Requirements specified in Clause 4, the following requirements are specified for the deployment of CMAS. These CMAS specific requirements are based on the FCC Report and Orders listed in clause 6.1 and further specified in [4].

Warning Notifications shall support up to 360 characters of GSM 7 bit Default Alphabet [9] for E-UTRAN.

The following classes of Warning Notifications shall be supported: Presidential, Imminent Threat, Public Safety, Child Abduction Emergency (e.g. AMBER) , and State/Local WEA Test.

When a Presidential Warning Notification is received, it shall always be presented to the user whenever Cell Broadcast Service via GSM/UMTS or warning notification delivery via E-UTRAN is enabled on the UE.

If the Presidential Warning Notification is received in English, then it shall be displayed by the UE. If the Presidential Warning Notification is received in a language other than English, then it shall only be displayed by the UE if the User has selected that language.

A CMAS-capable User Interface (UI) shall support the ability for the user to opt-out of only the Imminent Threat, Public Safety and Child Abduction Emergency Warning Notifications. A CMAS-capable User Interface (UI) shall support the ability for the user to opt-in to the State/Local WEA Test notification.

If the User has opted out of a Warning Notification class, then the Warning Notification for that class shall not be displayed by the UE.

If the User has not opted out of a Warning Notification class, then any Warning Notifications for that class received in English shall be displayed by the UE. This also applies to Warning Notifications received in any additional languages that may be selected by the User.

Warning Notifications may contain URLs and/or phone numbers.

A CMAS-capable User Interface (UI) shall support the ability (e.g., a “click” touch input) for the user to navigate to the URL or initiate a voice call to the phone number which may be included in the Warning Notification [8].

The PLMN broadcasting CMAS Warning Notifications shall also broadcast corresponding CMAS Warning Notification Area(s) when the CMAS Warning Notification is required to adhere to the maximum 0,1 of a mile CMAS Warning Notification Area(s) overshooting as specified in [10]. This requirement applies only to LTE and NR accesses.

A CMAS-capable UE shall utilize UE-Based Location Calculation (see 3GPP TS 22.071 [11]) to determine whether it is located within the corresponding CMAS Warning Notification Area(s) in order to minimize overloading network LCS resources.

Subject to opt-out and opt-in settings described above, a CMAS-capable UE shall present the CMAS Warning Notification sent if there is a corresponding CMAS Warning Notification Area(s) and if the CMAS-capable UE is able to determine that its location is within the corresponding CMAS Warning Notification Area(s). Otherwise, if a CMAS-capable UE is unable to or fails to determine its location, the CMAS-capable UE shall present the Warning Notification to the user.

A CMAS-capable UE shall store presented Warning Notifications for at least 24 hours unless explicitly deleted by the user.

## 7.2	Additional PWS Requirements Specific to EU-ALERT

In addition to the General Requirements specified in Clause 4, the following requirements are specified for the deployment of EU-ALERT. These EU-ALERT specific requirements are further specified in ETSI TS 102 900 [6].

EU-ALERT shall support three types of Warning Notifications: Alert messages to warn citizens of an imminent emergency situation, Advisory messages of lesser urgency, and Amber alerts.

The Alert messages shall be supported with three levels of severity. EU-Alert level 1 shall have no opt-out; levels 2 and 3 shall allow opt-out by the user. All levels of EU-ALERT messages shall be associated with a dedicated alerting indication.

NOTE: 	EU-Alert level 1 is compatible with the Presidential Alert in CMAS. EU-Alert level 2 is compatible with CMAS Extreme Alerts and EU-Alert level 3 is compatible with CMAS Severe Alert.

The Advisory messages have only one level. Advisory messages shall not be associated with any dedicated alerting indication.

Depending on national requirements, Amber alerts may need to be broadcast as part of the EU-ALERT service. Amber alerts shall not be associated with any dedicated alerting indication.

EU-ALERT shall support Warning Notifications in various languages. To support international roaming, it is expected that countries adopting EU-ALERT use the same Message Identifier for Warning Notifications in the local language. If Warning Notifications are broadcast in other languages besides the local language, then the Message Identifier for such Warning Notifications are expected to be the same across the countries adopting EU-ALERT.

EU-ALERT shall be supported on GERAN, UTRAN, E-UTRAN and NG-RAN.

EU-ALERT shall support that the PLMN broadcasting EU-ALERT Warning Notifications also broadcasts corresponding Warning Notification Area(s). This requirement applies only to E-UTRAN and NG-RAN.

A EU-ALERT-capable UE shall utilize UE-Based Location Calculation (see 3GPP TS 22.071 [11]) to determine whether it is located within the corresponding Warning Notification Area(s) in order to minimize overloading network LCS resources.

Subject to opt-out and opt-in settings described above, a EU-ALERT-capable UE shall present the EU-ALERT Warning Notification sent if there is a corresponding Warning Notification Area(s) and if the EU-ALERT-capable UE is able to determine that its location is within the corresponding Warning Notification Area(s). Otherwise, if a EU-ALERT-capable UE is unable to or fails to determine its location, the EU-ALERT-capable UE shall present the EU-ALERT Warning Notification to the user.