3GPP TSG-SA1 Meeting #98e	S1-221132r1

Electronic Meeting, 09 – 19 May 2022	(revision of S1-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0015 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call restriction based on subparts of functional identities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eFRMCS |  |  |  |  |  | Date: |  |  | 2022-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Existing call restriction is based on source and destination FRMCS User identities (e.g., functional identities of FRMCS Users). It is not allowed to deny/permit calls based on subpart of functional identities of FRMCS Users. An example of a full functional identity is a specific driver on a specific train. An example of a subpart of a functional identity is all drivers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed changes allow call restriction based on subparts of functional identities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Call restriction for Railway needs are not fully covered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.8.2.1, 9.8.2.2, 9.8.2.3, 9.8.2.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * New change * * *

### 9.8.2	Use case: Permit / deny communication

#### 9.8.2.1	Description

The purpose of the authorisation function it to allow the network operator to control and regulate communications in order to avoid disruption/distraction to the users (for example Drivers during voice communication) and preventing unauthorised communication and to minimise network load.

The FRMCS System may therefore restrict communication based on a configurable access matrix related to the FRMCS subscriber identity, the FRMCS Functional Identity of the equipment, the FRMCS User Identity, the FRMCS Functional Identity of the FRMCS Users considering the source and destination address.

Decision to permit or deny communication is based on full or subparts of the FRMCS Functional Identity of the FRMCS Users. An example of a full FRMCS Functional Identity of an FRMCS User is a specific driver on a specific train. An example of a subpart of an FRMCS Functional Identity of an FRMCS User is all train drivers.

#### 9.8.2.2	Pre-conditions

An FRMCS User attempts to establish a communication.

#### 9.8.2.3	Service flows

Upon an attempt to establish a communication, the FRMCS System checks if the source FRMCS User address is allowed to invite the destination FRMCS User address. The FRMCS System shall consider for the verification process the source and destination FRMCS User address.

If multiple identities are simultaneously active to a FRMCS User, the following order shall be applied to determine the authorisation of the communication establishment:

- Functional identities or subparts of functional identities related to the FRMCS User

- FRMCS User Identity

- FRMCS functional identities related to the equipment

- FRMCS subscriber identity

If the FRMCS User is allowed to establish the communication, the FRMCS System proceeds the communication setup.

If the FRMCS User is not allowed establish the communication, the FRMCS System shall reject the attempt including the reason of the rejection.

#### 9.8.2.4	Post-conditions

Either the communication has been established or the FRMCS User has been informed about the denial of the call attempt.

#### 9.8.2.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-9.8.2-001] | The FRMCS System shall be able to control and restrict the establishment of communication in order to avoid disruption/distraction of the FRMCS Users (for example Drivers). | A | TS 22.280 | TS 22.280 sub-clause 6.7.3 Requirement [R-6.7.3-007a] |
| [R-9.8.2-002] | The FRMCS System shall be able to restrict communication based on related to the FRMCS subscriber identity, the FRMCS Equipment Identity, the FRMCS Functional Identity of the equipment, the FRMCS User Identity, the FRMCS Functional Identity or subparts of the FRMCS Functional Identity of the FRMCS Users considering the source and destination address. | A | TS 22.280 | TS 22.280 Requirements [R-5.17-001] for group calls and [R-6.7.2-002] for private calls. Covered by [R-5.9a-020] and [R-5.9a-021] for functional identities. Not covered for subparts of functional identities. |
| [R-9.8.2-003] | The FRMCS System shall inform the FRMCS User in case the FRMCS User is not allowed to establish a communication. | A | TS 22.280 | Implicit in many requirements. |

* * * End of change * * *