3GPP TSG-SA WG1 Meeting #98e 	S1-221156r1

Electronic Meeting, 9 – 19 May 2022	(revision of S1-22xxxx)

Source:	KPN

pCR Title:	Pseudo-CR on flower auction traffic scenario for AmbientIoT

Draft Spec:	3GPP TR 22.840

Agenda item:	7.3

Document for:	Approval

Contact:	Toon . Norp @tno .nl

Abstract: This is a traffic scenario for AmbientIoT on flower auction logistics.

1. Proposal

It is proposed to agree to add the following traffic scenario to 3GPP TR 22.840 (all new text)

## 6.X	Traffic scenario Flower Auction

### 6.X.1	Description

In the Netherlands, there is extensive logistics industry for flowers and vegetables. A specific case are the auctions where flowers from all over the world are brought in by the growers, then auctioned and subsequently distributed to buyers all over the world.

Flowers are transported on four wheel containers that can be rented and are that used throughout the logistic chain. These containers are now equipped with a RFID tag. It would be beneficial if AmbientIoT tags could be used. RFID tags are scanned when containers with flowers arrive or leave the auction; tracking and tracing with AmbientIoT could get regular reports from all containers anywhere at the auction.

Figure 6.x.1-1: Logistics at a flower auction

Shipments of flowers can be tracked and traced based on the containers they are on. This is of interest for growers, buyers and the auction. There is also logistics of empty containers, where also the company that owns the containers can benefit from AmbientIoT. Finally the auction has an interest in managing its space. Companies that own or rent containers are charged for leaving containers on the auction grounds overnight.

Communication service availability is important. Tracking and tracing is mainly done to find out where things have gone wrong in the logistics, e.g. missing containers. If the communication from tags is not significantly more reliable than the logistics itself, then tracking and tracing does not provide a benefit.

Some numbers:

-	There are multiple flower auctions in The Netherlands.

-	The size of the flower auction location in Aalsmeer is 1 732 769 m2.

-	44 million flowers are auctioned per day

### 6.X.2	Assumptions

We assume every four wheeled container is equipped with a tag.

Figure 6.x.2-1: Container with flowers (Photo: Container Centralen)

Density of containers can be estimated based on the dimensions of the containers. A container is (lxwxh) 1350 mm x 565 mm x 1900m. Packing these containers closely together gives a density of 740 x 1770 = 1,3 million containers per km2.

Figure 6.x.2-1: Density of flower containers (Photo: Royal FloraHolland)

Ceiling in the flower auction is at 9 meters. We assume that base stations are attached to the ceiling. Number of base stations that is needed to cover the flower auction is dependent on the communication range and on the number of devices per base station. Here we assume a base station spacing of one base station for every 50 m x 50 m of ceiling. This gives a maximum range of approx 35 meters from ceiling to container. The number of containers in that 50 m x 50 m area is approximately 3000.

Assumption is that containers are powered by radio energy. Sufficient radio energy may be provided on specific or regular intervals, or continuously. After receiving power, the device can store enough power for approximately 100 ms. It assumed that when not powered, the devices are in deep sleep (i.e. no processing nor radio transmissions or receptions).

Editor’s Note: It is FFS how to ensure that not all devices respond at the same time.

Assumption is that an identity can be provided within 500 bits.

Assumption is that probability of errors in logistics handling (e.g. a container is left behind) is <1%. A communication service availability of 99,99% would imply that the chance that communication for tracking a container is approximately 2 orders of magnitude better than the logistics handling reliability.

### 6.X.3	Potential Functional Requirements

None identified.

### 6.X.4	Potential Key Performance Requirements

[PR 6.x.4-001] The 5G system shall be able to provide Ambient_IoT service with the following KPIs:

Table 6.x.4-1

| Scenario | Communication Service Availability | Message Size | Data rate per UE | Communication range | Device density | Devices per base station |  | Service area dimension |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Container logistics in a flower auction | 99,99% (note 1) | [500] bits (note 2) | [5] kbit/s (note 3 | [35] m Indoors | < 1,3 Million/km2 (note 4) | <[3000] |  | 1 700 000 m2 (note 5) |
| NOTE 1: Chance of communication service unavailability needs to be significantly lower than chance of errors in logistics handling NOTE 2: Only an identifier for the tag is sent. NOTE 3: This value is calculated as the instant data rate for transmitting 500 bits within 100 ms time period. The need for data transmission is infrequent. NOTE 4: Based on closely packing containers. NOTE 5: Size of the flower auction location in Aalsmeer. |  |  |  |  |  |  |  |  |