3GPP TSG-SA WG1 Meeting #98e 	S1-221174

Electronic Meeting, 9 – 19 May 2022	(revision of S1-22xxxx)

Source:	Hansung University, LGUplus, KT, ETRI

pCR Title:	Pseudo-CR on a use case of smart kiosk of railway smart station

Draft Spec:	3GPP TR 22.890 v 0.5.0

Agenda item:	7.1

Document for:	Approval

Contact:	Andrew Min-gyu Han, andyhan@hansung.ac.kr

Abstract: A new use case is proposed to support passengers by operating the smart kiosk in railway smart station. The smart kiosk provides various information to passengers, such as location information service with 3D or metaverse enabled station map, simple ticketing service, and other information providing services via interfacing smart station devices and systems, e.g. CCTV, sensors.

1. Introduction

This pCR suggests a use case of smart kiosk of railway smart station for TR22.890.

2. Reason for Change

A use case should be provided to enable the smart kiosk service for railway smart station.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.890.

* * * First Change * * * *

7.x	Smart kiosk of railway smart station

7.x.1	Description

A smart kiosk in the railway smart station provides various information to passengers, such as location information service with 3D or metaverse enabled station map, simple ticketing service, and other information providing services via interfacing smart station devices and systems, e.g. CCTV, sensors.

The kiosk co-operates with a moving object such as robot, to support passengers if the kiosk operating system decides that it is necessary.

7.x.2	Pre-conditions

A passenger has ticketed of a train.

A passenger has a UE which has a smart station service enabled.

A passenger has made a permission to handle his/her identification to FRMCS, such as the smart station system.

7.x.3	Service Flows

1. A passenger is standing in front of a smart kiosk to search his train’s platform.

2. The passenger’s UE make a proximity connection with the kiosk or vice versa and send identification information on the passenger or the train.

3. The kiosk shows the path to the platform of the train with 3D or metaverse enabled map of the smart station and transmits the information to the passenger’s UE.

4. The passenger gets physical ticket or receipt from the kiosk and the passenger is a transport vulnerable, asks guidance to the kiosk by the moving object, a.k.a. guidance robot of the smart station.

5. The kiosk asks the location of the moving object to the FRMCS and arranges the closest one to the passenger and bring the moving object the passenger, for example, in front of the kiosk.

6. The moving object comes to the passenger and confirms the identification information of the passenger by connecting the UE of the passenger.

7. The passenger gets on the moving object and moves to the platform.

7.x.4	Post-conditions

The passenger gets on the train and enjoys his/her journey.

The moving object is released from the passenger service and ready to organise other service.

7.x.5	Existing features partly or fully covering the use case functionality

Not applicable.

7.x.6	Potential New Requirements needed to support the use case

[R-6.x-1] The 5G System shall support the proximity connections between the UE, the kiosk and the moving object. The connections should support at least 10m as the range of communication.

[R-6.x-2] The 5G System shall be able to provide a function to handle identification information which is used by the kiosk service.

[R-6.x-3] The 5G System shall be able to find a moving object location which is belongs to the moving object group.

[R-6.x-4] The 5G System shall be able to provide functions to handle the smart station information for making display 3D or metaverse enabled smart station map and generating path information by a kiosk.