| 3GPP TSG-S1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc 3GPP |  |  |  |  |  |  |  |  | S1-000395 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| City, Country, DD-DD MMM YYYY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 22.078 |  |  |  |  | CR |  |  |  | 042 |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | 3.3.0 |  |  |  |
| 3G specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by 3G support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to TSG |  |  |  |  | SA #8 |  |  |  | for approval |  |  |  |  |  | X |  |  | (only one box should |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| List TSG meeting no. here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  | be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: 3G CR cover sheet, version 1.0 The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-xx.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | USIM |  |  |  |  | ME |  |  |  |  |  |  |  |  |  | UTRAN |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | CAMEL ad hoc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  | 09/05/2000 |  |  |  |  |
| Subject: |  |  | Corrections to Interactions with Supplementary Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G Work item: |  |  | CAMEL Phase 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  | Phase 2 |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  | Release 99 |  |  | x |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |
| Reason for change: |  |  | Section 18 does not fully cover the interactions between CAMEL Phase 3 and Supplementary Services. There is no indication that some sections only apply to CAMEL Phase 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  | 18.2.2, 18.2.4, 18.4.1, 18.4.2, 18.5, 18.6, 18.9, 18.10, 18.xx (new). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other 2G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- Double-click here for help and instructions on how to create a CR.

****First Modified Section ****

### 18.2.2	Calling Line Identification Restriction (CLIR)

For an MT call, the CSE is not able to change the presentation indicator given to the called subscriber via the CLIP supplementary service.

$(begin$(CAMEL3$)

For an MO/MF call, the CSE shall be able to send to the VPLMN/IPLMN an instruction that the presentation indicator of the calling party number shall be set to ”presentation restricted”.

$(end$(CAMEL3$)

****Next Modified Section ****

### 18.2.4	Connected Line Identification Restriction (COLR)

$(begin$(CAMEL3$)

For an MT call, the CSE shall be able to send to the VPLMN an indication that the presentation indicator of the connected number shall be set to “presentation restricted”.

$(end$(CAMEL3$)

****Next Modified Section ****

### 18.4.1	Call Hold (CH)

For both originating and terminating calls, the Call Hold service is invoked after the CAMEL feature is invoked. A call created when a call has been put on hold may be subject to the CAMEL feature in the same way as a normal mobile originating call.

$(begin$(CAMEL3$)

When a call is established, the CSE shall be able to instruct the VPLMN of the served subscriber whether to prohibit Call Hold.

$(end$(CAMEL3$)

****Next Modified Section ****

### 18.4.2	Call Waiting (CW)

Incoming, waiting calls are treated by the CSE as any other mobile terminating calls which encounter an idle subscriber.

$(begin$(CAMEL3$)

When a call is established the CSE shall be able to instruct the VPLMN of the served subscriber whether to prohibit Call Waiting.

$(end$(CAMEL3$)

****Next Modified Section ****

## 18.5	Multi Party (MPTY)

A multi party call may include one or more call legs subject to CAMEL based services.

$(begin$(CAMEL3$)

If a call leg is subject to CAMEL based services the CSE shall be able to instruct the VPLMN of the served subscriber whether to prohibit the inclusion of that leg in a multi party call.

$(end$(CAMEL3$)

****Next Modified Section ****

## 18.6	Closed User Group (CUG)

The CSE shall decide whether to invoke the CUG supplementary service and shall perform the necessary processing for a Mobile Originated, Mobile Terminated or Forwarded call. When a terminating call with CUG information is received for a CAMEL marked subscriber, if the terminating CAMEL based service attempts to modify the called party number then:

-	if the called subscriber subscribes to CUG then the IPLMN shall release the call towards the calling party;

-	if the called subscriber does not subscribe to CUG then the IPLMN shall continue the call establishment towards the destination defined by the modified called party number.

$(begin$(CAMEL3$)

For an MO call, an MF call, or an MT call subject to CAMEL forwarding, the CSE shall be able to instruct the VPLMN/IPLMN to:

- continue the call establishment with the original CUG information, or

- use modified CUG information for that call, or

- remove CUG information from the call (i.e. continue the call as a non-CUG call).

For an MT call which is not subject to CAMEL forwarding, the CSE shall not be able to modify the CUG information for the call.

When a terminating call with CUG information is received for a CAMEL marked subscriber, if the terminating CAMEL based service attempts to modify the called party number then:

-	if the called subscriber subscribes to CUG then the VPLMN shall release the call towards the calling party;

-	if the called subscriber does not subscribe to CUG then the VPLMN shall continue the call establishment towards the destination defined by the modified called party number.

$(end$(CAMEL3$)

****Next Modified Section ****

## 18.9	Explicit Call Transfer (ECT)

An ECT call may include one or both call legs subject to CAMEL based services.

$(begin$(CAMEL3$)

If a call leg is subject to CAMEL based services the CSE shall be able to instruct the VPLMN of the served subscriber whether to prohibit the inclusion of that leg in an explicitly transferred call.

$(end$(CAMEL3$)

****Next Modified Section ****

## 18.10	Completion of Call to Busy Subscriber (CCBS)

$(begin$(CAMEL3$)

When a call is established the CSE shall be able to instruct the VPLMN/IPLMN whether Subscriber A is prohibited from activating a CCBS request if a subsequent “CCBS possible” indication is received from the destination network or the terminating served subscriber is busy.

$(end$(CAMEL3$)

****New Section ****

## 18.xx	Call Deflection

$(begin$(CAMEL3$)

When an MT call is established the CSE shall be able to instruct the VPLMN of the served subscriber whether to prohibit Call Deflection.

$(end$(CAMEL3$)

****End of document ****