| 3GPP SA1 CAMEL adHoc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  | S1-000408 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sophia Antipolis, France, 9-11 May 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 22.078 |  |  |  |  |  | CR |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | 4.x.x |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  |  |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  | x |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Nokia |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  | 9 May 2000 |  |  |  |  |  |
| Subject: |  |  | CAMEL4 & IPT services; address information & registration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | CAMEL Phase 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  | Release: |  |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  | x |  |  |
| Reason for change: |  |  | The IP telephony services will require other address formats as E.164 telephone numbers. This CR presents those changes to CAMEL phase 4 stage 1. The other addressing format will be the alias address, i.e. so called Logical Name (LN) in the format of First.Last@company.country. The changes are: - LN is conveyed in the CSCF and SCP interface. - LN can be a triggering criterion, but only the host part of the address on the level of a "label". - CSE shall be aware of Registration and de-registration of a IPT phone to a CSCF. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  | Since the S2 has not yet decided on the location of the CSCF this CR does not make any statements/changes about the VPLMN or HPLMN. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

*** First Modified Section ***

# 1	Scope

This standard specifies the stage 1 description for CAMEL feature (Customised Applications for Mobile network Enhanced Logic) which provides the mechanisms to support services consistently independently of the serving network. The CAMEL features shall facilitate service control of operator specific services external from the serving PLMN. The CAMEL feature is a network feature and not a supplementary service. It is a tool to help the network operator to provide the subscribers with the operator specific services even when roaming outside the HPLMN.

CAMEL is developed in phases. The following phases exist:

-	CAMEL phase 1. This is the default phase in this specification. Text that are only applicable to phase 1 are characterised with the formal designator - $(CAMEL1$)

-	CAMEL phase 2. It is characterised where necessary with the formal designation - $(CAMEL2$) and sometimes with an indication of CAMEL phase 2. - $(CAMEL2$)

-	CAMEL phase 3. It is characterised where necessary with the formal designation - $(CAMEL3$) and sometimes with an indication of CAMEL phase 2. - $(CAMEL3$)

- CAMEL phase 4. It is characterised where necessary with the formal designation - $(CAMEL4$)

The CAMEL feature is applicable

-	to mobile originated and mobile terminated call related activities;

-	as a CAMEL phase 2 function, to supplementary service invocations - $(CAMEL2$);

- as a CAMEL Phase 3 function, to SMS MO, to GPRS sessions, to the control of HLR subscriber data- $(CAMEL3$);

- as a CAMEL Phase 4 function to control different call party connections and, $(CAMEL4$)

-	as a CAMEL Phase 4 function, to CSE created calls - $(CAMEL4$).

The mechanism described addresses especially the need for information exchange among the VPLMN, HPLMN and the CAMEL Service Environment (CSE) for support of such operator specific services. Any user procedures for operator specific services are outside the scope of this standard.

This specification describes the interactions between the functions of the VPLMN, HPLMN, IPLMN and the CSE.

The second phase of CAMEL enhance the capabilities of phase 1 and are included in this standard. Following capabilities are added:

-	Additional event detection points.

-	Interaction between a user and service using announcements, voice prompting and information collection via in band interaction or USSD interaction.

-	Control of call duration and transfer of Advice of Charge Information to the mobile station.

-	The CSE can be informed about the invocation of supplementary services (e.g ECT, CD, MPTY).

-	For easy postprocessing, charging information from a serving node can be integrated in normal call records.

The third phase of CAMEL enhances the capabilities of phase 2. Following capabilities are added :

-	Capabilities for enhanced handling of call party connections-	Support of facilities to avoid overload situations.

-	Capabilities to support Dialled Services.

-	Capabilities to handle mobility events, such as (Not-)reachability and roaming.

-	Control of GPRS sessions.

-	Control of circuit switched mobile originating  SMS and packet switched mobile originating SMS.

-	Support of SoLSA. Support of Localised Service Area interworking is an optional feature. The support for interworking with mobile terminating calls is a requirement, however the specific details of how this will be realised is for further study - $(CAMEL3$)

-	The CSE can be informed about the invocation of GSM supplementary services (CCBS) - $(CAMEL3$)

The fourth phase of CAMEL enhances the capabilities of phase 3. The following capabilities are added

- Capabilities for enhanced handling of call party connections, together with the ability to handle more than 2 parties in a call.

- Control of PS switched services, especially IP telephony services.

Detailed information can be found in the respective sections.

*** Next Modified Section ***

# 3	Definitions and abbreviations

Operator Specific Service (OSS): Any service offered to a mobile user that is not standardised.

Interrogating PLMN (IPLMN): This is the PLMN that performs the interrogation of the HPLMN for information on the treatment of a terminating call.

CAMEL Service Environment (CSE): A CSE is a logical entity which processes activities related to Operator Specific Services (OSS).

Route select failure: A condition when routeing to the called party fails. Route Select Failure can be reported in an existing relationship $(CAMEL2$) or a new relationship can be initiated.- $(CAMEL3$)

Service event: A specific event of a process that may be used as part of an operator specific service.

Service procedure: A part of the CAMEL feature to be used to detect a specific CAMEL service event.

Network CAMEL Service Information (N-CSI): The N-CSI identifies services offered on a per-network basis by the serving PLMN operator for all subscribers. - $(CAMEL3$)

NOTE: 	These services may also be provided using a technology other than CAMEL.

CAMEL Subscription Information (CSI): The CSI identifies that CAMEL support is required for the subscriber and the identities of the CSEs to be used for that support. The CSI also contains information related to the OSS of the subscriber, e.g. Service Key.

These OSS may include both services provisioned for individual subscribers and services provisioned equally for all subscribers of an HPLMN. - $(CAMEL3$)

Location Information: The location information shall be an identification of the location of the served subscriber.

The following location information should be sent to the CSE (if available):

-	Geographical information (longitude and latitude)when Cell ID or Location Area Code is known) this may be calculated as the nominal central point of the cell or of the location area; alternative mechanisms for determining latitude and longitude may also be supported. The resolution and accuracy of the indicated location information may also be provided.

-	Cell ID indicates the global identity of the current or last cell which the subscriber is using or has used. The VPLMN shall update the stored Cell ID at establishment of every radio connection and whenever the subscriber is handed over between cells.

-	VLR number is the number of the serving VLR stored in the HPLMN.

-	Location status indicates whether or not the location information has been confirmed by radio contact. If the location information has not been confirmed by radio contact a time stamp is sent indicating the time elapsed since the last radio contact with the subscriber.

-	Location number is the number received on the incoming circuit (for an incoming call) or to be sent on the outgoing circuit (for an outgoing call).

Logical Name (LN): An alias internet address, typically in the format of FirstName.LastName@company.country.

Domain: The address part in a logical name after the @ sign.

Label: Each string in the logical names' domain that is between dots is one label. E.g. in a address First.Last@zzz.xxx.yyy the labels are xxx, yyy and zzz.

Service Key: An identifier of the OSS which shall be transparent to the IPLMN/VPLMN.

Subscriber Status: An indication of the status of a subscriber, determined by the state of the subscriber’s MS. The subscriber status can take one of three values:

-	CAMEL-busy: the MS is engaged in a mobile-originated or mobile-terminated circuit-switched call.

-	Network determined not reachable: the network can determine from its internal data that the MS is not reachable. This includes detached and purged mobile stations.

-	Assumed idle: any MS that is not CAMEL-busy or network determined not reachable.

GPRS attach: Is the period where the GPRS subscriber is registered to the GPRS data network. - $(CAMEL3$)

GPRS session: A session starts when a GPRS subscriber activates the start to send or receive data packets and ends when the subscriber deactivates the PDP transport. - $(CAMEL3$)

PDP: Packet Data Protocol (as defined in TS 22.060 [6]) - $(CAMEL3$)

Route select failure: - $(CAMEL3$)

[test to be added]

Carrier Identification Code: Identifies uniquely the Carrier (NAEA). - $(CAMEL2$)

Carrier Selection Information: Is an indication whether the subscriber selected a carrier, or the carrier is predefined for the subscriber (NAEA). - $(CAMEL2$)

Originating Line Identification: Identifies uniquely the subscriber to be charged for the usage of the carrier (NAEA). - $(CAMEL2$)

Charge Number: Identifies uniquely the organisation to be charged for the usage of the carrier (NAEA). - $(CAMEL2$)

NAEA: North American Equal Access (NAEA). - $(CAMEL2$)

Subscribed Dialled Services: Identifies a set of maximum of ten service numbers. These service numbers are globally available to all HPLMN subscribers. No subscription is needed to access the service numbers within the HPLMN. Each service number is chosen on the HPLMN operators discretion. In the case of international roaming, the set of service numbers forms a part of the subscribers profile. - $(CAMEL3$)

*** Next Modified Section ***

# 4	Description

The CAMEL network feature enables the use of Operator Specific Services (OSS) by a subscriber even when roaming outside the HPLMN.

## 4.2	General Procedures

Each process is made up of a series of telecommunication events, some of which are service events. At a service event, the IPLMN or VPLMN may suspend the process and make contact with a CSE to ask for instructions or to send a notification. When a service event occurs, the IPLMN or VPLMN shall send to the CSE the information listed in this specification. All information sent to the CSE relates to the served CAMEL subscriber unless otherwise stated. The initial service events, and the corresponding CSE identity, which can initiate contact with the CSE are defined in the CAMEL Subscription Information.

The serving network shall accept the instruction from the CSE and continue call processing with the received information.

The CAMEL feature is applicable in a PLMN when the CAMEL subscription information is handled properly and when the communication to the CSE is compliant with the CAMEL protocol [8].

- The CAMEL network capabilities are used at a PLMN when the CAMEL feature is applicable and:

-	the CSI is received from the HPLMN; or

-	the CSE requests congestion control in the VPLMN or IPLMN; or - $(CAMEL3$);

-	the CSE creates a call - $(CAMEL4$).

In addition dialled network-based services may be applicable in a PLMN if so administered. -$(CAMEL3$)

The CSE shall be capable of responding to the CAMEL request with instructions on how to resume the suspended process. In the case of subscriber-based services the CSE shall be possible to instruct the IPLMN or VPLMN to:

-	Activate further service events for potential invocation. These events shall remain active only for the life-time of the telecommunication service;

-	Alter information relating to the suspended process;

-	Alter information relating to the parties involved in the process;

-	Indicate which of the possible parts of the process should occur next (e.g. terminate the call);

-	Perform Charging activities -$(CAMEL2$);

-	Order in band user interaction -$(CAMEL2$);

-	 Create additional parties in the call - $(CAMEL4$);

-	 Change the configuration of the connections between call parties - $(CAMEL4$).

$(begin$(CAMEL3$)

It shall be possible in the case of subscribed dialled services for the CSE to instruct the  serving PLMN to:

-	Continue the processing of the call, or;

-	Connect the calling party to a specified called party;

- After one of the above instructions, the relation between the serving network and the CSE shall be released. Any other behaviour may cause misoperation of CAMEL based services.

Serving network-based service numbers may be treated after the above described behaviour. These services are outside the scope of the CAMEL specification.

Serving network based service numbers may be provided on the discretion of the network operator but the are outside this specification.

$(end$(CAMEL3$)

CAMEL features shall form an integral part of the following processes:

- Mobile Originated call (MO call);

- Mobile Terminated call in GMSC (MT call);

- Mobile Terminated call in VMSC (MT call); $(CAMEL3$);

- Mobile Forwarded call (MF call) - early call forwarding; early forwarded calls are treated as MO calls;

- Mobile Forwarded call (MF call) - late call forwarding; late forwarded calls are treated as MO calls;

-	supplementary service invocation -$(CAMEL2$);

-	USSD user interaction. The of service codes for CAMEL services can be allocated on subscriber basis or globally for all subscribers of the HPLMN. -$(CAMEL2$);

- MO SM service; both via MSC and SGSN of GPRS $(CAMEL3$);

- GPRS $(CAMEL3$).

- Mobility Management events $(CAMEL3$)

- Subscriber registration and de-registration in the PS networks  $(CAMEL4$)

- IP telephony (IPT) services $(CAMEL4$)

The CSE shall be able to interrogate the HPLMN for information about a particular subscriber at any time.

It shall be possible for the CSE to originate calls on behalf of a CAMEL subscriber - $(CAMEL3$).

## 4.3	Applicability of CAMEL Procedures

CAMEL procedures are applicable to all circuit switched Basic Services without distinction (except Emergency calls).

CAMEL procedures are applicable to all packet switched  IPT Services without distinction (except Emergency calls). $(CAMEL4$)

- CAMEL procedures are applicable to GPRS sessions $(CAMEL3$).

- CAMEL procedures are applicable to circuit switched Mobile Originating Short Message Service and to packet switched Mobile Originating Short Message Service $(CAMEL3$).

# 5	Procedures for Mobile Originated Calls and Forwarded Calls

NOTE: 	Other information elements not listed in the following subclauses may be necessary to meet some Stage 1 service requirements. Refer to the Stage 2 specification TS 23.078 for complete information element lists.

## 5.1	Initial service events

It shall be possible to specify which of the following initial service events shall initiate contact with the CSE:

-	Collection of dialled digits or Logical Name (LN) $(CAMEL4$);

$(begin$(CAMEL3$)

-	Analysis of dialled digits  $(CAMEL3$) or Logical Name (LN) $(CAMEL4$);

-	Detection of unsuccessful call establishment - $(CAMEL3$).
Unsuccessful call establishment may be caused by:

-	route select failure.

$(end$(CAMEL3$)

The definition of which of the above initial service events shall initiate contact with the CSE is part of the subscriber’s CAMEL subscription information. Analysis of dialled digits can open a new dialogue regardless whether there exists a relationship or not. Upon detection of unsuccessful call establishment no new relationship is opened if there is already a dialogue open due to same CSI.

Type of number (TON): Identifies the format a number for an E.164 telephone number.

## 5.2	Criteria for contact with the CSE $(CAMEL2$)

It shall be possible for the HPLMN to specify criteria which must be satisfied before the CSE is contacted.

The following criteria may be defined:

### 5.2.1	CSI criteria applicable at call setup

#### 5.2.1.1	CSI criteria applicable at call setup when dialled digits or Logical Name $(CAMEL4$) have been collected $(CAMEL2$)

CSI criteria may be defined for a subscriber for the case where collection of dialled digits or Logical Name (LN) $(CAMEL4$) has been performed $(CAMEL3$).

Criteria on the dialled number; these consist of:

-	The contents of the dialled number (a list of up to 10 dialled number strings may be defined in the criteria. Each dialled number string may be of any type of number (TON) format. supported by the access protocol);

-	The length of the dialled number (a list of up to three lengths may be defined.).

-	The criteria on the dialled number may be collectively defined to be either "enabling" triggering criteria or "inhibiting" triggering criteria (see below). The HPLMN may also choose not to define any criteria on the dialled number.

Criteria on the logical name; these consist of $(CAMEL4$):

-	The contents of the logical (a list of up to 10 logical names may be defined in the criteria. Each dialled number string may consist of one to five labels);

-	The criteria on the logical name may be collectively defined to be either "enabling" triggering criteria or "inhibiting" triggering criteria (see below). The HPLMN may also choose not to define any criteria on the logical name.

-	A criterion on the basic service: this consists of a list of basic service codes for individual basic services or basic service groups (the list shall be able to contain at least 5 basic service codes). The HPLMN may also choose not to define any criterion on the basic service.

-	A criterion on the type of call: this consists of defining whether or not the call must be a forwarded call.

A call is treated as forwarded in this respect when either a forwarding supplementary service applies or when the call is forwarded as a result of a terminating CAMEL based service. The HPLMN may also choose not to define any criterion on the type of call.

If the criteria on dialled number are "enabling" then the dialled number criteria are satisfied if:

-	the dialled number matches a dialled number string defined in the criteria; or

-	the length of the dialled number matches a dialled number length defined in the criteria.

If the criteria on the dialled number are "inhibiting" then the dialled number criteria are satisfied if:

-	the dialled number does not match any of the dialled number strings defined in the critera; and

-	the length of the dialled number is not the same as any dialled number length defined in the criteria.

In these tests the dialled number matches one of the dialled number strings if:

-	the two numbers are of the same Type Of Number (TON); and

-	the dialled number is at least as long as the dialled number string in the criteria; and

-	all the digits in the dialled number string in the criteria match the leading digits of the dialled number.

If no criterion on the dialled number is specified then the dialled number criteria are satisfied.

If the criteria on logical name (LN) are "enabling" then the logical name criteria are satisfied if:

-	the dialled/ logical name matches logical name string defined in the criteria;

If the criteria on the logical name are "inhibiting" then the logical name criteria are satisfied if:

- the dialled/typed logical name does not match any of the logical name strings defined in the criteria;

In these tests the logical name criterion matches one of the dialled/typed logical name strings if:

- each label matches exactly. The labels are compared in the specified order. First the most significant label is compared. If there are more labels in the dialled/typed address than the in the criterion, then the addresses match if all the given labels in the criterion have matched.

If no criterion on the logical name is specified then the logical name criteria are satisfied.

If the dialled address is a E.164 number then it is compared to the dialled number criteria, and the logical name criteria are satisfied. If the dialled/typed address is a logical name, then the dialled/typed address is compared against the logical name criteria, and the dialled number criteria are satisfied.

The criterion on the basic service is satisfied if the basic service used for the call corresponds to any basic service code defined in the criterion or if no basic service criterion is specified.

The criterion on the type of call is satisfied if the type of the call is the same as the type defined in the criterion or if no call type criterion is specified.

The criteria on the call setup event procedure are satisfied if:

- the criteria on the dialled number are satisfied; and

- the criteria on the logical name are satisfied; and

-	the criterion on the basic service is satisfied; and

-	the criterion on the type of call is satisfied.

#### 5.2.1.2	CSI criterion applicable at call setup for subscribed dialled services $(CAMEL3S)

A CSI criterion on the contents of the called number/address $(CAMEL4$) shall be defined for subscribed dialled services. A list of up to 10 called number strings and a list of 10 logical names may be defined in the criterion. Each entry in the called number/address list has associated with it a CSE identity and a service key which defines the service to be triggered if the criterion is satisfied.

The called number/address criterion is satisfied if the called number matches a called number string defined in the criterion.

In this test the called E.164 number matches one of the called number strings if:

-	the two numbers are of the same Type Of Number (TON); and

-	the called number is at least as long as the called number string in the criteria; and

-	all the digits in the called number string in the criteria match the leading digits of the called number.

For the logical name criteria the principles of section 5.2 apply. If the dialled address is a E.164 number then it is compared to the dialled number criteria, and the logical name criteria are satisfied. If the dialled/typed address is a logical name, then the dialled/typed address is compared against the logical name criteria, and the dialled number criteria are satisfied.

#### 5.2.1.3	CSI criterion applicable on detection of unsuccessful call establishment $(CAMEL3$)

A criterion on the release cause may be defined. This consists of a list of up to 5 cause values. The criterion on the release cause is satisfied if the received call release cause corresponds to any cause value defined in the list or if no criterion is defined.

*** Next Modified Section ***

# 7 	Procedures for serving network dialled services $(CAMEL3$)

CR editor's note: The applicability to the IPT depends on the TSG SA WG2 decision on the CSCF location. Thus this section may need a revisit.

The purpose of this procedure is to detect a  match between the called party number and a stored network service number at the call set-up request. It is to allow the CSE to modify the handling of the call set-up request. If this procedure is triggered it shall happen after processing of Subscribed Dialled Services triggered via the CSI. If any other CAMEL dialogue has changed called party number then the modified called party number is compared to triggering criteria.

# 12	Notifications of non-traffic events to the CSE  $(CAMEL3$)

## 12.1	Mobility management $(CAMEL3$)

It shall be possible to mark for a subscriber that a notification shall be sent to the CSE when the VPLMN has completed the processing of any of the following mobility events:

-	Location update of MS. It shall be possible to define the following criteria:

-	Location update to a different VLR service area;

-	Location update within the same VLR service area;

-	MS-initiated detach (MS switched off);

-	Network initiated detach (periodic location update of MS failed);

-	Attach of MS (MS switched on, successful location update after network initiated detach);

The notification shall contain the following information if available:

-	Event met;

- Service Key;

-	IMSI;

-	Basic MSISDN;

-	Location information;

-	LSA identity;

-	CAMEL phase supported at the VPLMN.

## 12.x	IPT registration/de-registration $(CAMEL4$)

It shall be possible to mark for a subscriber that a notification shall be sent to the CSE when the CSCF has completed the processing of any of the following mobility events:

-	Registration of MS. It shall be possible to define the following criteria:

-	<to be defined>;

-	de-registration;

The notification shall contain the following information if available:

-	Event met;

- Service Key;

-	subscriber identity of the access network;

-	subscriber address (e.g. telephone number);

-	CSCF address;

-	CAMEL phase supported at the CSCF.

## 12.2	Notification to CSE of change of subscriber data $(CAMEL3$)

It shall be possible to mark for a subscriber that a notification shall be sent to the CSE when any of the following subscriber data are changed as a result of a request from any entity except the CSE to which the notification shall be sent:

-	CF SS data

-	CB SS data;

-	ODB data;

-	CAMEL subscription information

One ore more CSEs may be defined to which the notification shall be sent.

## 12.3	Supplementary service invocation notification to CSE $(CAMEL2$)

It shall be possible to mark for a subscriber that a notification shall be sent to the CSE when any of the following supplementary services are invoked:

-	ECT

-	CD

-	MPTY

- CCBS - $(CAMEL3$)

*** Next Modified Section ***

# 18	Interactions with supplementary services

## 18.1	General

This subclause defines the interaction between supplementary services and the CAMEL feature. However, it should be noted that the most effective way to control those service interactions is through managing the provisioning of services. Where possible, subscribers provisioned with services using the CAMEL feature shall not be provisioned with services having an adverse interaction with the CAMEL based services. supplementary services shall not have any knowledge of CAMEL based services.

In general, call independent supplementary service operations (registration, erasure, activation, deactivation and interrogation) are not modified by CAMEL. The exceptions to this for CAMEL phase 2 are the call forwarding services, described in subclause 12.3.1.

## 18.2	Line Identification

### 18.2.1	Calling Line Identification Presentation (CLIP)

The CSE shall be able to create or modify an additional calling line identity (additional calling party number) and/or logical name which is presented to the called subscriber via the CLIP supplementary service. There shall be no restriction to the format of the additional calling line identity determined by the CSE.

For an MT call, the CSE shall not be able to modify the calling line identity (calling party number).

For an MO call, the CSE shall be able to send to the VPLMN-IPLMN an indication that the presentation indicator shall be set to Withheld in the calling party number parameter. However, the CSE shall not be able to change the actual number contained in the calling party number parameter. For an MO IPT call, the CSE shall be able to send to the VPLMN-IPLMN an indication that the calling party logical name shall be hidden $(CAMEL4$).

### 18.2.2	Calling Line Identification Restriction (CLIR)

No interaction. The CSE is not able to change the presentation indicator given to the called subscriber via the CLIP supplementary service.

### 18.2.3	Connected Line Identification Presentation (COLP)

No interaction. The CSE is not able to change the connected line identity.

### 18.2.4	Connected Line Identification Restriction (COLR)

No interaction. The CSE is not able to change the presentation indicator given to the calling subscriber via the COLP supplementary service.

## 18.3	Call Forwarding

$(begin$(CAMEL2$)

Call Forwarding supplementary service is not applicable for IP telephony. $(CAMEL4$)

For the registration of call forwarding supplementary services the network shall accept any forwarded to number for a subscriber provided with a TIF-CSI. In this case the HPLMN shall treat the forwarded-to number transparently at the time of registration, i.e. it shall not perform validity checks or translations of the format of the number. The forwarding PLMN shall treat the forwarded-to number transparently when the call forwarding service is invoked. The CSE may modify the forwarded-to number within the MO CAMEL Service provided for the subscriber when the call forwarding service is invoked.

NOTE:	Network operators should ensure that the TIF-CSI is provided only to subscribers who are provided with an MO CAMEL service which is capable of translating the registered forwarded-to number.

If the forwarding PLMN does not support CAMEL phase 2, the HPLMN shall consider the call forwarding service as not registered if the forwarded-to number is not stored in international format.

NOTE:	If the served subscriber requires invocation of call forwarding services even when the forwarding PLMN does not support CAMEL phase 2, she has to register a forwarded-to number in E.164 international format.

NOTE:	Network operators should be aware that unpredictable service behaviour could be experienced if the detection points for ‘Busy’, ‘Not Reachable’ or ‘No Answer’ are armed when the corresponding ‘conditional’ call forwarding supplementary service is active.

$(end$(CAMEL2$)

### 18.3.1	Call Forwarding Unconditional (CFU)

The Call Forwarding Unconditional service will be invoked after any terminating CAMEL based service. Any forwarded call resulting from a Call Forwarding supplementary service may cause invocation of any mobile originated CAMEL based services.

$(begin$(CAMEL3$)

The CSE shall be able to instruct the forwarding PLMN to suppress the normal handling of Call Forwarding Unconditional and replace it by one or other of the following:

-	If Call Forwarding Unconditional is active and operative for the call being processed, the call shall fail, with a reason of Forwarding Violation;

-	The call shall be handled as if Call Forwarding Unconditional was not active and operative.

$(end$(CAMEL3$)

### 18.3.2	Call Forwarding on Busy (CFB)

As for Call Forwarding Unconditional (see subclause 18.3.1).

$(begin$(CAMEL3$)

The CSE shall be able to instruct the forwarding PLMN to suppress the normal handling of Call Forwarding on Busy and to handle the call as if Call Forwarding on Busy was not active and operative.

If no terminating CAMEL service is active then a new relationship is opened to the CSE prior to CF invocation. In order to open new relationship the failure reason criterion shall be checked. In order to check triggering criteria the forwarding PLMN shall derive reason code from call forwarding reason; this derived reason code is compared to the triggering criteria.

$(end$(CAMEL3$)

### 18.3.3	Call Forwarding on No Reply (CFNRy)

As for Call Forwarding on Busy (see subclause 18.3.1).

### 18.3.4	Call Forwarding on Not Reachable (CFNRc)

As for Call Forwarding on Busy (see subclause 18.3.1).

## 18.4	Call Completion

### 18.4.1		Call Hold (CH)

Call Hold supplementary service is not applicable for IP telephony. $(CAMEL4$)

For both originating and terminating calls, the Call Hold service is invoked after the CAMEL feature is invoked. When a call is established the CSE shall instruct the VPLMN of the served subscriber whether Call Hold is possible.A call created when a call has been put on hold may be subject to the CAMEL feature in the same way as a normal mobile originating call.

### 18.4.2	Call Waiting (CW)

Call Waiting supplementary service is not applicable for IP telephony. $(CAMEL4$)

Incoming, waiting calls are treated by the CSE as any other mobile terminating calls which encounter an idle subscriber. When a call is established the CSE shall instruct the VPLMN of the served subscriber whether Call Waiting is possible.

## 18.5	Multi Party (MPTY)

MPTY supplementary service is not applicable for IP telephony. $(CAMEL4$)

A multi party call may include one or more call legs subject to CAMEL based services. If a call leg is subject to CAMEL based services the CSE shall be able to instruct the VPLMN of the served subscriber whether that leg may be included in a multi party call.

## 18.6	Closed User Group (CUG)

CUG supplementary service is not applicable for IP telephony. $(CAMEL4$)

The CSE shall decide whether to invoke the CUG supplementary service and shall perform the necessary processing for a Mobile Originated, Mobile Terminated or Forwarded call.When a terminating call with CUG information is received for a CAMEL marked subscriber and if the terminating CAMEL based service attempts to modify the called party number:

-	the IPLMN shall release the call towards the calling party when the called subscriber subscribes to CUG;

-	the IPLMN shall continue the call establishment towards the modified called party number when the called subscriber does not subscribe to CUG.

## 18.7	Advice of Charge (AoC)

Advice of Charge is not guaranteed to operate correctly for calls subject to CAMEL phase 1 based services. It is recommended that subscribers are not provisioned with Advice of Charge and any CAMEL based service for which there is an adverse interaction.

$(begin$(CAMEL2$)

If CAMEL phase 2 is supported and the phase 2 charging function "CSE controlled e-values" is used, the VPLMN shall use the received e-values from the CSE for the purpose of the AoC supplementary service. Once the VPLMN has received e-values from the CSE, only CSE provided e-values are applicable for this call. The e-values shall only be sent by the VPLMN to the MS if the served subscriber is provided with the AoC supplementary service according to TS 22.086. CAMEL phase 2 allows CSE to modify e-values in MO calls only

$(end$(CAMEL2$)

$(begin$(CAMEL3$)

If CAMEL phase 2 or 3 is supported and the phase 2 or 3 charging function "CSE controlled e-values" is used, the VPLMN/IPLMN shall use the received e-values from the CSE for the purpose of the AoC supplementary service. Once the VPLMN/IPLMN has received e-values from the CSE, only CSE provided e-values are applicable for this call. The e-values shall only be sent by the VPLMN/IPLMN to the MS if the served subscriber is provided with the AoC supplementary service according to GSM 02.86.  CAMEL phase 3 allows CSE to modify e-values in MO and MT calls.

$(end$(CAMEL3$)

## 18.8	Call Barring

$(begin$(CAMEL2$)

NOTE:	CAMEL may be used to establish forwarded-legs and CAMEL based re-routing-legs that violate conditional outgoing call barring and ODB services. Network operators should take care to avoid problems that may arise because of this interaction.

$(end$(CAMEL2$)

Call Barring supplementary service is not applicable for IP telephony. $(CAMEL4$)

### 18.8.1	Barring of all outgoing calls

#### 18.8.1.A	Mobile originated calls

No interaction. The Barring of all outgoing calls supplementary service will be invoked. Thus, originating CAMEL based services will not be invoked.

#### 18.8.1.B	Forwarded Calls

No interaction. If the Barring of all outgoing calls supplementary service is active and operative, it shall prevent the registration or activation of Call Forwarding as specified in TS 22.082.

[Editor's note: this may be TS 22.088 - needs checking]

### 18.8.2	Barring of outgoing international calls

#### 18.8.2.1	Mobile originated calls

No interaction. Any originating CAMEL based services shall be invoked before the Barring of outgoing international calls supplementary service.

$(begin$(CAMEL3$)

The CSE shall be able to instruct the originating VPLMN to suppress the invocation of Barring of outgoing international calls and to handle the call as if Barring of outgoing international calls was not active and operative.

$(end$(CAMEL3$)

#### 18.8.2.2	Forwarded Calls

No interaction. The interaction between call forwarding and call barring is not be modified by CAMEL. This means that the interaction is applied prior to the invocation of call forwarding. When call forwarding is invoked (possibly with originating CAMEL services in the forwarding leg) then the VPLMN or IPLMN shall not apply outgoing call barring services.

$(begin$(CAMEL2$)

If the served subscriber is provided with a TIF-CSI the network shall not perform the interaction of call forwarding services with this barring program, i.e.

-	the registration request is accepted even if this barring program is active and operative;

-	the activation of this barring program is accepted even if a call forwarding supplementary service is active.

When call forwarding is invoked(possibly with originating CAMEL services in the forwarding leg) the VPLMN or IPLMN shall not invoke outgoing call barring services.

$(end$(CAMEL2$)

$(begin$(CAMEL1$)

NOTE:	This behaviour means that CAMEL may be used to establish forwarded-legs that violate conditional outgoing call barring and ODB services. Network operators should take care to avoid problems that may arise because of this interaction.

$(end$(CAMEL1$)

### 18.8.3	Barring of outgoing international calls except those directed to the HPLMN country

As for Barring of outgoing international calls (see subclause 12.8.2).

### 18.8.4	Barring of all incoming calls

[Editors Note: text to be drafted]

No interaction. The Barring of all incoming calls supplementary service shall be invoked. Thus, terminating CAMEL based services will not be invoked.

### 18.8.5	Barring of incoming calls when roaming

Same as Barring of all incoming calls (see subclause 12.8.4).

## 18.9	Explicit Call Transfer (ECT)

ECT supplementary service is not applicable for IP telephony. $(CAMEL4$)

An ECT call may include one or both call legs subject to CAMEL based services. If a call leg is subject to CAMEL based services the CSE shall be able to instruct the VPLMN of the served subscriber whether that leg may be included in an explicitly transferred call.

## 18.10	Completion of Call to Busy Subscriber (CCBS)

When a CCBS request is planted the CSE shall be able to instruct the VPLMN of the served subscriber whether CCBS is possible. CCBS supplementary service is not applicable for IP telephony. $(CAMEL4$)

## 18.11	Multiple Subscriber Profile (MSP)

See TS 22.097 [5].

*** Next Modified Section ***

# 19	Interactions with Operator Determined Barring (ODB)

ODB supplementary service is not applicable for IP telephony. $(CAMEL4$)

## 19.1	Barring of all outgoing calls

Same principle as for subclause 12.8.1.

## 19.2	Barring of all outgoing international calls

Same principle as for subclause 12.8.2.

## 19.3	Barring of all outgoing international calls except those directed to the home PLMN country

Same principle as for subclause 12.8.3.

## 19.4	Barring of outgoing calls when roaming outside the home PLMN country

If the subscriber is outside her home PLMN country the Barring of outgoing calls when roaming outside the home PLMN country service will be invoked. Thus, originating CAMEL based services will not be invoked.

## 19.5	Barring of outgoing premium rate calls

Same principle as for subclause 13.3. The handling will be the same both for Premium rate information and Premium rate entertainment.

## 19.6	Barring of incoming calls

Same principle as for subclause 12.8.4.

## 19.7	Barring of incoming calls when roaming outside the home PLMN country

Same principle as for subclause 12.8.4.

## 19.8	Operator Specific Barring

No interaction. Any originating or terminating CAMEL based services shall be invoked before Operator Specific Barring of type 1,2,3,4. Operator Specific Barring is only applicable when registered in HPLMN.

NOTE:	Operators should be aware of this interaction when defining Operator Specific ODB categories.

## 19.9	Barring of Supplementary Services Management

No interaction.

*** Next Modified Section ***

# 20	Interactions with Optimal Routeing (OR)

OR is not applicable for IP telephony. $(CAMEL4$)

Invocation of OR shall not have any impact of any CAMEL based service.

If OR is applied to a late Call Forward then the interrogating PLMN shall invoke a mobile originated CAMEL based service, if required for the served subscriber.

$(begin$(CAMEL2$)

If OR of a basic mobile-to-mobile call is invoked, mobile originating services based on CAMEL phase 2 which rely on the destination of the MO call leg being determined by the dialled number (in particular, prepayment services) will not necessarily operate correctly.

If OR of late call forwarding is invoked from an IPLMN which is also the forwarding subscriber’s HPLMN, then mobile terminating services based on CAMEL phase 2 which rely on the destination of the leg from the IPLMN being determined by the MSRN (in particular, prepayment services) will not necessarily operate correctly.

$(end$(CAMEL2$)

$(begin$(CAMEL4$)

When VPLMN-A contacts the CSE of the originating subscriber, it shall indicate whether it supports OR. If the CSE of the originating subscriber indicates that the call may be subject to basic OR, VPLMN-A shall act as an IPLMN and interrogate HPLMN-B as specified for SOR.

If a call is subject to basic OR, VPLMN-A shall pass the address defining the ultimate destination of the call (whether VPLMN-B, HPLMN-B or the forwarded-to destination) to the CSE of the originating subscriber.

If a call is subject to OR of late call forwarding from an IPLMN which is also the forwarding subscriber’s HPLMN, then the IPLMN shall pass the forwarded-to number to the CSE which handles mobile terminating CAMEL-based services for the forwarding subscriber.

[Editors Note: 	Interaction with OR LCF R’96 has to be studied and text might be changed.]

$(end$(CAMEL4$)

Specific interaction is described in TS 22.079 [2].

*** Next Modified Section ***

# 22	Location Information ($CAMEL3$)

<The applicability for the IPT is for further study>

The purpose of this procedure is to obtain the current location of a particular subscriber. The resolution of the location information may be based on the current Cell Identity of the subscriber’s location, or may be based on more accurate positioning information.

The CSE may interrogate the HLR in order to obtain a particular subscriber’s current location based on the cell identity. The HRL has the possibility to reject any interrogation from any CSE.

The CSE may interrogate the GMLC in order to obtain a particular subscriber’s current location based accurate geographical information as defined by LCS in reference [8]. The GMLC has the possibility to reject any interrogation from any CSE.

Annex A (normative):
Information Tables

CR editor's note: Parameters like IMSI, IMEI etc may have to be identified to be not applicable for IPT. Redirection information is also FFS.

# A.1	Information provided to the CSE

The following table shows the information that is transferred towards the CSE on various events. The numbers are reflecting the applicable Camel phase (!, 2, 3).

|  | Call set-up request procedure 1 | Call set-up request procedure 2 | Call set-up request procedure 3 | Call set-up request procedure 4 | Unsuccessful call establishment | Unsuccessful call establishment (MT) | Incoming call request procedure | Procedures for serving network dialled services 1 | Procedures for serving network dialled services 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Event met | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| IMSI | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| Calling Party’s Number | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| Calling Party’s Category | 1 | 1 | 3 | 3 | - | 3 | 1 | 3 | 3 |
| Additional Calling Party Number | - | 1 | - | 3 | - | 3 | 1 | - | 3 |
| Calling Party's Logical Name | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Called Party BCD Number | 1 | - | 3 | - | 3 | - | - | 3 | - |
| Called Party Number | - | 1 | - | 3 | - | 3 | 1 | - | 3 |
| Called Party Logical Name | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Original Called Party Number | - | 1 | - | 3*1 | - | 3 | 1 | - | 3*1 |
| Redirecting (Party) Number | - | 1 | - | 3 | - | 3 | 1 | - | 3 |
| Redirection Information | - | 1 | - | 3 | - | 3 | 1 | - | 3 |
| Service Key | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| ISDN Bearer Capability | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| High Layer Compatibility | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| Basic Service Code | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| Call Identification Information | 1 | 1 | 3 | 3 | 3 | - | - | 3 | 3 |
| Location Information of the Calling Subscriber | 1 | - | 3 | - | 3 | - | - | 3 | - |
| Location Number of the Calling Subscriber | - | - | - | - | - | 3 | 1 | - | - |
| Location information of the called subscriber | - | - | - | - | - | 3 | 1 | - | - |
| Subscriber State of the called subscriber | - | - | - | - | - | 3 | 1 | - | - |
| Cause - $(CAMEL3$) | - | - | - | - | 3 | 3 | - | - | - |
| Time and Time Zone Information - $(CAMEL2$) | 2 | 2 | 3 | 3 | 3 | 3 | 2 | 3 | 3 |
| Optimal Routing Indication - $(CAMEL3$) | - | - | ? | ? | - | 3 | - | 3 | 3 |
| Calling Party LSA (if available) $(CAMEL3$) | 3 | - | 3 | - | - | - | - | 3 | - |
| IMEI -$(CAMEL4$) | 4 | - | 4 | - | 4 | - | - | 4 | - |
| Terminal characteristics and capabilities (see MExE, 02.57) -$(CAMEL3$) | 4 | - | 4 | - | 4 | - | - | ? | - |
| MExE classmark (see MExE, 02.57) -$(CAMEL3$) | 4 | - | 4 | - | 4 | - | - | 4 | - |
| MExE / SAT application identifier -$(CAMEL3$) | 4 | - | 4 | - | 4 | - | - | 4 | - |
| MExE / SAT Free formatted information -$(CAMEL3$) | 4 | - | 4 | - | 4 | 3 | - | 4 | - |
| NAEA Carrier Identification Code (CIC) -$(CAMEL3$) | 2 | 2 | 3 | 3 | 3 | 3 | 2 | 3 | 3 |
| NAEA Carrier Selection Information (pre-subscribed or on-demand) -$(CAMEL3$) | 2 | 2 | 3 | 3 | 3 | - | 2 | 3 | 3 |
| CUG Index if received from the calling subscriber | 3 | - | - | - | - | - | - | - | - |
| CUG Interlock Code | - | 3 | - | - | - | - | 3 | - | - |
| CUG Outgoing Access Indicator | - | 3 | - | - | - | - | 3 | - | - |

Table A-1: Information transferred towards the CSE

*1: If any other CAMEL dialogue has modified called party number then the modified number is reported to the CSE of dialled services.

[CR editor’s note: S1 tdoc S1-99674 has no column for MT unsuccessful case. However, it indicates that some elements are supported in CAMEL ph 2 for unsuccessful cases. This fact needs clarification in future meetings.]

# A.2	Information sent by the CSE

The following table shows the information that is sent by the CSE on various events. The numbers are reflecting the applicable Camel phase (1, 2, 3).

|  | Call set-up request procedure 1 | Call set-up request procedure 2 | Unsuccessful call establishment (MO) | Call disconnection procedure (MO) | Creation of called parties | Incoming call request procedure | Unsuccessful call establishment (MT) | Call disconnection procedure (MT) | Procedures for serving network dialled services | CSE initiated call set up |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Called Party Number | 1 | 3 | 2 | 2 | 4 | 1 | 2 | 2 | 3 | 4 |
| Called Party Logical Name | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Calling Party Number | - | - | - | - | 4 | - | - | - | - | 4 |
| Calling Party Logical Name | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Calling Party’s Category | 1 | 3 | 2 | 2 | 4 | 1 | 2 | 2 | 3 | 4 |
| Calling IMSI | - | - | - | - | - | - | - | - | - | 4 |
| ISUP CUG information | - | - | - | - | 4 | - | - | - | - | - |
| Additional Calling Party’s Number | 1 | 3 | 2 | 2 | 4 | 1 | 2 | 2 | 3 | 4 |
| Original Called Party Number | 1 | 3 | 2 | 2 | - | 1 | 2 | 2 | 3 | - |
| Redirection Party Number | 1 | 3 | 2 | 2 | 4 | 1 | 2 | 2 | 3 | - |
| Redirection Information | 1 | 3 | 2 | 2 | 4 | 1 | 2 | 2 | 3 | - |
| Alerting Pattern | - | - | - | - | 4 | 2 | - | - | - | 4 |
| ISDN Access releated Information | - | - | - | - | 4 | - | - | - | - | 4 |
| ISDN Bearer Capability | - | - | - | - | 4 | - | - | - | - | 4 |
| High Layer Compatibility | - | - | - | 3 | - | - | - | - | - | 4 |
| Basic Service Code | - | - | - | 3 | - | - | - | - | - | 4 |
| Called Party to be Created | - | - | - | - | 4 | - | - | - | - | 4 |
| New Call Segment | - | - | - | - | 4 | - | - | - | - | 4 |
| In Service Compatibility Response | - | - | - | - | 4 | - | - | - | - | 4 |
| Service Interaction Indicators Two | - | - | - | - | 4 | - | - | - | - | 4 |
| Location Number | - | - | - | - | 4 | - | - | - | - | 4 |
| Optimal Routing Indication $(CAMEL4$) | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | - | 4 |
| MExE / SAT Free formatted information -$(CAMEL4$) | - | - | - | - | - | 4 | - | - | - | - |
| NAEA Carrier Identification Code (CIC) - $(CAMEL2$) | 2 | 2 | 2 | 2 | 4 | 2 | 2 | 2 | 3 | 4 |
| NAEA Carrier Selection Information (pre-subscribed or on-demand) - $(CAMEL2$) | 2 | 2 | 2 | 2 | 4 | 2 | 2 | 2 | 3 | 4 |
| NAEA Originating Line Identification (OLI) - $(CAMEL2$) | 2 | 2 | 2 | 2 | 4 | 2 | 2 | 2 | 3 | 4 |
| NAEA Charge Number (CN) - $(CAMEL2$) | 2 | 2 | 2 | 2 | 4 | 2 | 2 | 2 | 3 | 4 |
| CSE Address | - | - | - | - | - | - | - | - | - | 4 |
| CUG Interlock Code | 3 | 3 | - | - | - | 3 | - | - | - | - |
| CUG Outgoing Access Indicator | 3 | 3 | - | - | - | 3 | - | - | - | - |
| Service Interaction Indicators | 3 | 3 |  |  | 4 | 3 |  |  |  | 4 |

Table A-2: Information sent by the CSE

# A.3	GPRS Information provided to the CSE

The following table shows the information that is transferred towards the CSE on various GPRS events. The numbers are reflecting the applicable Camel phase (3).

|  | Attach procedure | PDP activation / Session Establishment | PDP activation / Session Establishment Acknowledgement |  |
| --- | --- | --- | --- | --- |
| Event met | 3 | 3 | 3 |  |
| Type of monitoring | - | 3 | 3 |  |
| MSISDN | 3 | 3 | 3 |  |
| IMSI | 3 | 3 | 3 |  |
| Service Key | 3 | 3 | 3 |  |
| Location information at least to the resolution of Routing Area of the attaching subscriber | 3 | 3 | 3 |  |
| Time stamp information | 3 | 3 | 3 |  |
| Time zone information | 3 | 3 | 3 |  |
| GPRS MS Class | 3 | 3 | 3 |  |
| MExE / SAT application identifier | 4 | 4 | 4 |  |
| MExE / SAT Free formatted information | 4 | 4 | 4 |  |
| PDP transport protocol, i.e. IP or X.25 | - | 3 | 3 |  |
| Quality of Service information (subscribed, requested, negotiated) | - | 3 | 3 |  |
| Destination address information | - | 3 | 3 |  |
| GPRS charging correlation ID | - | - | 3 |  |
| Destination address information | - | - | 3 |  |

Table A-3: GPRS Information transferred towards the CSE