| 3GPP TSG-SA1 CAMEL Ad Hoc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| City, Country, DD-DD MMM YYYY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 22.078 |  |  |  |  | CR |  |  |  | xxx |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | 3.4.0 |  |  |  |
| 3G specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by 3G support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to TSG |  |  |  |  |  |  |  |  | for approval |  |  |  |  |  | X |  |  | (only one box should |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| List TSG meeting no. here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  | be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: 3G CR cover sheet, version 1.0 The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-xx.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | USIM |  |  |  |  | ME |  |  |  |  |  |  |  |  |  | UTRAN |  |  |  |  |  |  | Core Network |  |  |  |  |  | X |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Centre For Development Of Telematics , India |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  | 05/09/2000 |  |  |  |  |
| Subject: |  |  | CSE capability to get the charging information from the serving PLMN. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G Work item: |  |  | CAMEL Phase 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  | Phase 2 |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  | Release 97 |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  | X |  |  |
| Reason for change: |  |  | Enable CSE to use the existing PLMN charging information for performing its charge control |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  | 1 , 15.X ( new ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  | X |  |  List of CRs: |  |  |  |  |  |  |  |  | 23.78, 29.78 |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other 2G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  | This feature is required in cases when the CSE does not have the charging information of the served PLMN of the subscriber.For Refrence Please see the Annex B.5 of the ETSI IN CS1 core INAP ( Page 206 ). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- Double-click here for help and instructions on how to create a CR.

****First Modified Section ****

# 1	Scope

This standard specifies the stage 1 description for CAMEL feature (Customised Applications for Mobile network Enhanced Logic) which provides the mechanisms to support services consistently independently of the serving network. The CAMEL features shall facilitate service control of operator specific services external from the serving PLMN. The CAMEL feature is a network feature and not a supplementary service. It is a tool to help the network operator to provide the subscribers with the operator specific services even when roaming outside the HPLMN.

CAMEL is developed in phases. The following phases exist:

-	CAMEL phase 1. This is the default phase in this specification. Text that are only applicable to phase 1 are characterised with the formal designator - $(CAMEL1$)

-	CAMEL phase 2. It is characterised where necessary with the formal designation - $(CAMEL2$) and sometimes with an indication of CAMEL phase 2. - $(CAMEL2$)

-	CAMEL phase 3. It is characterised where necessary with the formal designation - $(CAMEL3$) and sometimes with an indication of CAMEL phase 2. - $(CAMEL3$)

The CAMEL feature is applicable

-	to mobile originated and mobile terminated call related activities;

-	as a CAMEL phase 2 function, to supplementary service invocations - $(CAMEL2$);

-	as a CAMEL Phase 3 function, to SMS MO, to GPRS sessions and PDP contexts, to the control of HLR subscriber data, to the control of network signalling load - $(CAMEL3$).

The mechanism described addresses especially the need for information exchange among the VPLMN, HPLMN and the CAMEL Service Environment (CSE) for support of such operator specific services. Any user procedures for operator specific services are outside the scope of this standard.

This specification describes the interactions between the functions of the VPLMN, HPLMN, IPLMN and the CSE.

The second phase of CAMEL enhance the capabilities of phase 1 and are included in this standard. Following capabilities are added:

-	Additional event detection points.

-	Interaction between a user and service using announcements, voice prompting and information collection via in band interaction or USSD interaction.

-	Control of call duration and transfer of Advice of Charge Information to the mobile station.

-	The CSE can be informed about the invocation of supplementary services (e.g ECT, CD, MPTY).

-	For easy postprocessing, charging information from a serving node can be integrated in normal call records.

The third phase of CAMEL enhances the capabilities of phase 2. Following capabilities are added :

-	Support of facilities to avoid overload situations.

-	Capabilities to support Dialled Services.

-	Capabilities to handle mobility events, such as (Not-)reachability and roaming.

-	Control of GPRS sessions and PDP contexts.

-	Control of circuit switched mobile originating  SMS and packet switched mobile originating SMS.

-	Support of SoLSA. Support of Localised Service Area interworking is an optional feature. - $(CAMEL3$)

-	The CSE can be informed about the invocation of GSM supplementary services (CCBS) - $(CAMEL3$)

$begin$(CAMEL4$)

The fourth phase of CAMEL enhances the capabilities of phase 3. Following capability  is  added :

-     CSE capability to get the charging information from the serving PLMN.

$end$(CAMEL4$)

Detailed information can be found in the respective sections.

****Next Modified Section ****

$begin$(CAMEL4$)

5.X  CSE capability to get the charging information from serving PLMN

In this scenario , the CSE shall be able to ask the VPLMN/IPLMN for the charging information determined at the PLMN..

- The charging events occurring in the PLMN can be

- receipt of charge related signalling information from the higher exchange , or

- determined by self.

There are three options identified to handle this type of interaction .For a call instance controlled by  the CSE , the CSE should be able to select one of these options.

Option 1

In this option, the CSE has the control of the charging information and instructs the  gsmSSF to monitor and intercept the charging processing  in the PLMN. Based on the criteria supplied by the CSE , the gsm SSF  will send the charging information to CSE :

- Immediately on the occurrence of charging event;

- After occurrence of a specified number of events of the appropriate type; or

- At the end of the call.

Subsequently CSE may use this information performing its charging control ( use the received charging information in the call record generation or to adjust new charge parameters to be sent to the SSF for on-line charging.)

Option 2

In this option,  the CSE may decide to leave the charge control to the serving PLMN and monitor  the charging events without interception. .In this case the PLMN shall perform normal charging procesing , e.g. update the charging information in the charge/call record generated in the PLMN, and notify the CSE.  Based on the criteria supplied by the CSE , the SSF  will send this information to CSE :

- Immediately on the occurrence of charging event;

- After occurrence of a specified number of events of the appropriate type; or

- At the end of the call.

Option 3: Same case as for option 2 except that the CSE does not wish to monitor the charging events.

****End of document ****