TSG-SA Working Group 1 (Services) meeting #2	TSGS1#2(99)215

Edinburgh, Scotland 9th-12th March 1999

Agenda Item:	9.4.3

Source:	S1_Charging

Title:	CR to 22.15

Document for:	Approval

| CHANGE REQUEST No : |  |  |  |  | A002 |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 22.15 |  | Version: |  |  | 3.00 |  |  |  |
| Submitted to SMG |  |  | For approval |  |  |  | Without presentation ("non-strategic") |  |  |  |  |  |
| list SMG plenary meeting no. here |  | for information |  |  |  |  | With presentation ("strategic") |  |  |  | X |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM | X | ME | X | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | UMTS Charging and Billing |
| --- | --- |

| Source: | S1_Charging | Date: | 30/07/26 |
| --- | --- | --- | --- |

| Subject: | Upgrading of UMTS Charging and billing features |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature | X |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | Clarification and improvement of concepts related to charging control mechanisms. |
| --- | --- |

| Clauses affected: | Sections 6, 7 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

# 6	Transfer of Charging Information

The efficient transfer of charging information between serving networks and from serving networks to home environments requires a standardised interface between these entities. Transfer of charging information between serving network and home environment shall be done at the following times:

- when a chargeable event occurs

- when a chargeable event is initiated by the user

- when a chargeable event is initiated by the user

- when a chargeable event terminates

- at regular intervals during a chargeable event

- The format of the charging information exchanged (see 5.1) shall be standardised. It shall be possible for the relevant parties to agree minimum and maximum age of call information transferred between themselves.

## 6.1	Integrity, Secrecy and Validation of Content and Receipt of Charging Information

- The transmission mechanism for charging information collected in 5.1 above shall ensure its integrity and secrecy.

- A mechanism to validate the source and integrity of the information shall be provided so that:

- -	The home environment shall be able to validate the source and integrity of the charging information supplied by the serving network;

- -	The serving network shall be able to validate the source and integrity of the charging information supplied by the user;

- -	The serving network shall have proof that services were provided to a specified user.

# 7	Accounting and Settlement

- The serving network shall collect and process the charging data generated in its network elements. The record of each individual transaction shall be reported to the home environment at short notice in order to provide itemised bills, and to deal with any disputes regarding charges both for users and for other UMTS networks and home environment.

## 7.1	Delegation of charging authority

- The registration process allows the home environment to authenticate users before they incur any charges. Once authenticated, the home environment then delegates authority to the serving network operator with which he has a direct commercial relationship to incur charges for services supplied to that user.

- A mechanism to allow the control of chargeable activities in the serving network shall be provided. Through this mechanism the Home Environment may monitor and influence the duration of the chargeable activities in the serving network.

- The charging control mechanism shall be able to supervise all the possible traffic cases (e.g. incoming, outgoing, roaming, call forwarding) and all the possible services (e.g. speech, data, SMS). In release 99 the charging control mechanism might not support the full range of traffic cases.

- The direct commercial relationship may be with either the serving network operator if known directly by the home environment or a network operator known to the home environment. This procedure uses each network as trusted third parties in a chain of delegation between entities, thus allowing commercial transactions between entities who have no direct commercial dealings. There shall be an authentication procedure between all entities in the UMTS system which have a commercial relationship.

## 7.2	Fraud Control and Cost Control

- Mechanisms shall be provided which allow fraud control by the serving networks and the home environment, and shall allow cost control by the user.

### 7.2.1	Fraud Control by the Home Environment

- Charging information shall be collected by the home environment in short time intervals from all serving networks which its users are allowed to use. The billing system in the home environment shall process the information in real time and provide the means to set charge thresholds per time interval upon which some actions may be started, such as informing the customer care centre or even barring the user in the HLR.

### 7.2.2	Fraud Control by the Serving Network

- Charging information shall be collected from the network elements and processed in short time intervals. This will allow the serving network to always be aware of the exposure to visitors. A limit for the accumulated charges for all visitors from one home environment or a limit per visitor may be agreed between the home environment and the serving network.

## 7.2.3	Cost Control by the User

### 7.2.3.1	Charging Limit

- The user shall be able to set in his home environment a limit for the accumulated charges per time interval. Upon exceeding this limit or prior to incurring a charge which would exceed the limit, certain actions may be desired by the user:

- notification to the user, requesting to extend the limit, or

- HLR barring allowing no further originating calls, or

- HLR barring cancelling the roaming permission.

### 7.2.3.2	Advice of Charge

- A mechanism shall be standardised providing an indication to the chargeable party(if involved in the chargeable event) of the charges to be levied for a chargeable event. This mechanism shall be able to handle all possible charging scenarios, and all service and tariff variants that a home environment may offer to the user.

- In particular it shall be possible to inform the charged party of:

- tariff changes due to changes of the location of the user

- tariff changes due to service modifications (e.g. bearers added/subtracted during a multimedia call, QoS changes)

- charges expected for incoming traffic (e.g. roaming, call forwarding, broadcast information received)

The charging indication might be given before, during or at the end of the chargeable event.

In release 99 the charging control mechanism might not support the full range of traffic cases  and indications modes.