TSG-SA Working Group 1 (Services) meeting #4	TSGS1#4(99)521

Québec, Canada, 5th-9th July 1999						Agenda 6.5.4

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification / Report UMTS |  |  |  | 22.121 |  | Version: |  |  | 3.0.0 |  |  |  |
| Submitted to TSG_SA | 5 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  |  |  |
| list TSG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  | X |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | USIM |  | TE |  | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | VHE |
| --- | --- |

| Source: | France Télécom | Date: |  |
| --- | --- | --- | --- |

| Subject: | Generic Service Capability Feature |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | UMTS 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | 1)To clarify the role of the service capability features and their interactions with service capabilities and applications 2) QoS negotiation shall be considered as a framework Service Capability Feature. |
| --- | --- |

| Clauses affected: | 5, 5.1, 7, 8, 10.1 and 10.2. |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

3G TS 22.121 3.0.0 (1999-04)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects

Service aspects;

The Virtual Home Environment

(3G TS 22.121 version 3.0.0)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

DTS/TSGS-0122121U

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Contents

Foreword	6

1	Scope	7

2	References	7

2.1	Normative references	7

2.2	Informative references	8

3	Definitions and abbreviations	8

3.1	Definitions	8

3.2	Abbreviations	8

4	General Description of the VHE	9

5. Framework for Services	11

5.1 	Ways to realise services	12

6	User Requirements of VHE	13

6.1	Personal Service Environment	14

6.1.1 	User Profiles	14

6.1.2 	User Profiles and Multiple Subscriptions	15

6.1.3	Management of the user profile	15

6.1.4 	Requirements for Standardisation	15

7	Home Environment Requirements for VHE Provision	15

8 	Serving Network Requirements for VHE Provision	16

9  	VASP Relationship to VHE	16

10	Service Capability Features	17

10.1	Framework service capability features	17

10.1.1	Authentication service capability feature	17

10.1.2	Authorisation service capability feature	17

10.1.3	Registration service capability feature	18

10.1.4	Discovery service capability feature	18

10.1.5	Notification service capability feature	18

10.1.6	Negotiation service capability feature	18

10.2	Non-Framework service capability features	18

10.2.1	Session Control service capability features	19

10.2.2	Security/Privacy service capability features	19

10.2.3	Address Translation service capability features	19

10.2.4	User Location service capability features	20

10.2.5	User Status service capability features	20

10.2.6	Terminal Capabilities service capability features	20

10.2.7	Message Transfer service capability features	21

10.2.8	Data Download service capability features	21

10.2.9	User Profile Management service capability features	21

10.2.10	Charging service capability features	22

11	VHE execution environment	22

12	Charging requirements	22

13	Security requirements	23

Annex A (Informative)	23

History	25

# Foreword

This Technical Specification has been produced by the 3GPP.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of this TS, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version 3.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	Indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the specification;

# 1	Scope

This document specifies the content of the stage one requirement for realisation of VHE.

Virtual Home Environment (VHE) is defined as a concept for personal service environment (PSE) portability across network boundaries and between terminals. The concept of the VHE is such that users are consistently presented with the same personalised features, User Interface customisation and services in whatever network and whatever terminal (within the capabilities of the terminal and the network), wherever the user may be located.

A key feature to support VHE is the ability to build services using a standardised application interface.

Requirements not applicable for R99 will be explicitly indicated.

# 2	References

References may be made to:

a)	Specific versions of publications (identified by date of publication, edition number, version number, etc.), in which case, subsequent revisions to the referenced document do not apply; or

b)	All versions up to and including the identified version (identified by "up to and including" before the version identity); or

c)	All versions subsequent to and including the identified version (identified by "onwards" following the version identity); or

d)	Publications without mention of a specific version, in which case the latest version applies.

A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

## 2.1	Normative references

[1]	GSM 01.04 (ETR 350): ”Digital cellular telecommunication system (Phase 2+); Abbreviations and acronyms”

[2]	GSM 02.57: ”Digital cellular telecommunication system (Phase 2+); Mobile Station Application Execution Environment (MExE); Service description”

[3]	GSM 02.78: ”Digital cellular telecommunication system (Phase 2+); Customised Applications for Mobile network Enhanced Logic (CAMEL); Service definition - Stage 1”

[4]	GSM 11.14: ”Digital cellular telecommunication system (Phase 2+); Specification of the SIM Application Toolkit for the Subscriber Identity Module - Mobile Equipment; (SIM - ME) interface”

[5]	UMTS TS 22.01: ”Universal Mobile Telecommunications System (UMTS): Service Aspects; Service Principles”

[6]	UMTS TS 22.05: ”Universal Mobile Telecommunications System (UMTS); Services and Service Capabilities”

[7]	ITU-T Recommendation Q.1701, Framework for IMT-2000 networks

[8]	ITU-T Recommendation Q.1711, Network Functional Model for IMT-2000

[9]	UMTS TS 22.00 UMTS phase 1

## 2.2	Informative references

[1]	UMTS TR 22.70: ”Universal Mobile Telecommunications System (UMTS); Virtual Home Environment”

[2]	World Wide Web Consortium Composite Capability/Preference Profiles (CC/PP): A user side framework for content negotiation (www.w3.org)

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of this TS, the following definitions apply:

HE-VASP: Home Environment Value Added Service Provider. This is a VASP that has an agreement with the Home Environment to provide services.

Local Service: A service, which can be exclusively provided in the current serving network by a Value added Service Provider.

Service Capabilities: Bearers defined by parameters, and/or mechanisms needed to realise services. These are within networks and under network control.

Service Capability Feature: Functionality offered by service capabilities that are accessible via the  standardised application interface

Services: Services are made up of different service capability features.

Applications / Clients: These are services, which are designed using service capability features.

Application Interface: Standardised Interface used by application/clients to access service capability features.

Personal Service Environment: contains personalised information defining how subscribed services are provided and presented towards the user. The Personal Service Environment is defined in terms of one or more User Profiles.

Home Environment: responsible for overall provision of services to users

User Interface Profile: Contains information to present the personalised user interface within the capabilities of the terminal and serving network.

User Services Profile: Contains identification of subscriber services, their status and reference to service preferences.

User Profile: This is a label identifying a combination of one  user interface profile, and one user services profile.

Value Added Service Provider: provides services other than basic telecommunications service for which additional charges may be incurred.

Virtual Home Environment: A concept for personal service environment portability across network boundaries and between terminals.

Further UMTS related definitions are given in 3G  TS 22.101.

## 3.2	Abbreviations

For the purposes of this TS the following abbreviations apply:

API	Application Programming Interface

CAMEL	Customised Application For Mobile Network Enhanced Logic

CORBA	Common Object Request Broker Architecture

FFS	For Further Study

ME	Mobile Equipment

MExE	Mobile Station (Application) Execution Environment

MMI	Man Machine Interface

MS	Mobile Station

MSC	Mobile Switching Centre

HLR	Home Location Register

GSN	GPRS Support Nodes

SSF	Service Switching Function

PLMN	Public Land Mobile Network

HE	Home Environment

SAT	SIM Application Tool-Kit

SIM	Subscriber Identity Module

SMS	Short Message Service

USIM	User Service Identity Module

USSD	Unstructured Supplementary Service Data

VASP	Value Added Service Provider

HE-VASP	Home Environment Value Added Service Provider

PSE	Personal Service Environment

VHE	Virtual Home Environment

LCS	LoCation Services

CAP	Camel Application Part

MAP	Mobile Application Part

CSE	Camel Service Environment

OSA	Open Service Architecture

Further GSM related abbreviations are given in GSM 01.04. Further UMTS related abbreviations are given in UMTS TS 22.01.

# 4	General Description of the VHE

Virtual Home Environment (VHE) is defined as a concept for personal service environment portability across network boundaries and between terminals. The concept of the VHE is such that users are consistently presented with the same personalised features, User Interface customisation and services in whatever network and whatever terminal (within the capabilities of the terminal and network), where ever the user may be located.

- The key requirements of the VHE are to provide a user with a personal service environment which consist of:

- Personalised services;

- Personalised User Interface (within the capabilities of terminals);

- Consistent set of services from the user’s perspective irrespective of access e.g. (fixed, mobile, wireless etc. Global service availability when roaming

The standards supporting VHE requirements should be flexible enough such that VHE can be applicable to all types of future networks as well as providing a framework for the evolution of existing networks.  Additionally the standards should have global significance so that user’s can avail of their services irrespective of their geographical location. This implies that VHE standards should:

- provide a common  access for services  in future networks;

- enable the  support of  VHE  by future networks.

- enable the creation of services,

- enable personal  service environment to be recoverable (e.g in the case of loss/damage of user equipment)

Roles and components involved in realisation of VHE consist of the following also see fig 1:

- Home Environment

- User Identifiers

- Users

- Terminals (simultaneous activation of terminals providing the same service per single subscription is not allowed)

- Serving Networks

- Subscriptions

- Possibly Value added service providers.

- Personal Service Environment

- User Profiles

- Fig 1: The Set of Services from the User’s Point of View

- The Home Environment provides and controls services to the user in a consistent manner. The User’s personal service environment is a combination of services and personalisation information (described in the user profile).  The user may have a number of user profiles which enable her to manage communications according to different situations or needs, for example being at work, in the car or at home.  Services provisioned to the user may allow or require personalisation by the user.

- The Home Environment provides services to the user in a managed way, possibly by collaborating with HE-VASPs, but this is transparent to the user.  The same service could be provided by more than one HE-VASP and HE-VASP can provide more than one service.

- Additionally, but not subject to standardisation, the user may access services directly from Value Added Service Providers. The Home Environment does not manage services obtained directly from VASPs.   A mechanism may be provided which allows the user to automate access to those services obtained directly from VASPs and personalise those services. However such a mechanism is outside of the scope of this specification.

# 5. Framework for Services

- The implementation of VHE in UMTS release 99 shall support both GSM phase 2+ release 99 teleservices, bearer services and supplementary services as applied in 3G TS 22.100 and new services built by service capability features.  Later UMTS developments will provide support for a wider range of services in later releases.

- Fig 2: Framework for Services.

- The goal of standardisation in UMTS with respect to services is to provide a framework within which services can be created based on standardised service capability features see figure 2 and 3. UMTS services will generally not rely on the traditional detailed service engineering (evident for supplementary services in second-generation systems), but instead provides services using generic toolkits.

- Services can be built using service capability features ([1] [2],[3],[4],[9], [10]), which are accessed  via a standardised interface.  An example of how a service can be built on service capability features could be “call to nearest restaurant”, this will make use of call set-up, authorisation, location and database lookup.

The available service capability features are visible to  applications through the standardised application interface. The application uses the service capability features but does not have knowledge of the underlying mechanisms implied, i.e. the service capabilities used to implement the service capability features: for example, the User Location service capability feature can be based on several location server (like HLR, LCS) but the application does not know how the service capability feature is effectively implemented.

The standardised application interface shall be:

- Independent of vendor specific solutions,

- Independent of programming languages, operating systems, etc, used in the service capabilities,

- Secure scalable and extensible,

- Independent of the location where service capabilities are implemented,

- Independent of supported server capabilities in the network,

Access to Service Capability Features shall be realised using modern state of the art access technologies, e.g. distributed object oriented technique might be considered.

## 5.1 	Ways to realise services

The information contained in this clause is only to aid understanding and is not an extensive list.

Figure 3 illustrates how the concept of VHE makes use of the standardised application interface and how that fits to the service capability features and service capabilities for release 99. It is not to be implied as the agreed architecture as this is a stage 2 issue.

Figure 3	 Possible realisation of Framework for Services

- STANDARDISED SERVICES (Supplementary Services, Tele-Services, etc.) are implemented on existing GSM/UMTS entities (e.g. HLR , MSC/VLR and terminal) on a vendor specific basis, using standardised interfaces (MAP, etc.) for service communication (e.g. downloading of service data).   Availability and maintenance of these Services is also vendor dependent.

- OPERATOR SPECIFIC SERVICES (OSS) are not standardised and could be implemented at the GSM/UMTS entities (e.g. HLR) on a vendor specific basis or using GSM ph 2+ mechanisms (CAMEL, SAT, MExE). These tool-kits use standardised interfaces to the underlying network (CAP, MAP) or use GSM Bearers to transport applications and data from the MExE/SAT server to the MS/SIM. The implementation of these operator specific services on the different platforms (CSE, MExE/SAT Server, MSs) is done in a completely vendor specific way and uses only proprietary interfaces.

- Other APPLICATIONS are like OSS not standardised. These applications will be implemented using standardised interfaces to the Service Capabilities Features

- Within the terminals Service Capabilities are accessible via the existing MExE and SAT APIs, i.e. there will be no service capability features within the terminal. 
The terminal can communicate, using GSM/UMTS bearers, with applications in the network via the service capability features defined for MExE- and SAT-servers.

# 6	User Requirements of VHE

- The user shall have the possibility to manage services as well as the appearance of the services.  It shall be possible for the user to:

- Personalise services,

- Personalise User Interface (within the capabilities of terminals),

- Access services from any network or terminal subject to network capabilities, terminal capabilities and any restrictions imposed by the home environment,

- Use services in a consistent manner irrespective of serving network and terminal, within the technical limitations,

- Access new services in the Home Environment,

- Modify a user profile(for example to include new services) from any location,

- Activate or deactivate user services,

- Discover which local services are available,

- Access local services in a secure manner,

- Interrogate current user service and user interface settings,

- select a particular User Profile,

- indicate (on a session by session basis if necessary) to which subscription charges are to be applied to,

- recover MS resident User Profile information to protect against loss or damage of user equipment,

- Be aware of limitations of services, which may result from different terminals and or serving network capabilities.

## 6.1	Personal Service Environment

The Personal Service Environment describes how the user wishes to manage and interact with their communications services. The PSE is a combination of a list of subscriptions (detailing provisioned services), preferences associated with those services, terminal interface preferences and other information related to the user's experience of the system. Within the PSE the user can manage multiple subscriptions e.g. both business and personal, multiple terminal types and express location and temporal preferences. The Personal Service Environment is defined in terms of one or more User Profiles.

### 6.1.1 	User Profiles

A combination of different preferences is described by a User Profile. The user can define one or more User Profiles according to their needs.

Each User Profile consists of two kinds of information:

- interface related information (User Interface Profile)

- services related information (User Services Profile)

- A User Interface Profile consists of the following type of information:

- menu settings, e.g. menu items shown, menu structure, the placement of icons,

- terminal settings, e.g. ringing tone and volume, font type and size, screen and text colour, language, content types and sizes accepted,

- network related preferences e.g. language used for announcements, (editor’s note: for clarification).

A User Services Profile consists of the following information:

- a list of services subscribed to and references to Service Preferences for each of those services if applicable. Service Preferences could be information such as redirection numbers, redirection conditions, caller screening lists, time-of-day variations etc,

- service status (active/deactive).

The user may define one or more User Interface Profiles and many User Services Profiles, but a given User Profile consists of a single combination of these. In this way a user could for example have a different User Profile to suit each of the three different terminals she owns. The User Services Profile is the same in each case but the User Interface Profile is different to suit the display capabilities of each terminal. User Profiles could also exist which use the same User Interface Profile but different User Services Profiles. This might simply imply that business calls are forwarded to an answering service when the user leaves the office because a new User Profile is now active.

Where the user has more than one User Profile the activation of a particular one could be done in the following ways:

- Statically: the user explicit selects one of the User Profiles as the active one

- Dynamically: the appropriate User Profile is selected automatically based upon some criteria such as time of day, location, terminal used or many other possibilities

Each User Profile must have an identity.

- For UMTS Release '99 the information in the User Profiles enables the service capabilities SAT, MExE and CAMEL toolkits in R'99 and existing GSM services to support the user’s PSE across network boundaries and between different terminals.

- It shall be possible for the service capabilities to access the user profile information from the home environment if appropriate.

### 6.1.2 	User Profiles and Multiple Subscriptions

The user may wish to manage more than one subscription in their PSE. This would allow them to have a single USIM but specify different preferences for the services provisioned in each subscription. In this case the User Services Profile will need to detail all of the services provided per subscription and provide references to the service preferences for each service. When initiating a chargeable event the user will need to indicate which subscription the charges should be applied to.

### 6.1.3	Management of the user profile

- The user and the home environment may modify the user’s characterisation of the Personal Service Environment as described in the User Profiles at any time, and changes become effective at the earliest possible opportunity. The home environment shall be able to update distributed User Profiles to reflect any user or home environment modification of the user’s Personal Service Environment.

- The User Profiles may be stored in the Mobile Station (the SIM or the ME), and/or the home environment. The information in User Services Profiles is distributed between the home environment and the MS. In the event of loss/damage of mobile station (SIM or ME), the User Profiles must be fully recoverable and be used to reconfigure a new mobile station.

- Some aspects of the User Profiles such as aspects related to terminal configuration, must be stored in a standardised format to support VHE.

- (Note: To ensure that User Profiles are applicable to as wide a community of terminal and network types as possible, existing work on this topic in other standards fora should be considered. One possibility is the work of the World Wide Web Consortium on the Composite Capability/Preference Profile [2].

### 6.1.4 	Requirements for Standardisation

- To facilitate the provision of PSE and User Profiles the standard shall:

- uniquely identify User Profiles,

- provide a standardised format for describing terminal configuration preferences.

# 7	Home Environment Requirements for VHE Provision

It shall be possible for the home environment to:

- Control access to services depending on the location of the user, and serving network,

- Control access to services on a per user basis, e.g. subject to subscription,

- Control access to services depending on available service capabilities in the serving network, and terminals,

- Manage service delivery based on for example end to end capabilities and/or user preferences,

- Request  version of specific services supported in serving network and terminal,

- Request details (e.g. protocol versions and API versions) of available service capabilities features supported in the serving network, ,

- Request information on the available service capabilities supported by its users’ terminals,

- Inform the serving network on its ability to support any given service capability,

- Inform the serving network on the ability of its users’ terminals to support any given service capability,

- Define the scope for management of services by the user, for services provided by the HE,

- Handle charging for services (as defined in clause 11),

- Deploy services to users or groups of users,

- Manage provision of services to users or groups of  users.

# 8 	Serving Network Requirements for VHE Provision

- The serving network should not need to be aware of the services offered via the home environment.

- The user/home environment may request capabilities, which are necessary to support, home environment services.

It shall be possible for the serving network to perform the following:

- The serving network shall support user access to services in the home environment,

- The serving network shall provide the necessary service capability features to support the services from the home environment as far as possible,

- Dynamically provide information on the available service capability features in the serving network,

- Request information on the available service capabilities supported by the home environment and the terminals of its users,

- Provide transparent communication between clients and servers in terminals and networks.

# 9  	VASP Relationship to VHE

- The user may access services directly from Value Added Service Providers. Services obtained directly from VASPs are not managed by the Home Environment and therefore are not part of the VHE offered by the Home Environment.  A mechanism should be provided which allows the user to automate access to those services obtained directly from VASPs and personalise those services. However such a mechanism is outside of the scope of this specification.

- There may be some information, which is shared between the Home Environment and the HE-VASP (for example current capabilities).

- The Home Environment may grant the HE-VASP access to standardised service capabilities in order to allow the development and deployment of services on behalf of the Home Environment.

- There are no VASP requirements to support VHE.

# Service Capability Features

Services Capability Features are open, technology independent building blocks accessible via a standardised application interface. This interface shall be applicable for a number of different business and applications domains (including besides the telecommunication network operators also service provider, third party service providers acting as HE-VASPs, etc.).

All of these businesses have different requirements, ranging from simple telephony and call routing, virtual private networks, fully interactive multimedia to using MS based applications.

The service capability features shall enable applications to make use of the service capabilities (e.g. CAMEL, MExE, etc) of the underlying UMTS network in an open and secure way.

Application/Clients access the service capability features via the standardised application interface. This means that a single service capability feature is accessible and visible to application/clients via the method/operation invocations in the interface.

Two different types of service capability features can be distinguished:

- Framework service capability features: these shall provide commonly used utilities, necessary for the non-framework service capability features to be accessible, secure, resilient and manageable.

- Non-Framework service capability features: these shall enable the applications to make use of the functionality of the underlying network capabilities (e.g. User Location service capability feature).

## 10.1	Framework service capability features

Framework service capability features will be used e.g. for authentication, registration, notification, etc. and provide functionality that is independent of any particular type of service. Other commonly used service capability features may be added later.

### 10.1.1	Authentication service capability feature

Authentication is used to verify the identity of an entity (user, network, and application).

Three types of authentication are distinguished:

- User-Network Authentication. Before a user can access her subscribed applications, the user has to be authenticated by the network that provides access to the application. This allows the network to check to what applications the user has subscribed to. User-network authentication is handled within the network and therefore outside the scope of this specification.

- Application-Network Authentication. Before an application can use the capabilities from the network, a service agreement has to be established between the application and the network. Establishment of such a service agreement starts with the mutual authentication between application and network. If a service agreement already exists, modification might be needed or a new agreement might supersede the existing.

- User-Application Authentication. Before a user can use an application or perform other activities (e.g. modifying profile data) the application provider must authenticate the user. When the network already authenticates the user, authentication is not needed anymore. When the network is transparent and the user accesses an application directly, authentication is needed between user and application but this is outside the scope of this specification.

### 10.1.2	Authorisation service capability feature

Authorisation is the activity of determining what an authenticated entity (user, network, and application) is allowed to do (note: authentication must therefore precede authorisation).

Two types of authorisation are distinguished:

- Application-Network Authorisation. The network verifies what non-framework service capability features s  (or even some framework service capability features) the application is allowed to use. Once an application has been authorised to use one, more or all (non-framework) service capability features no further authorisation is required as long as the "allowed" (non-framework) service capability features  are used.

- User-Application Authorisation. The application verifies what actions the user is allowed to perform (e.g. [deactivation of functionality, modification of application data). This is transparent to the network and therefore outside the scope of this specification.

### 10.1.3	Registration service capability feature

The Registration service capability feature enables the non-framework service capability features  (e.g. User Location) to register at the Framework.  Registration must take place before authorised applications can find out from the Framework which non-framework service capability features are available. This means that the non-framework service capability features must be registered before they can be discovered and used by authorised applications.

Note that only the non-framework service capability features have to be registered. The Framework service capability features (defined in section 10.1) are available by default since they provide basic mechanisms.

### 10.1.4	Discovery service capability feature

The Discovery service capability feature enables the application to identify the total collection of service capability features that it can use. Upon request of the application, the Discovery service capability feature will indicate  the non-framework service capability features  that are available for the application. The list of available service capability features  is created through the Registration process described in section 10.1.3. This means that a service capability feature must be registered at the Framework before it can be discovered by the application.

### 10.1.5	Notification service capability feature

The Notification service capability feature allows applications to enable, disable and receive notifications of application related events that have occurred in the underlying GSM/UMTS network, e.g. indication that a new call is set-up or a message is received.

Note: It should be further studied if Notification is only a Framework service capability feature  or also specialised as non-framework service capability features (e.g. for notifications on location update, disconnected party etc.)

### 10.1.6	Negotiation service capability feature

The negotiation service capability feature allows applications to negotiate the Quality of service provided by the non-framework service capability features.

## 10.2	Non-Framework service capability features

The Non-Framework service capability features represent the total collection of service capability features that are not included in the Framework. These non-framework service capability features  enable the application to make use of the functionality provided by the network and service capabilities.

Service capability features shall be defined in a generic way to hide the network specific implementation. To achieve this, it is necessary to identify the functionality that is provided by more than one service capabilities. For example, User Location can be produced in several underlying ways. This functionality is captured once when defined the service capability features in a generic way. Using the generic service capability features, the applications become independent of (portable over) underlying service capabilities.

The service capability features shall be able to detect which service capabilities are available: e.g. in the case of a service capability based on a SAT application, the SCF shall be able to detect the presence of this application in the MS.

The following sections define generic service capability features e.g. for Session Control and Message Transfer.

### 10.2.1	Session Control service capability features

This section details the Session Control related service capability features.  Session Control service capability features shall offer the functionality to establish, maintain, modify and release bearers to/from other parties or entities.

Herein, the term “session” can mean anything from a simple voice call to a complex multimedia “call” (including exchange of non delay-sensitive data). To define the necessary service capability features it is proposed to use a generic model (including the "session party handling").

For example, the following Session Control service capability features shall be provided (the list is not exhaustive):

- initiate and create session (e.g. used to set-up a Telephony session “out of the blue”)

- allow the session to continue with modified information (e.g. changed destination number)

- release the session (i.e. removing all parties from the session)

- add bearer to the session

- remove bearer from the session

- resume bearer to the session (i.e. move party from “on-hold” into Telephony Session)

- suspend bearer from the session  (i.e. move party from Telephony Session to “on hold”)

- request session  information (i.e. information like session duration, session end time)

- supervise session (e.g. monitor for session duration or data volume, tariff switching moments and changes in QoS)

- presentation of, or restriction of, information associated with a party involved in a session  (e.g. calling line ID, calling name)

For each session it shall be possible to specify:

- the desired media type (e.g. video, voice, non-real time data etc.)

- the events on which monitoring is required ([3])

Note: the mapping to service capabilities is for further study. (It shall be investigated to which extend the requirements above fit to CAMEL, MEXE and other service capabilities.)

### 10.2.2	Security/Privacy service capability features

For the Security/Privacy the following service capability features shall be supported:

- encryption of user data and signalling;

### 10.2.3	Address Translation service capability features

The Address Translation enables the application to find out from the underlying network what the user's addresses are. Based on a known user address, the application may request another address (e.g. based on the E.164 number, the user's e-mail is retrieved). The range of addressing options includes:

- E.164 Numbering (e.g. GSM MS-ISDN);

- ASEA Numbering (ATM);

- IP v6 numbering;

- X.25 Numbering;

- Internet symbolic naming.

### 10.2.4	User Location service capability features

The User Location service capability features provide an application with information concerning the user’s location.

The user location information contains the following attributes:

- location (e.g. in terms of universal latitude and longitude co-ordinates)

- accuracy (value depending on local regulatory requirements and level of support in serving/home networks; note that the accuracy of the serving network might differ from that in the home environment)

- age of location information (last known date/time made available in GMT)

The following service capability features shall be provided:

- Report of location information

The application shall be able to request user location information.

By default the location information is provided once; the application may also request periodic location reporting (i.e. multiple reports spread over a period of time)

- Notification of location update

The application shall be able to request to be notified when the user’s location changes, i.e. when:

- the user enters or leaves a specified geographic area

- the user’s location changes more than a specified lower boundary. The lower boundary can be selected from the options provided by the network.

The application shall be able for each user to start/stop receipt of notifications and to modify the required accuracy by selecting another option from the network provided options.

- Access control to location information

The user shall be able to restrict/allow access to the location information. The restriction can be overridden by the network operator when appropriate (e.g. emergency calls).

### 10.2.5	User Status service capability features

The User Status service capability features enable an application to retrieve the user's status, i.e. to find out on which terminals the user is available.

The following service capability features shall be provided:

- Retrieval of User Status

The application shall be able to retrieve the status of the user

- Notification of User Status Change

The application shall receive notifications when the user's terminal attaches or detaches.

- Detach: the user's terminal is switched on or the network initiates detach upon location update failure

- Attach: the user's terminal is switched on or there has been a successful location update after network initiated detach.

The application shall be able for each terminal to start/stop receipt of notifications.

### 10.2.6	Terminal Capabilities service capability features

(* Editor’s note: this section needs to be checked against the MExE specifications *)

The Terminal Capabilities service capability features enable the application to find out what capabilities the user's terminal supports (note: "terminal" covers both (mobile) equipment and USIM).

The following service capability features shall be provided:

- Retrieval of Terminal Capabilities

The application shall be able to retrieve the capabilities of the terminal. This includes:

- the media that the terminal is capable to deal with (e.g. audio, video, PC data, WAP data; this information is needed by the application e.g. when the user wants to download messages from the mailbox);

- the number of calls that the terminal can deal with simultaneously.

### 10.2.7	Message Transfer service capability features

The Message Transfer service capability features enable an application to put a message in the user’s mailbox and to send message notifications to the user. A message can e.g. be of type video, audio, e-mail, fax, SMS etc.; a message can also contain an attachment (e.g. a video file attached to an e-mail).

The following service capability features shall be supported:

- Send message to mailbox

The application shall be able to put a message in the user’s mailbox. The application can e.g. leave a message for a user indicating a missed call. The type of the message (video, audio, e-mail) needs to be specified. Messages may contain attachments.

- Send message to user

The application shall be able to send a message to the user directly (i.e. the message is not stored in the mailbox). Examples are a fax message and an announcement like "your call is being diverted".

- Get message from mailbox

The application shall be able to fetch a message from the mailbox (when requested by the user to do so).

- Send message notification

The application shall be able to send a notification to the user e.g. when it has put a message in the mailbox or when it has received a notification from the mailbox that a new message has arrived for the user.

- Request message receipt notification

The application can request to receive a notification every time a message is received in the mailbox for the user. This allows the application to take the appropriate action, e.g. informing the user.

- Collect message/data from user

The application shall be able to collect a message/data from the user. For example, the user might enter a PIN code.

### 10.2.8	Data Download service capability features

To allow the support of home environment / serving network specific services the following service capability features shall be supported;

- capability to download applications, data to the terminal;

- capability to download applications, data to the USIM;

### 10.2.9	User Profile Management service capability features

The User Profile Management service capability features allow the application to retrieve the user profile (see section 7.1 for more information on user profiles).

### 10.2.10	Charging service capability features

The Charging service capability features enable the application to instruct the network and inform the user with charging information and to add some additional charging information to the network generated Call Detail Records.

The following service capability features shall be provided:

- Define session duration, data volume, etc.

- Send charging data (this data is included in a “free format” field in the network generated Call Detail Records. It may contain information like a application generated Call Id, used by the application provider to relate application generated charging information to the network generated charging information)

- Transfer of Advice of Charge data (as defined in GSM02.24) to the terminal.

# 11	VHE execution environment

- The following service execution environments shall be standardised and could be used to provide a VHE for the user:

- User equipment execution environment

- IC card execution environment

- Network execution environment not required for  R99

- For UMTS release 99 one or more of the following shall provide the execution environments:

- MExE

- SIM  Application Tool kit (SAT)

- CAMEL

# 12	Charging requirements

Services, which are provided as part of the VHE, may be subject to charge at the discretion of the home environment

There are several forms of charging which shall be available to the home environment. It shall be possible for the home environment to charge in the following instances: -

-	Subscription;

the user’s registration to use services may be subject to charge;

-	Service transfer;

the transfer of services and/or information to the user MS or USIM may be subject to charge;

-	Service upgrading;

the upgrading of previously transferred services to the user’s MS or USIM may be subject to charge (automated upgrading of services may be subject to a different charge);

-	Service usage;

the usage of services by a user may be subject to a charge;

-	Roaming

the usage of VHE services when roaming may be subject to additional charges;

Refer to UMTS 22.15 for further details.

Other charging requirements may be identified and are for FFS.

# 13	Security requirements

The mechanisms supporting VHE shall maintain a secure environment for the user and home environment.

The specific security requirements are FFS.

# Annex A (Informative)

The following table shows the service examples to be considered in VHE

| Benchmark Services | Abb | Priority |
| --- | --- | --- |
| Abbreviated Dialling | ABD | A |
| Account Card Calling | ACC | B |
| Automatic Alternative Billing | AAB | A |
| Call Distribution | CD | A |
| Call Forwarding | CF | A |
| Call Hold | CH | A |
| Call Rerouting Distribution | CRD | A |
| Call Transfer | TRA | A |
| Call Waiting | CW | A |
| Completion of Call to Busy Subscriber | CCBS | A |
| Conference Calling | CON | A |
| Credit Card Calling | CCC | B |
| Destination Call Routing | DCR | A |
| Follow-Me Diversion | FMD | A |
| Freephone | FPH | A |
| Global Virtual Network Service | GVNS | A |
| Hot Line | HOT | A |
| International Telecommunication Charge Card | ITCC | B |
| Internetwork Freephone | IFPH | A |
| Internetwork Mass Calling | IMAS | A |
| Internetwork Premium Rate | IPRM | A |
| Internetwork Televoting | IVOT | A |
| Malicious Call Identification | MCID | A |
| Mass Calling | MAS | A |
| Message store and forward | MSF | A |
| Multimedia | MMD | B |
| Originating Call Screening | OCS | A |
| Premium Rate | PRM | A |
| Security Screening | SEC | A |
| Selective Call Forward on Busy / Dont' answer | SCF | A |
| Split Charging | SPL | A |
| Televoting | VOT | A |
| Terminating Call Screening | TCS | A |
| Terminating Key Code Protection | TCKP | B |
| Universal Access Number | UAN | B |
| Universal Personal Telecommunication | UPT | A |
| User-Defined Routing | UDR | B (FFS) |
| Virtual Private Network | VPN | A |

Benchmark services listed above could be realised by service capability features.

# History

| Document history |  |  |
| --- | --- | --- |
| Date | Version | Comment |
| Sep 1998 | 0.0.0 | Initial Draft at Ipswich |
| Oct 1998 | 0.1.0 | Initial Draft at Uxbridge Meeting |
| Nov 1998 | 0.2.0 | Draft at Newbury Meeting |
| Jan 1999 | 0.3.0 | Draft after Stockholm meeting |
| Jan 1999 | 0.4.0 | Draft to include requirement for R99 |
| Feb 1999 | 0.5.0 | Draft to include corrections from Ericsson and TTC requirements |
| Feb 1999 | 0.6.0 | Draft to include changes made after adhoc meeting in Vienna |
| March 1999 | 0.7.0 | Change to include text to fig 2 and 3 and for presentation to v1.0 |
| March 12/99 | 1.0.0 | Input text to state the inclusion of OSA requirements |
| March 17/99 | 1.1.0 | Addition of Annex B, OSA text |
| April 30/99 | 1.2.0 | Modification after Dublin Meeting |
| May 19/99 | 1.3.0 | Modification after Hampton CT. |
| June 99 | 2.0.0 | Modification at Sophia Antipolis |
| Rapporteur: Jumoke Ogunbekun Fujitsu Europe Telecom Email: j.ogunbekun.co.uk Telephone: +44-181-606-4671 |  |  |