TSG-SA Working Group 1 (Services) meeting #6	TSG S1 (99)873

San Diego, California, USA 29th Nov-3rd Dec 1999	Agenda Item: x.y.z

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification / Report UMTS |  |  |  | 22.121 |  | Version: |  |  | 3.1.0 |  |  |  |
| Submitted to TSG_SA | 5 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  |  |  |
| list TSG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  | X |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | USIM |  | TE |  | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | Virtual Home Environment |
| --- | --- |

| Source: | Motorola | Date: | 10/10/99 |
| --- | --- | --- | --- |

| Subject: | The role of MExE and SAT servers in OSA |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature | X |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | UMTS 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | The MExE group has informed S1 that the MExE server does not support OSA. The current text is confusing and inconsistent with the information received. The text is modified to reflect the role of MExE servers as stated by the MExE experts. The role of SAT servers is considered to be the same as that of MExE servers in a terminal execution environment. |
| --- | --- |

| Clauses affected: | 5.1 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 5.1 	Ways to realise services

The information contained in this clause is only to aid understanding and is not an extensive list.

Figure 3 illustrates how the concept of VHE makes use of the standardised application interface and how that fits to the service capability features and service capabilities for release 99. It is not to be implied as the agreed architecture as this is a stage 2 issue.

Figure 3	 Possible realisation of Framework for Services

- Figure 3 shows the service capabilities and service capability features that have been defined for VHE/OSA in UMTS Release 99.  MExE and SAT provide execution environments in the terminal and SIM respectively. This means that MExE and SAT applications reside and execute in the terminal and SIM respectively, and not in the MExE or SAT servers.  This behaviour is significantly different to that of the CSE and HLR "servers" where applications reside and execute in the servers.

For a full understanding of the role of MExE and SAT servers in figure 3 it is important to note the following:-

- MExE servers

- The MExE server is an entity that can be either inside or outside the Home Environment. The functionality of a MExE server is not defined and may be realised in many different ways (e.g. a web server, a WAP gateway, another MExE terminal, a web homepage or even another application etc.).  Because of this virtual nature of a MExE server, and the fact that realisations of MExE servers exist outside the scope of 3GPP, MExE servers in general do not support OSA.

- Support of OSA service capability features on a manufacturer's MExE server is out of scope of VHE/OSA standardisation, however it is noted that the in some instances (i.e. where the MExE server is a WAP Gateway or similar server), that the operator of this entity may conditionally provide OSA service capability features.

- MExE supports VHE through provision of terminal related parts of the user profile, and service capability features in the form of APIs available in the MExE device.

- SAT servers

- The SAT server is an entity that is typically in the Home Environment. As for the MExE server, the functionality of a SAT server is not defined and could be realised in many different ways.

- SAT supports VHE through the use of service capability features in the form of APIs available in the SIM/terminal.

Three kinds of services can be distinguished:

STANDARDISED SERVICES (Supplementary Services, Tele-Services, etc.) are implemented on existing GSM/UMTS entities (e.g. HLR , MSC/VLR and terminal) on a vendor specific basis, using standardised interfaces (MAP, etc.) for service communication (e.g. downloading of service data).   Availability and maintenance of these Services is also vendor dependent.

OPERATOR SPECIFIC SERVICES (OSS) are not standardised and could be implemented at the GSM/UMTS entities (e.g. HLR, MSC/VLR, CSE and terminal) on a vendor specific basis or using GSM ph 2+ mechanisms (CAMEL, SAT, MExE). These tool-kits use standardised interfaces to the underlying network (CAP, MAP) or use GSM Bearers to transport applications and data from the MExE/SAT server to the MS/SIM. The implementation of these operator specific services on the different platforms (e.g. CSE, MExE, SIM, MSs) is done in a completely vendor specific way and uses only proprietary interfaces.

Other APPLICATIONS are like OSS not standardised. These applications will be implemented using standardised interfaces to the Service Capabilities (Bearers, Mechanisms). The functionality offered by the different Service Capabilities are defined by Service Capability Features. These Service Capability Features will be standardised and can be used by the application designers to build their applications.

Within the terminals Service Capabilities are accessible via the existing MExE and SAT APIs which are the equivalent of service capability features in a terminal, i.e. there are no service capability features within the terminal.  The terminal can communicate, using GSM/UMTS bearers, with applications in the network via the APIs provided in the terminal.