###### TSG-SA Working Group 1 (Services) meeting #5	TSG S1 (99) 982

San Diego, USA 29th Nov – 3rd Dec 1999	Agenda Item: 4.3.3

3GPP TSG-CN3 / ETSI SMG3 WPD	 N3-99374

Meeting #6, Sophia Antipolis, France, 
18-22 October 1999

- To:	TSG-CN WG1, TSG-T WG2

- Cc:	TSG-SA WG1, TSG-CN WG2, CN WG2 ss adhoc,

- From:	TSG-CN WG3

Title:	Response to LS on Service/Baseline Implementation Capabilities

___________________________________________________________________________

TSG-CN WG3 have studied the LS from TSG CN WG1 (N1-99B33) on Service/Baseline Implementation Capabilities and would like to offer the following comments related to table 2.  Note: Table 2 (Terminal Service Implementation Capability for NAS) is attached to this LS, for reference:

- Within the UMTS Session Management sections, the “Secondary PDP Context Activation” and “Network Requested PDP context activation” procedures (as defined in 23.060) appear to have been omitted.

- N3 were also wondering if it is worth mentioning that the “PDP context modification” and “PDP context deactivation” procedures could also be initiated from the SGSN or GGSN as well as the Terminal?

Table 2: Terminal Service Implementation Capability (1/2) for NAS	M: mandatory, O: Optional, C: Conditional

|  |  | Service Implementation Capabilities |  | Ref. Doc | Section(s) | Tele-service for Terminals |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Speech (w/ E. call) | FAX | SMS-PP | SMS-CB |  |  |
|  | Layer 3 specification | UMTS Call Control (Optional) | Mobile originating call Establishment | 24.008 | 5.2.1 | M | M | - | - |  |  |
|  |  |  | Mobile terminating call Establishment | Ditto | 5.2.2 | M | M | - | - |  |  |
|  |  |  | Call clearing | Ditto | 5.4.2 | M | M | - | - |  |  |
|  |  |  |  |  | 5.4.3 | M | M | - | - |  |  |
|  |  |  |  |  | 5.4.4 | M | M | - | - |  |  |
|  |  |  | In-band tones and announcements | Ditto | 5.5.1 | M | M | - | - |  |  |
|  |  |  | Status procedure | Ditto | 5.5.3 | M | M | - | - |  |  |
|  |  |  | DTMF protocol control procedure | Ditto | 5.5.7 | M | - | - | - |  |  |
|  |  | UMTS Session Management (Optional) | PDP context activation | Ditto | 6.1.3.1 | - | - | - | - |  |  |
|  |  |  | Secondary PDP Context Activation | Ditto | ? | - | - | - | - |  |  |
|  |  |  | MS Initiated PDP context modification procedure | Ditto | 6.1.3.2 | - | - | - | - |  |  |
|  |  |  | SGSN Initiated PDP context modification procedure | Ditto | ? | - | - | - | - |  |  |
|  |  |  | GGSN Initiated PDP context modification procedure | Ditto | ? | - | - | - | - |  |  |
|  |  |  | MS Initiated PDP context deactivation procedure | Ditto | 6.1.3.3 | - | - | - | - |  |  |
|  |  |  | SGSN Initiated PDP context deactivation procedure | Ditto | ? | - | - | - | - |  |  |
|  |  |  | GGSN Initiated PDP context deactivation procedure | Ditto | ? | - | - | - | - |  |  |
|  |  |  | AA PDP context activation | Ditto | 6.1.3.4 | - | - | - | - |  |  |
|  |  |  | AA PDP context deactivation | Ditto | 6.1.3.5 | - | - | - | - |  |  |
|  |  |  | Receiving a SM STATUS message by a SM entity | Ditto | 6.1.3.6 | - | - | - | - |  |  |
|  |  | SMS (Optional) | CM-procedure | 24.011 | 5 | - | - | M | - |  |  |
|  |  |  | SM-RL-procedure | Ditto | 6 | - | - | M | - |  |  |
|  |  |  | Message format on BTS-MS I/F | 04.12 | 3 | - | - | - | M |  | 04.12 For further study |
|  |  | Supplement. (Opt) | Generic Procedure for the control of SS | 24.010 | 2 | See comments(CFU+CC) | See comments(CC) | - | - |  | If speech call supported then CFU is Mandatory. SS&CC are Related. |
|  |  |  | SS Support procedure | Ditto | 3 | See comments (CFU+CC) | See comments(CC) | - | - |  |  |

Table 2: Terminal Service Implementation Capability (2/2)

|  |  | Service Implementation Capabilities |  | Ref. Doc | Section(s) | Bearer service for Terminals |  | Supplementary service for Terminals | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Circuit SW data | Packet SW data | Call Forward, Advise of Change, USSD, Explicit Call transfer, and others |  |
|  | Layer 3 specification | UMTS Call Control (Optional) | Mobile originating call Establishment | 24.008 | 5.2.1 | M | - | - |  |
|  |  |  | Mobile terminating call Establishment | Ditto | 5.2.2 | M | - | - |  |
|  |  |  | Call clearing | Ditto | 5.4.2 | M | - | - |  |
|  |  |  |  |  | 5.4.3 | M | - | - |  |
|  |  |  |  |  | 5.4.4 | M | - | - |  |
|  |  |  | In-band tones and announcements | Ditto | 5.5.1 | M | - | - |  |
|  |  |  | Status procedure | Ditto | 5.5.3 | M | - | - |  |
|  |  |  | DTMF protocol control procedure | Ditto | 5.5.7 | - | - | - |  |
|  |  | UMTS Session Management (Optional) | PDP context activation | Ditto | 6.1.3.1 | - | M | - |  |
|  |  |  | Secondary PDP Context Activation | Ditto | ? | - | ? | - |  |
|  |  |  | MS Initiated PDP context modification procedure | Ditto | 6.1.3.2 | - | ? | - |  |
|  |  |  | SGSN Initiated PDP context modification procedure | Ditto | ? | - | ? | - |  |
|  |  |  | GGSN Initiated PDP context modification procedure | Ditto | ? | - | ? | - |  |
|  |  |  | MS Initiated PDP context deactivation procedure | Ditto | 6.1.3.3 | - | ? | - |  |
|  |  |  | SGSN Initiated PDP context deactivation procedure | Ditto | ? | - | ? | - |  |
|  |  |  | GGSN Initiated PDP context deactivation procedure | Ditto | ? | - | ? | - |  |
|  |  |  | AA PDP context activation | Ditto | 6.1.3.4 | - | O | - |  |
|  |  |  | AA PDP context deactivation | Ditto | 6.1.3.5 | - | O | - |  |
|  |  |  | Receiving a SM STATUS message by a SM entity | Ditto | 6.1.3.6 | - | M | - |  |
|  |  | SMS (Optional) | CM-procedure | 24.011 | 5 | - | - | - |  |
|  |  |  | SM-RL-procedure | Ditto | 6 | - | - | - |  |
|  |  |  | Message format on BTS-MS I/F | 04.12 | 3 | - | - | - | 04.12 For further sturdy |
|  |  | Supp. (Opt) | Generic Procedure for the control of SS | 24.010 | 2 | - | - | M | SA defines items. SS&CC are Related. |
|  |  |  | SS Support procedure | Ditto | 3 | - | - | M |  |