TSG-SA Working Group 1 (Services) meeting #7	TSG S1 (00) 038

Sophia Antipolis, France 7 - 11 February 2000	Agenda Item: 4.5

| 3GPP TSG-T3 (USIM) Meeting #11 Sophia Antipolis, France, 8 - 10 December, 1999 | Tdoc T3 - 99422 |
| --- | --- |

## Liaison Statement

From: 	T3

To:	S1

Subject:	Access Control Classes

The EFACC file contains the assigned access control class. The access control class is a parameter to

control the RACH utilisation. The 15 classes are split into 10 classes randomly allocated to normal subscribers, and 5 classes allocated to specific high priority users. Class 10 is not assigned in SIMs, but is used for emergency calls.

From a technical point of view, it is possible for the USIM to store two separate Access Control Classes, one for each network technology, so that the operator can assign different access control classes, which depend on the network technology. For example, a normal priority (0 to 9) could be assigned for GSM access, and a higher priority (11 to 15) for UMTS access.

On the other hand it is, of course, possible to have one common ACC for both technologies.

During the discussion T3 recognised that this is a service requirement issue, and seek the opinion of S1 in this regard.

A fast response would be highly appreciated since the information is needed in order to finalise specification 3G 31.102 for Release 99.