| 3GPP TSG-SA-WG1, Meeting #7 country , area, date | Tdoc S1-(00)062 |
| --- | --- |

Agenda Item:

Source:	NTT DoCoMo

Title:	LS on the handling of procedure for call progress indication

Effected Specifications / Releases: Release '99

Document for:	information and approval

- Introduction

At the last TSG-SA-WG1#6 meeting held in San Diego, S1 Chairman proposed to investigate condition about comfort tones to each country. Because only the conditions of Europe (CEPT) and North America(ANSI) are described to TS22.001 Annex F at present version. (Refer to TS 22.001 AnnexF.2.5)

NTT DoCoMo investigated this condition in Japan.

At first, this paper is the report of the conditions of comfort tones in Japan. Finally NTT DoCoMo proposed CR (Tdoc S1-00XXXX) to clarify the handling of the TS22.001 Annex F about “procedure for call progress indication”.

- Information

This table represents the information about comfort tones defined in Japan.

| Tone |  | Frequency | Tolerance | Type |
| --- | --- | --- | --- | --- |
| 1 | Dial tone | 400Hz | 20Hz | Continuous |
| 2 | Subscriber Busy | 400Hz | 20Hz | Tone on 500ms Silence 500ms |
| 3 | Radio Path acknowledgement | 400Hz | 20Hz | Tone on 1 Sec Silence 2 Sec |

It isn't defined clearly about other comfort tones, which is operator or vendor option in Japan.

- Proposal

It seems that handling of Indications of call progress, such as ringing, engaged, unobtainable, and no radio channel etc is different in each area or country.

The user or PLMN operator should be able to select Indications of call progress.

It is not the best way to write the value of all the countries or the area in the standard specification.

NTT DoCoMo proposes that handling of procedure for call progress indication is not  “normative” but it is “informative”. (See CR Tdoc S1-00XXXX)